3GPP TSG-SA4 Meeting #-e (AH) Video SWG post 134	S4aV250069

# Online, , 16th Dec 2025 - 16th Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.966 | CR | 0001 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AVFOPS_MED] New scenario: Video with changeable background |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi Technology Netherlands |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AVFOPS_MED |  |  |  |  |  | Date: |  |  | 2025-12-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress objective 1 on identifying relevant new representation formats not yet documented in TS 26.265. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a scenario on video with changeable background. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No pogress of the study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Change marks are against the previous CR version. All changes are new text in TR 26.966. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-252016 |  |  |  |  |  |  |  |  |

==============First change==============

[x1]	HEVC Video with Alpha (presentation slides), Shiva Sundar, CoreMedia Engineering, WWDC19

[x2]	Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AN1005, Future CICP extensions (Draft 2), 40th Meeting: Geneva, CH, 3–12 October 2025

[x3]	ISO/IEC JTC 1/SC 29/WG 03 N1628, Text of ISO/IEC 14496-12 8th edition DAM 2 Carriage of depth and alpha, October 2025, Geneva, CH

==============2nd change==============

## 5.6	Scenario #5: Video with changeable background

### 5.6.1	Overview

There is an increasing number of mobile device’s users who record videos and produce an edited video clip on their device itself. The video editing work may happen in the cloud via a service provider, in which case the UE would upload the source video, or alternatively, an application can work fully locally to generate the final video. In both cases, a common video editing effect is video compositing in which a video may be overlaid on top of another visual content. To this end, the overlay video is typically alpha blended using its alpha channel.

Examples of use cases were presented in [x2].

Because the alpha blending operation requires a pixel accurate information about the transparency level of the video frame samples, alpha channel is typically carried in a lossless manner.

### 5.6.2	Review of previous work

The coded representation of alpha auxiliary channels as part of a video bitstream has not been addressed until now in 3GPP specifications.

==============3rd change==============

### 5.6.3	Review of related work

The encoding of alpha channel as an auxiliary layer is enabled by the MV-HEVC video coding standard. The auxiliary layer is identified as an alpha auxiliary layer and this layer contains the alpha channel information SEI message. For more details, see Clause 6.9 Solution #4.1: MV-HEVC with auxiliary depth/alpha channels.

The ISO/IEC JTC 1/SC 29/WG 5 is the owner of the specification ISO/IEC 23091-2:2025 Coding-independent code points part 2 Video which defines codec-agnostic code points and fields that describing properties of a video representation. In the current Working Draft of this specification [x2], a new code point of the colour primaries field has been created to indicate that the decoded picture is representing an alpha channel. The following is extracted from [x2] to reflect the current status.

# Table 3 — Interpretation of colour primaries (ColourPrimaries) value

| Value | Colour primaries | HasChromaticityCoordinates | Informative remarks |
| --- | --- | --- | --- |
| … |  |  |  |
| 129 | Alpha channel | 0 | One colour component that represents an alpha channel |
| … |  |  |  |

This colour primaries field can be carried in VUI parameters in AVC and HEVC video bitstreams in order to indicate that a certain layer, in a single or multi layer bitstrteam, represents an alpha channel without relying on auxiliary layers design.

Regarding the storage of video sequenc of alpha maps, the MPEG Systems WG03 is developing the amendment 2 of ISO/IEC 14496-12 8th edition [x3] for specifying the storage of alpha maps in an auxiliary video track linked to a main video track. This amendment covers aspects of the storage related to media handler, track referencing and metadata box for the interpretation of the alpha maps.

==============4th change==============

### 5.6.4	Functional requirements

The functional analysis for video with alpha channel is done based on the following aspects:

1.	Assessment/discussion of hardware impact: there are two possibilities for this:

a.	There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

b.	There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

2.	Codec capabilities:

a.	Capability to losslesly encode the alpha information.

==============End of change==============