3GPP TSG-SA4 Meeting #-e (AH) Video SWG post 134	S4aV250071

# Online, , 16th Dec 2025 - 16th Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.966 | CR | 0002 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AVFOPS_MED] New scenario: Refocusable video |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi Technology Netherlands |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AVFOPS_MED |  |  |  |  |  | Date: |  |  | 2025-12-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress objective 1 on identifying relevant new representation formats not yet documented in TS 26.265. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a scenario on refocusable video. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No pogress of the study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Change marks are against the previous CR version. All changes are new text in TR 26.966. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-252018 |  |  |  |  |  |  |  |  |

==============First change==============

[x1]			“Camera bokeh | Android Open Source Project”, https://source.android.com/docs/core/camera/bokeh

[x2]			“MP4 With Auxiliary Tracks Extension (MP4-AT) File Format 0.9”, https://developer.android.com/media/platform/mp4-at-file-format

[x3]			Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AN1005, Future CICP extensions (Draft 2), 40th Meeting: Geneva, CH, 3–12 October 2025

[x4]			ISO/IEC JTC 1/SC 29/WG 03 N1628, Text of ISO/IEC 14496-12 8th edition DAM 2 Carriage of depth and alpha, October 2025, Geneva, CH

==============2nd change==============

## 5.7	Scenario #6: Refocusable video

### 5.7.1	Overview

For portrait pictures, a common esthetic effect is to present the person on a blurred background. This effect was tradittionaly achieved through the proper choice of lenses. With digital photography, this effect can be simulated before encoding as described in [x1].

With the advent of image editing and filtering, mobile device users are keen on modifying always the captured content. Refocusing an already captured content is becoming an expectation for users, especially for prosumers.

To achieve this refocusing, the typical image processing technique, described in [x2], is to record a sharp video and to generate for every video frame of the video sequence the bokeh effect based on the depth map sequence.

==============3rd change==============

### 5.7.2	Review of previous work

The coded representation of depth maps as part of a video bitstream has not been addressed until now in 3GPP specifications.

==============4th change==============

### 5.7.3	Review of related work

The encoding of depth maps as an auxiliary layer is enabled by the MV-HEVC video coding standard. The auxiliary layer is identified as an depth auxiliary layer and this layer contains the depth represenation information SEI message. For more details, see Clause 6.9 Solution #4.1: MV-HEVC with auxiliary depth/alpha channels.

The ISO/IEC JTC 1/SC 29/WG 5 is the owner of the specification ISO/IEC 23091-2:2025 Coding-independent code points part 2 Video which defines codec-agnostic code points and fields that describing properties of a video representation. In the current Working Draft of this specification [x3], a new code point of the colour primaries field has been created to indicate that the decoded picture is representing a depth map. The following is extracted from [x3] to reflect the current status.

# Table 3 — Interpretation of colour primaries (ColourPrimaries) value

| Value | Colour primaries | HasChromaticityCoordinates | Informative remarks |
| --- | --- | --- | --- |
| … |  |  |  |
| 130 | Depth map | 0 | One colour component that represents a depth map |
| … |  |  |  |

This colour primaries field can be carried in VUI parameters in AVC and HEVC video bitstreams in order to indicate that a certain layer, in a single or multi layer bitstrteam, represents depth maps without relying on auxiliary layers design.

Regarding the storage of video sequence of depth maps, the MPEG Systems WG03 is developing the amendment 2 of ISO/IEC 14496-12 8th edition [x4] for specifying the storage of depth maps in an auxiliary video track linked to a main video track. This amendment covers aspects of the storage related to media handler, track referencing and metadata box for the interpretation of the depth maps.

==============5th change==============

### 5.7.4	Functional requirements

The functional analysis for video with depth map is done based on the following aspects:

1.	Assessment/discussion of hardware impact: there are two possibilities for this:

a.	There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

b.	There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

2.	Codec capabilities:

a.	TBD, if any.

==============End of change==============