Source:	ORANGE

Title:	Mode sets for EVSoCS

## Document for:	Discussion, Approval

## Agenda Item:	4

## Introduction

In this contribution we propose EVS configurations (mode sets) for EVSoCS (UTRAN).

## Possible mode sets

We propose to consider the mode sets in Table 1 – the justification was already provided in in S4-151046: in addition to mode sets 0,1,2 (which follow the onion principle for SF 128, 256, 64 respectively), mode sets 3,4,5 add the flexibility of not using VBR, and mode set 6 brings the possibility to have a single audio bandwidth in normal conditions for EVS Primary modes.

Note that following comments raised by the Source on ambiguity of audio bandwidth, when a Primary bit rate is allowed, the associated default bandwidth (when no adaptation is requested, if allowed) is specified.

Table 1: Codec sets from [1].

| Configuration → (Config-EVS-Code) ↓ Codec Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EVS Primary 24.4 |  |  | FB |  |  | FB |  |
| EVS Primary 16.4 |  |  | FB |  |  | FB |  |
| EVS Primary 13.2 | SWB |  | SWB | SWB |  | SWB | SWB |
| EVS Primary 9.6 | SWB |  | SWB | SWB |  | SWB | SWB |
| EVS Primary 8 | WB | WB | WB | WB | WB | WB |  |
| EVS Primary 7.2 | WB | WB | WB | WB | WB | WB |  |
| EVS Primary 5.9VBR | WB | WB | WB |  |  |  |  |
| AMR-WB IO 12.65 | WB |  | WB | WB |  | WB | WB |
| AMR-WB IO 8.85 | WB |  | WB | WB |  | WB | WB |
| AMR-WB IO 6.60 | WB | WB | WB | WB | WB | WB | WB |
| NOTE: the maximum audio bandwidth at a given bit rate shall be used for EVS Primary modes – if bandwidth adaptation is allowed for EVS Primary modes (e.g. by CMR), this specified audio bandwidth may be reduced to a lower bandwidth during the call; if the input signal has a lower audio bandwidth, the coded audio bandwidth may also be lower in some cases. |  |  |  |  |  |  |  |

Concerns were expressed on avoiding too many mode sets. We believe that it is very unlikely that mode sets 2 and 5 get deployed, therefore these two mode sets could be skipped with high priority if a reduction of mode set is needed. Moreover, mode sets 1 and 4 are meant for SF256 with the goal to provide more capacity, and in this case one may consider that VBR might be allowed – this implies that mode set 4 could have relatively lower priority.

If the number of mode sets has to be reduced one could there consider the list given in Table 2.

Table 2: Proposed codec sets.

| Configuration → (Config-EVS-Code) ↓ Codec Mode | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| EVS Primary 13.2 | SWB |  | SWB | SWB |
| EVS Primary 9.6 | SWB |  | SWB | SWB |
| EVS Primary 8 | WB | WB | WB |  |
| EVS Primary 7.2 | WB | WB | WB |  |
| EVS Primary 5.9VBR | WB | WB |  |  |
| AMR-WB IO 12.65 | WB |  | WB | WB |
| AMR-WB IO 8.85 | WB |  | WB | WB |
| AMR-WB IO 6.60 | WB | WB | WB | WB |
| NOTE: the maximum audio bandwidth at a given bit rate shall be used for EVS Primary modes – if bandwidth adaptation is allowed for EVS Primary modes (e.g. by CMR), this specified audio bandwidth may be reduced to a lower bandwidth during the call; if the input signal has a lower audio bandwidth, the coded audio bandwidth may also be lower in some cases. |  |  |  |  |

## Proposal

To progress the discussion on EVSoCS, we propose to consider the reduced list of mode sets given in Table 2.

## References

[1] Tdoc S4-151046, Comments on onion principle for EVS and proposals for EVSoCS, Source: ORANGE

[2] Tdoc S4-151009, Call Setup and Handover Scenarios for EVS, Source: Ericsson