3GPPSA4-MBS SWG for EMM-EFEC	Tdoc S4-AHI232

13-15 December, 2011, Copenhagen, Denmark

Agenda item: 		EMM-EFEC

Source: 		Broadcom

Title: 			Proposed FEC Overhead Evaluation Procedure

Document for		Discussion

# 1. Introduction

One objective of current work item EMM is to improve the reliability of streaming and download services delivery over MBMS [1]. This requires comparison of the proposed codes. This document recommends how evaluate code performance and overhead using a channel model.  Comments are also provided on how to correlate the BEC channel model.

# 2. Overhead Evaluation Procedure for BEC

Data to be transmitted is partitioned into K symbols.  These K symbols are used to generate N total symbols to be transmitted, where N>=K.  Typically, N is set by a network operator based on the maximum amount of overhead that can be tolerated.  The N symbols are transmitted through a binary erasure channel with erasure probability P_e.  (The BEC may also be correlated, perhaps based on a Markov model)  Successful decoding requires at least K symbols to be received, but in some cases additional received symbols may be necessary.   Denote the number of symbols received in excess of K to be O.  The decoding failure probability distribution is a function of O and is given as P(O)=Pr{decoding with O overhead symbols or less fails}.

The transmit overhead describes the number of symbols that need to be transmitted for successful decoding.  Define the number of transmitted symbols as TX, where by necessity 0<=TX<=N.  The decoding failure probability distribution is a function of TX and is given as P(TX)=Pr{decoding with TX transmit symbols or less fails}.

- Fix K, the number of encoded symbols

- Fix N, the maximum number of symbols (systematic or repair) to be transmitted

- Use a Binary Erasure Channel with probability of error Pe for each symbol.  [i.e. 1% to 10%]  A correlated BEC is also possible.

- Set O=-1 and TX=-1

- Set Number_of_received_symbols=0

- While (Number_of_received_symbols<K)

  - TX=TX+1

  - Transmit a symbol through the BEC.  If the symbol is delivered by the BEC

    - Number_of_received_symbols=Number_of_received_symbols+1

- Attempt to Decode with the received symbols

- If decoding is not successful

  - TX=TX+1

  - Transmit a symbol through the BEC.  If the symbol is delivered by the BEC

    - O=O+1

  - Goto 7

- If decoding is successful

  - Note TX and O

  - Goto 4

- Compare code performance in range of 100% to 1% probability of failure

  - Compare P(O)

  - Compare P(TX)

In this way, code performance is compared in a realistic manner in which there is a maximum number of possible transmit symbols.  Additionally, the decoder receives data in the order in which it is transmitted.

# 2. BEC Parameters

Simulation results should at least be compared based on an IID BEC with probability or erasure P_e.

In additional, simulation results could also be compared based on a correlated BEC.  As proposed in other documents, a correlated BEC could be constructed using a Markov model.  An example two state Markov model is shown below.

Figure 1: BEC Markov Model

BEC parameters may also be derived from a channel model, such as Pedestrian B or Vehicular A.  This requires abstracting the physical layer performance to a BEC equivalent.  This also requires defining a time scale of how the symbols are transmitted through the channel.  Given a target MCS level and fading process, channel capacity could be used to determine the binary erasure channel state.  If the channel SNR exceed that needed for the MCS level, then the channel would pass the data.  If the channel SNR is less than what is needed for the MCS level, then the channel would erase.  This could be simulated in the following manner:

- Fix a channel model and initialize it to a given initial condition

- Fix an average SNR value.

- Fix an MCS level and corresponding required SNR value, Required_SNR.

- Generate a channel sample.  Determine the resulting SNR value Instantaneous_SNR

- If Instantaneous_SNR < Required_SNR

  - Channel in the erasure state

Else

  - Channel is the non-erasure state

- Goto 4

Optionally, a fading channel model could be approximated by a Markov model with some number of states.

# 4. Conclusion

This document proposes how to compare FEC performance using a BEC.  Comments about correlating the FEC are also discussed.

References

[1] S4-110786, “Work Item Description (Enhancements to Multimedia: PSS, MMS, and MBMS Enhancements and Performance Improvements)”