3GPP SA4 MBS SWG Ad-Hoc for EMM-EFEC	S4-AHI243

13 – 15 December 2011

Copenhagen, Denmark

Agenda item: 	4

Source: 	Samsung Electronics

Title: 	Proposed code performance evaluation for enhanced MBMS FEC (EMM-EFEC)

Document for	Consideration

Introduction

To evaluate the performance of a new code for certain environments a code performance evaluation is proposed to harmonize proposed S4-AHI241 and S4-AHI242 as provided in section 2.

Code Performance

Introduction

For error resilience streaming and file delivery service, FEC scheme based on block (N, K) code is applied. For given N and K and loss rate, FEC recovery performance is mainly dependent on loss model and is dependent FEC code structure (e.g. parity check matrix structure). For given loss rate, some FEC code may have higher performance on burst losses rather than on random losses and the others may have higher performance on random losses rather than burst losses.

Therefore, we need to evaluate FEC code performances both random loss model and burst loss model.

For evaluation purpose, we need to define common parameters. In this point of view, we may not have agreed common parameters on burst loss model because there could be lots of cases for parameters on burst loss model. So, we need to simplify burst loss model for only evaluation purpose.

Proposed Evaluation Procedure

For random loss model, Section 2.2 of S4-AHI242 is used.

For burst loss model, step 3: “Randomly pick K of the N symbols” of Section 2.2 of S4-AHI242 is replaced as follows:

3. Randomly pick consecutive K of the N symbols.

Test Cases

The following test cases are determined for the purpose of evaluating the code performance.

Table 1 Test Cases for Code Performance

| Number | K | N |
| --- | --- | --- |
| CP1 | 32 | 34 |
| CP2 | 32 | 38 |
| CP3 | 32 | 64 |
| CP4 | 256 | 269 |
| CP5 | 256 | 307 |
| CP6 | 256 | 512 |
| CP7 | 1024 | 1075 |
| CP8 | 1024 | 1229 |
| CP9 | 1024 | 2048 |
| CP10 | 8192 | 8601 |
| CP11 | 8192 | 9830 |
| CP12 | 8192 | 16384 |

Performance Metrics

For reporting, “2.3.1 Performance Metrics” of S4-AHI242 is used.

Conclusion

It is proposed to add the code performance evaluation in sections 2.