3GPP TSG-SA4 MBS SWG AH#79 Telco on FS_IS3	S4-AHI693

9 Jan 2017

Agenda item: 	4

Source: 	Qualcomm Incorporated (Rapporteur)

Title: 	Revised TR 26.953 Text on ATSC Service Interactivity

Document for	Discussion and Agreement

# Introduction

Currently, Section 6.2 in TR 26.953 describes service interactivity mechanisms as described in the ATSC A/105 standard [1] associated with broadcast TV service delivery based on the ATSC A/53 emission standard [2], popularly referred to as “ATSC 1.0”. Service interactivity and other related enhanced services specifications associated with ATSC 1.0 services delivery, such as the Non-Real Time Services, A/103 [3], comprise the so-called “ATSC 2.0” family of standards. The interactivity mechanisms for ATSC 2.0 are now effectively out-of-date as the ATSC 1.0 system and associated standards are being completely updated with the ongoing development and publication of the ATSC 3.0 family of broadcast TV standards. The ATSC 3.0 standards include interactive services enabling functionality. Therefore, the corresponding text in TR 26.953 should be replaced by the description of service interactivity mechanisms for the ATSC 3.0 system. For that purpose, this document proposes the replacement text for Section 6.2 in TR 26.953.

# Proposed Replacement Text on ATSC Service Interactivity

[The associated numbering of sections/sub-sections section below in this Discussion Paper is intended to represent the exact replacement text for Section 6.2 and its sub-sections in the TR. Modifications to the existing text in TR 26.963 are shown by change marks to make the proposed changes easier to identify.]

==================

## 6.2	ATSC Service Interactivity

### 6.2.1	Introduction to ATSC

ATSC, or Advanced Television Systems Committee, Inc., is an international, non-profit organization which develops standards for digital television transmission over terrestrial, cable, and satellite networks. It was formed in 1982 by the member organizations of the Joint Committee on InterSociety Coordination (JCIC): the Electronic Industries Association (EIA), the Institute of Electrical and Electronic Engineers (IEEE), the National Association of Broadcasters (NAB), the National Cable Telecommunications Association (NCTA), and the Society of Motion Picture and Television Engineers (SMPTE). Its current member organizations represent the broadcast, broadcast equipment, motion picture, consumer electronics, computer, cable, satellite, and semiconductor industries.

ATSC is developing ATSC 3.0 as the next generation ATSC Broadcast TV transmission standard for use over terrestrial broadcast, cable and satellite networks. At the time of completion of this Technical report, it is expected that ATSC 3.0 systems will be deployed in South Korea in 2017, within several years in the USA after the FCC incentive spectrum auction and subsequent channel re-packing, and further on in the future in other major North American countries such as Canada and Mexico. ATSC 3.0 comprises a family of 19 individual specifications which include physical layer, IP-based transport protocols, service and application level signaling, content formats for carriage of streaming media services, security and content protection, and a W3C-compliant and Web application based runtime environment.

### 6.2.2	ATSC Service Interactivity Enabling Functionality

Service interactivity related functionality, or more precisely, service interactivity enablers, are defined in two separate ATSC 3.0 standards: A/337 “Application Signaling” [X] and A/344 “ATSC 3.0 Interactivity Content” [Y].

A/337 defines signaling of the delivery method and properties of broadcaster applications bound to linear services (e.g., linear TV), and synchronization of application-initiated actions to the underlying audio/video content. Broadcaster applications are Web/HTML5 applications provided by TV broadcasters and associated with TV programs transmitted by those broadcasters for providing interactive features to the end-user – for example, synchronized interactive displays and targeted interactive advertising. Application-initiated actions refer to time-specific functionality defined by the broadcaster application logic – for example, the display of certain media files or advertisements, the incidents of which may be pre-scheduled or occur dynamically and unexpectedly in time. The occurrences of these actions are triggered by notifications referred to as “Events”.

A/344 defines the details of a W3C-compliant User Agent (i.e., web browser) based execution environment that enables a broadcaster applications to run. Such application may employ graphical capabilities of the ATSC 3.0 receiver to render the user interface or access certain resources or information related to service interactivity and provided by the receiver. Those resources/information may comprise application launch pages and, for example, corresponding Javascript, CSS and XML documents which provide the logic, mark-up and display control of the broadcaster application and/or media files for rendering under application control. Such resources may be delivered to the receiver via broadcast, or fetched from a network server by the application via broadband/Internet access. If a broadcaster application requires access to broadcast-delivered resources from the receiver, or if the application requires the receiver to perform specific actions not defined by the User Agent APIs, it can request that resource from a built-in WebSocket server in the receiver via a set of ATSC-defined JSON-RPC messages as specified in A/344.

#### 6.2.2.1	Application Signaling

A/337 [X]

defines application signaling in the form of an XML-based Service Layer Signaling metadata fragment called HELD (HTML Entry pages Location Description). This metadata fragment specifies the properties and other information of files belonging to broadcaster application(s) associated with ATSC 3.0 linear service(s). Those properties/information include:

- the delivery method (broadcast, broadband or via both transports) of application files,

- whether a file associated with the file URI (‘Content-Location’ of the extended FDT describing that file) is an individual launch page for the application, or a package of files (aggregated using multipart MIME) which includes the launch page,

- the TSI value of the LCT session/channel for broadcast delivery of application files using the ROUTE (Real-time Object delivery over Unidirectional Transport) protocol as defined in the ATSC A/331 standard [Z],

- time intervals during which application content such as media files are broadcast prior to their use by the application.

The XML schema of the HELD is shown in Figure 6.1

Figure 6.1: XML Schema of the HELD

Notifications of actions to be taken by broadcaster applications, or Events, may be delivered by broadcast or broadband. For a broadcast streaming service associated with a broadcast application and delivered by the ROUTE protocol (i.e., corresponding to a DASH-formatted streaming service), the DASH Event mechanism is used for the notification delivery. DASH Events may be delivered using either of the two mechanisms for Event delivery as defined in the MPEG-DASH specification [8]:

- via EventStream element(s) appearing in a Period element of the MPD;

- as Event(s) carried in ‘emsg’ box(es) appearing in Media Segments, with their presence signaled by one or more InbandEventStream elements of the Representation element in the MPD.

These two delivery mechanisms may be mixed in the carriage of Event. A single Event stream may include some Events delivered via an EventStream element and others delivered via emsg boxes.

When delivered via broadcast in an MMT-based system, Events may be delivered in an XML document called an Application Event Information (AEI) document, whose syntax and semantics are very similar to that of DASH Events.

#### 6.2.2.2	Interactivity Content

From the service interactivity perspective, A/344 [X] defines the interactions between one or more service interactivity specific broadcaster applications, running in the ATSC 3.0 receiver, and the receiver platform/middleware. As indicated in Sec. 6.2.2, A/344 specifies a W3C-compliant User Agent/browser environment within which broadcaster applications are run. The application provides the user with enhanced functionality linked to the main service or programs, such as interactivity features. An interactivity application may include additional assets such as JavaScript files, images and multimedia files which provide, for example, user interface and interactivity display control functions. If the interactivity application requires access to broadcast-delivered resources from the receiver, or if the application requires the receiver to perform specific actions not defined by the User Agent APIs, it can request that resource from a built-in WebSocket server in the receiver, via a set of ATSC-defined JSON-RPC messages. These JSON-RPC messages allow the application to a) query information that was gathered or collected in the receiver, b) receive notifications via broadcast signaling, and c) request the receiver to perform actions that are not otherwise available via the standard JavaScript APIs. As described in Sec. 6.2.2.1, the received notifications may comprise DASH Event messages which trigger certain UI functionality or media assets to be presented to the user at specific times during the playout of the main program.

Figure 6.2 below depicts a logical representation of the functional components in the ATSC 3.0 receiver.

Figure 6.2: Logical Architecture of ATSC 3.0 Receiver

The interactivity application is launched after the receiver obtains application signaling information (as described in Sec. 6.2.2.1) and forwards the launch URL to the User Agent, which, in turn, loads the entry application document from the URL. Once the interactive application is running, it may request content from various local or external URLs. Any content received over broadcast via ROUTE file delivery will be saved into a local receiver cache associated with and accessed from the receiver’s local web server.

#### ==================

# Summary and Recommendation

This document proposes revised description for TR 26.953 on ATSC 3.0 service interactivity enabling functionality, as replacement to the existing, outdated description of ATSC 2.0 interactivity mechanisms. It is proposed that the text as shown in Section 6.2 and its sub-sections of this document be incorporated as the revised Section 6.2 contents in this TR.

# References

- [1]	ATSC Standard A/105, “Interactive Services Standard”, 29 October 2015, http://atsc.org/wp-content/uploads/2015/11/A105-2015.pdf.

- [2]	ATSC Standard A/53, “A/53: ATSC Digital Television Standard, Parts 1-6, 2007”, 3 January 2007, http://atsc.org/wp-content/uploads/2015/03/a_53-Part-1-6-2007.pdf.

- [3]	ATSC Standard A/103:2014, “Non-Real-Time Content Delivery”, 25 July 2014, http://atsc.org/wp-content/uploads/2015/03/Non-Real-Time-Content-Delivery.pdf.

- [X]	ATSC A/337 Working Draft, “Application Signaling” (undergoing Technical Working Group 3 ballot; expect to be approved and publicly available in January 2017).

- [Y]	ATSC Candidate Standard A/344, “ATSC 3.0 Interactive Content”, 29 December 2016, http://atsc.org/wp-content/uploads/2016/12/A344S34-230r1-CS-Interactive-Content.pdf.

- [Z]	ATSC Candidate Standard A/331, “Signaling, Delivery, Synchronization, and Error Protection”, 5 January 2016, http://atsc.org/wp-content/uploads/2016/01/A331S33-174r5-Signaling-Delivery-Sync-FEC.pdf.