Source:	Samsung Electronics Co., Ltd.

Title:	Options to support ROHC and FEC over MBMS

Document for:	discussion and agreement

Agenda Item:	4

# Introduction

FRASE is expected to enable ROHC and FEC for the delivery of streams over MBMS for the Group Communication Service Enabler to support Public Safety services. The MB2 and the xMB interfaces are to be extended to enable the signaling of these options, so as to inform the BM-SC to perform ROHC and FEC.

This document lists open questions and discusses the options.

# Open Questions and Discussion

## Applicability

It can be assumed that traffic to be FEC protected or on which ROHC is to be applied is a UDP stream. Depending on whether the source or the BM-SC are to generate the multicast packets, ROHC and FEC are only to be applied to the IP/UDP packet headers of a multicast packet. The two scenarios are described by the following diagrams:

In addition, [1] also states that the RTP header may be compressed. However, it is not clear if this implies unprotected RTP packets and excludes SRTP and DTLS or if all options are to be supported.

## Per Flow vs Global Activation

In [1], it is assumed that activation of ROHC and FEC can be activated on a per stream basis. This of course increases the complexity of the solution. At a first step, xMB was extended to only support global activation, i.e. for all UDP streams of the session.

## Confirmation

If the origin server is performing the service announcement, the BM-SC will be required to inform the BM-SC about the ROHC and FEC configuration, so that the origin server conveys the information to the receiving devices. This information can be encoded as part of an SDP that is returned back to the origin server.

## Target Delivery Method

MBMS defines 4 delivery methods: Streaming, Download, Group Communication, and Transparent delivery modes. SA6 and SA2 are expecting that ROHC and FEC are enabled for the Group Communication Service Enabler, which implies the Group Communication delivery method. However, the Transparent delivery method is expected to replace the GC over the long run. This is why ROHC and FEC should be enabled for both GC and Transparent delivery methods. Both the Streaming and Download methods support FEC. Adding ROHC to these delivery methods is not needed.

## Performance

ROHC and FEC can be further configured to fine tune their performance. For instance the periodicity of the IR packet of ROHC may be adjusted to the expected residual error rate over MBMS.

The following text from [1] describes such behavior:

The GCS AS may request that RoHC is applied for IP flows within the MBMS bearer by including the following information elements: the RoHC configuration, the list of RTP and UDP flows to be header compressed, characterized by their destination IP addresses and port numbers. For each of these flows, the request may indicate a target periodicity for the full header packets.

Similar adjustments can be performed for FEC. To optimize this, feedback from receivers may be needed to estimate the packet loss rate and to adjust the ROHC and FEC operation accordingly.

## FEC Framework

The FEC Framework from the Streaming delivery method seems to be appropriate for application to the GC and Transparent delivery methods as it operates on UDP. However, limitations on the source block size, bundling, and signaling will be required to adjust the FEC Framework to the new applications.

# Proposal

To finalize the FRASE work item, the open questions and design choices, as discussed in section 2, need to be addressed and verified.

# References

[1]	3GPP TS 23.468, Group Services and System Aspects; Group Communication System Enablers for LTE