3GPP TSG-SA4 5GMS3 Ad-Hoc Meeting                                                         S4-AHI872

The Hague, Netherlands, Nov 12-14, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Metrics collection and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Metric signalling doesn't fully match the 5GMS architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Metric signalling updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================== CHANGE  ==============================

## 5.5	Metrics collection and reporting

Metrics collection and reporting can be done in different ways, depending on the relation between the Service Provider and the Network Operator. The following figures shows simplified signalling examples for two different use-cases.

In the first use-case, shown in figure 5.5-1 below, the MNO is controlling the metric reporting, using the out-of-band configuration and reporting option. In this case the metrics are configured via the RAN and the 5G control plane.

Figure 5.5-1: Metrics collection using the out-of-band 5G control plane option

The different steps are explained below:

1: Overall metrics configuration is done on the network level, for instance defining which geographical areas that shall have metrics collection active, and how often metrics shall be reported.

2: The metric configuration(s) is sent from the OAM to the RAN, which does not forward that information to the UE at this stage.

3: Time passes, and it assumes that the UE moves around during that period.

4: The UE enters an area (cell, location area, etc.,) which is inside the geographical constraint. This is discovered by the RAN, and it now needs to activate metrics reporting for the UE.

5: The actual metric configuration is sent from the RAN to the metrics handler function in the media session handler, via the 5G control plane.

6: Additional time passes, and the UE has a metrics configuration, but no streaming session has started.

7: A streaming session is started.

8: The session setup is done in conjunction with signalling transactions (not shown here).

9: A new metrics job is created in the media player.

10: A reference to the new metrics job is returned.

11: The configuration for the metrics collection job is sent to the media player (i.e. which metrics should be measured).

12: Media is delivered and rendered, and...

13: ...more media is delivered...

14: The configured metrics reporting interval has passed, and the metrics handler now requests the collected metrics from the media player.

15: The media player returns the collected metrics.

16: The metrics are reported via the 5G control plane.

17: The session continues...

18: more media is delivered, and then the session is finished.

19: The metrics handler requests the final metrics collected.

20: The media player returns the final collected metrics.

21: The metrics are reported to the OAM via the 5G control plane.

22: The metrics collection job is deleted.

23: Time passes, the UE moves around.

24: The UE leaves the geographical area specified by the metrics configuration.

25: RAN sends metrics (de)configuration to the UE, to stop future metrics collection.

The second use-case, shown in figure 5.5-2 below, illustrates a scenario where the metrics collection is configured in-band. Thus together with the media there are metadata which describe how metrics shall be collected, and how they shall be reported. The Media OAM and Media AS functions can both be either trusted or untrusted (but not mixed).

Figure 5.5-2: Metrics collection using the in-band metadata option

The different steps are explained below:

1: The content servers are set up, including any metrics configuration.

2: If the Media OAM and Media AS are located in a trusted domain, metadata regarding the metrics collection and reporting are sent to the AS. The procedure for configuring the Media AS, when the Media OAM and Media AS reside in an untrusted domain, is unspecified.

3: Time passes during which no streaming session is active...

4: A streaming session starts.

5: The App and the session handler initializes a new session.

6: The media player requests any metadata from the Media AS which is needed for downloading the media content.

7: Metadata is returned to the media player  and also forwarded to the session handler and (potentially) to the App.

8: A new metrics job is created in the media player.

9: A reference to the new metrics job is returned.

10: Based on the metadata, the configuration for the metrics collection job is sent to the media player (i.e. which metrics should be measured).

11: The streaming session is running.

12: Media is delivered and rendered, and...

13: more media is delivered...

14: The configured metrics reporting interval has passed, and the metrics handler now requests the collected metrics from the media player.

15: The media player returns the collected metrics.

16: The metrics are reported according to the

reporting method defined in the configuration, for instance HTTP upload.

17: The session continues...

18: more media is delivered, and then the session is finished.

19: The metrics handler requests the final metrics collected.

20: The media player returns the final collected metrics.

21: The metrics are reported.

22: The metrics collection job is deleted.

=========================== END OF CHANGE  ==========================