3GPP TSG-SA4 MBS Call	S4-AHIA04

# 18th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev |  | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Policy Template |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 18-06-2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution adds missing parameters to the policy template. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in Table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Visibility | Description |
| --- | --- | --- | --- | --- |
| policyTemplateId | Integer | 1..1 |  | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 |  | A Policy Template may be in the pending, ready, or suspended state. Only a Policy Template in the ready state may be instantiated as a Dynamic Policy Instance and applied to streaming sessions. |
| apiEndPoint | String | 1..1 | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | MNO Admin | N5: Npcf Policy Authorization Service. N33: AsSessionWithQoS or CHargableParty. |
| externalReference | String | 1..1 |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. |
| QoSSpecification | Object | 0..1 |  | Specifies the network quality of service to be applied to streaming sessions at this Policy Template. |
| marBwUl | BitRate | 0..1 |  | As defined in clause 8.6.2.7 of TS 29.514. |
| marBwDl | BitRate | 0..1 |  |  |
| maxPacketLossRateDl | PacketLossRateRm | 0..1 |  |  |
| maxPacketLossRateUl | PacketLossRateRm | 0..1 |  |  |
| maxSuppBwDl | BitRate | 0..1 |  |  |
| maxSuppBwUl | BitRate | 0..1 |  |  |
| minDesBwDl | BitRate | 0..1 |  |  |
| minDesBwUl | BitRate | 0..1 |  |  |
| desMaxLatency | Float | 0..1 |  |  |
| desMaxLoss | Float | 0..1 |  |  |
| mirBwUl | BitRate | 0..1 |  |  |
| mirBwDl | BitRate | 0..1 |  |  |
| ApplicationSessionContext | Object | 1..1 |  | Specifies information about the application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 | Read-Only | As defined in clause 5.6.2.3 of TS 29.514. |
| sliceInfo | Snssai | 0..1 |  |  |
| dnn | Dnn | 0..1 |  |  |
| aspId | AspId | 0..1 |  |  |
| ChargingSpecification | Object | 0..1 |  | Provides information about the charging policy to be used for this Policy Template. |
| sponId | SponId | 0..1 |  | As defined in clause 5.6.2.3 of TS 29.514. |
| sponStatus | SponsoringStatus | 0..1 |  |  |
| gpsi | array(Gpsi) | 0..* |  | List of UEs permitted to instantiate this Policy Template. |
| isBackgroundTraffic | Boolean | 0..1 |  | Indicates if the traffic of this policy is considered background traffic and shall only occur during authorized time. |