3GPP TSG-SA4 Meeting #110-e	S4-AHIA06

# Electronic, Online, 19th–28th August 2020	revision of S4-AHI995

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | – | rev | – | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of Ingest Protocols API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-07-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Complete the stage 3 design for Ingest Protocols. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing definition of procedures. - Modification of resource structure in response to feedback. - Addition of a simple HTTP pull-based media ingest protocol. An attempt has been made to expand the scope of this feature to describe supported protocols for uplink as well as downlink. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 design for Ingest Protocols will be incomplete in Release 16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4, 7.5.3, 7.5.4, 7.6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================================START OF FIRST CHANGE==============================

### 4.3.4	Ingest Protocols procedures

#### 4.3.4.1	General

The set of content ingest protocols supported by the 5GMSd AS at interface M2d and/or the set of content ingest protocols supported by the 5GMSu AS at interface M4u are described by the Ingest Protocols resource at M1, as specified in clause 7.5.3.1.

#### 4.3.4.2	Create Ingest Protocols

The Create operation is not permitted for the Ingest Protocols resource.

#### 4.3.4.3	Read Ingest Protocols

This procedure is used by the 5GMSd Application Provider to retrieve a list of media ingest protocols supported by the 5GMSd AS and/or the 5GMSu AS. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes an Ingest Protocols resource in the response message body, as specified in clause 7.5.3.1.

#### 4.3.4.4	Update Ingest Protocols

The Update operation is not permitted for the Ingest Protocols resource.

#### 4.3.4.5	Delete Ingest Protocols

The Delete operation is not permitted for the Ingest Protocols resource.

=================================END OF FIRST CHANGE===============================

===============================START OF SECOND CHANGE=============================

### 7.5.3	Data model

#### 7.5.3.1	IngestProtocols resource

The data model for the IngestProtocols resource is specified in table 7.5.3.1-1 below:

Table 7.5.3.1-1: Definition of IngestProtocols resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| downlinkIngestProtocols | array(ProtocolDescriptor) | 0..1 | An array of ProtocolDescriptor objects, as specified in clause 7.5.3.2, each one uniquely identifying a media ingest protocol supported at interface M2d by the 5GMSd AS(s) associated with the responding 5GMSu AF. |
| uplinkIngestProtocols | array(ProtocolDescriptor) | 0..1 | An array of ProtocolDescriptor objects, as specified in clause 7.5.3.2, each one uniquely identifying a media ingest protocol supported at interface M4u by the 5GMSu AS(s) associated with the responding 5GMSu AF. |

#### 7.5.3.2	ProtocolDescriptor type

The data model for the ProtocolDescriptor type is specified in table 7.5.3.2-1 below:

Table 7.5.3.2-1: Definition of ProtocolDescriptor type

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| termIdentifier | URI String | 1..1 | A fully-qualified term identifier from the controlled vocabulary urn:3gpp:5gms:ingest-protocol, as specified in clause 7.5.4. |
| descriptionLocator | URL String | 0..1 | The location of a description of the ingest protocol, for example the public web URL of its specification. |

================================END OF SECOND CHANGE==============================

================================START OF THIRD CHANGE==============================

### 7.5.4	Media ingest protocols

#### 7.5.4.1	Introduction

Every media ingest protocol shall be uniquely identified by a URI string term identifier allocated within the scope of the name space urn:3gpp:5gms:ingest-protocol.

#### 7.5.4.2	HTTP pull-based media ingest protocol

If IngestConfiguration[protocol] in the Content Hosting Configuration is set to urn:3gpp:5gms:ingest-protocol:http-pull, media resources shall be ingested by the 5GMSd AS using HTTP [9]. The IngestConfiguration[pull] property shall be set to True, indicating that a Pull-based protocol is used. The IngestConfiguration[entryPoint] property shall point at the 5GMSd Application Provider’s origin server, as specified in table 7.6.3.1-1 and may indicate the use of HTTPS [16]. It shall not contain a path part.

When the 5GMSd AS receives a request for a media resource at interface M4d that cannot be satisfied from its content cache, the request shall be transformed into a corresponding HTTP GET request directed to the 5GMSd Application Provider’s origin server via interface M2d, using the abovementioned entryPoint property concatenated with the mappedPath from the applicable path rewrite rule (if any) selected from DistributionConfiguration[PathRewriteRules] and the leaf path element from the M4d request URL to construct the M2d request URL.

#### 7.5.4.3	DASH push-based media ingest protocol

If IngestConfiguration[protocol] in the Content Hosting Configuration is set to urn:3gpp:5gms:ingest-protocol:dash-if-ingest, media resources shall be ingested by the 5GMSd AS as specified by [3]. The IngestConfiguration[pull] property shall be set to False, indicating that a Push-based protocol is used. The IngestConfiguration[entryPoint] property shall be set to the URL that will be used to upload the DASH segments and MPD to the 5GMSd AS at interface M2d. This entry point URL shall not contain a path: the path for the URL shall instead be specified by the IngestConfiguration[Path] property.

=================================END OF THIRD CHANGE===============================

===============================START OF FOURTH CHANGE=============================

#### 7.6.3.1 	ContentHostingConfiguration resource

The data model for the ContentHostingConfiguration resource is specified in table 7.6.3.1-1 below:

Table 7.6.3.1-1: Definition of ContentHostingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| IngestConfiguration | Object | 1..1 | Describes the 5GMSd Application Provider’s origin server from which media resources will be ingested via interface M2d. |
| name | String | 1..1 | A name associated with this origin server. |
| path | String | 1..1 | The relative path which will be used to address the media resources at interface M2d. This path is provided by the 5GMSd AF in the case of Push-based ingest. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSd AS shall use Pull or Push for ingesting the content. |
| protocol | String | 1..1 | Identifies the media ingest protocol. The set of supported protocols is defined in clause 7.5.4. |
| entryPoint | String | 1..1 | An entry point to ingest the content. The semantics of the entry point are dependent on the selected ingest protocol. In the case of Push ingest (pull flag is set to False), this parameter is returned by the 5GMSd AF to the 5GMSd Application Provider and indicates the entry point for pushing the content. In case of Pull (pull flag is set to True), the entryPoint shall be provided to the 5GMSd AF to indicate the location from which content is to be pulled. In this case, the entryPoint shall be used as the base URL. A request received by the 5GMSd AS is mapped to a URL using the provided base URL to fetch the content from the origin server. |
| DistributionConfiguration | array(Object) | 1..N | Specifies the distribution method and configuration for the ingested content. More than one distribution may be configured for the ingested content, e.g. to offer different distribution configurations such as DASH and HLS. |
| contentPreparationTemplateId | String | 0..1 | Indicates that content preparation prior to distribution is requested by the 5GMSd Application Provider. It identifies the Content Preparation Template that shall be used as defined in clause 7.4 |
| targetDomain | String | 1..1 | All resources of the current distribution shall be accessible through this targetDomain FQDN. This FQDN is assigned by the 5GMSd AF for the current distribution. There is a one-to-one association between the originDomain and the targetDomain for this distribution. |
| originDomain | String | 1..1 | The 5GMSd Application Provider assigns an originDomain for the distribution, which is for example used by the 5GMSd AS to select an appropriate certificate to present at M4d, and to set appropriate CORS HTTP response headers at M4d. The originDomain is associated with a targetDomain that is assigned by the 5GMSd AF. The 5GMSd Application Provider may choose to publish the content using its own originDomain (e.g. to use its own certificates or to avoid CORS issues). In such case, the 5GMSd Application Provider creates a CNAME DNS record to point from the originDomain to the targetDomain. |
| PathRewriteRules | array(Object) | 0..N | An ordered list of rules for rewriting the request URL paths of media resource requests handled by the 5GMSd AS. If multiple rules match a particular resource’s path, only the first matching rule, in order of appearance in this array, shall be applied. |
| requestPathPattern | String | 1..1 | A regular expression [5] against which the path of each 5GMSd AS request URL, including the leading “/”, is compared. In the case of Pull-based ingest, the M4d request path is compared with this regular expression. In the case of Push-based ingest, the M2d request path is compared with this regular expression. In either case, if the request path matches this pattern, the path mapping specified in the corresponding mappedPath shall be applied. |
| mappedPath | String | 1..1 | A replacement for the 5GMSd AS request path up to, but excluding, the final path element, i.e. starting with the leading “/” and ending with the final “/”. In the case of Pull-based ingest, IngestConfiguration[entryPoint] is concatenated with the mapped path to form the M2d origin request URL. In the case of Push-based ingest, DistributionConfiguration[targetDomain] is concatenated with the mapped path to form the distribution URL exposed over M4d. |
| CachingConfiguration | Array(Object) | 0..N | Defines a configuration of the 5GMSd AS cache for a matching subset of media resources ingested in relation to this Content Hosting Configuration. |
| urlPatternFilter | String | 1..1 | A pattern that will be used to match media resource URLs to determine whether a given media resource is eligible for caching by the 5GMSd AS. The format of the pattern shall be a regular expression as specified in [5]. |
| CachingDirectives | Object | 1..1 | If a urlPatternFilter applies to a resource, then the provided CachingDirectives shall be applied by the 5GMSd AS at M4d, potentially overwriting any origin caching directives ingested at M2d. |
| statusCodeFilters | Array(Integer) | 0..N | The set of HTTP origin response status codes to which these CachingDirectives apply. The filter shall be provided as a regular expression as specified in [5]. If the list is empty, the CachingDirectives shall apply to all HTTP origin response status codes at M2d. |
| noCache | Boolean | 1..1 | If set to True, this indicates that the media resources matching the filters shall not be cached by the 5GMSd AS and shall be marked as not to be cached when served by the 5GMSd AS at M4d. |
| maxAge | Integer | 0..1 | The caching time-to-live period that shall be set on ingested media resources matching the filters. This determines the minimum period for which the 5GMSd AS shall cache matching media resources as well as the time-to-live period signalled by the 5GMSd AS at interface M4d when it serves such media resources. The time-to-live for a given media resource shall be calculated relative to the time it was ingested. |
| GeoFencing | Object | 0..1 | Limit access to the content to the indicated geographic areas. |
| locationType | String | 1..1 | The type of the location information. |
| locations | Array(String) | 1..N | Array of locations from which access to the resources is to be allowed. |
| UrlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content resource at M4d. |
| urlPattern | String | 1..1 | A pattern that shall be used to match M4d media resource URLs. The 5GMSd AS shall not serve a matching media resource at M4d unless it includes a valid authentication token. The format of the pattern shall be a regular expression as specified in [5]. |
| tokenName | String | 1..1 | The name of the M4d request query parameter that the Media Player must use to present the authentication token when required to do so. |
| passphraseName | String | 1..1 | The name of the query parameter that is used to refer to the passphrase when constructing the authentication token. Note that the token is not included in the cleartext part of the M4d URL query component. |
| passphrase | String | 1..1 | The shared secret between the 5GMSd Application Provider and the 5GMSd AS for this DistributionConfiguration. The passphrase is used in the computation and verification of the M4d authentication token but is never sent in-the-clear over that interface. |
| tokenExpiryName | String | 1..1 | The name of the M4d request query parameter that the Media Player must use to present the token expiry field. |
| useIPAddress | Boolean | 1..1 | If set to True, the IP address of the UE is included in the computation of the authentication token for resources that match urlPattern and access to matching media resources shall be allowed by the 5GMSd AF only when the M4d request is made from a UE with this IP address. |
| ipAddressName | String | 0..1 | The name of the M4d request query parameter that is encoded as part of the authentication token if the useIPAddress flag is set to True. Note that the IP address is not passed in the cleartext part of the M4d URL query component. |
| certificateId | String | 0..1 | When content is distributed using TLS [16], the X.509 [8] certificate for the origin domain is shared with the 5GMSd AF so that it can be presented by the 5GMSd AS in the TLS handshake at M4d. This attribute indicates the identifier of the certificate to use. |

================================END OF FOURTH CHANGE==============================