3GPP TSG-<TSG/WG> Meeting # <MTG_SEQ><MTG_TITLE>	S4-AHIA14

# <Location>, <Country>,  <Start_Date> - <End_Date>

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Informative Annex on Parameter Population |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document is a followup of S4-AHI999, which was discussed during last SA4 adhoc. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# Annex X		Dynamic policy parameter propagation

## X.1	End-to-end model

The figure below depicts an e2e model. The arrows indicate the main information flow. The existing 26.501 interface terminology is used for the fitting functions. However, there are additional interfaces (i.e. P1 and U1), which are not in the 5GMS Architecture.

All interfaces are:

- M1: Provisioning interface between the 5GMS Application Provider and the 5GMS AF.

- P1: The 5GMS Application provider provisions the DASH MPD generator, e.g. provision Service Descriptors.

- U1: GUI, which is getting user input. Note, the 5GMS application provider controls the application, i.e. controls the GUI choices.

- M8: Non-standardized input from the 5GMS application Provider to the 5GMS aware application, like country specific application behaviors (languages, VoD catalog, etc).

  - Input on subscriptions (4K subscription vs SD subscription).

  - Device specific content selection rules (e.g. SmartPhone vs. Smart TV).

  - Additional service offering features (e.g. background download possible).

- C1 (one of M6 or M7): Information from the application to Media Client, e.g. user selections.

- M6: Information flow from the DASH Player to the Media session handler.

- M7: Information flow from the Media Session Handler to the Dash player

- M5_1: Information flow into the media session handler for parameter provisioning (Policy Descriptions, which originate from 5GMS AF and 5GMS Application Provider). Policy descriptions contains or references the detailed Service Access Information, i.e. URLs to activate a certain policy.

- M5_2: Information flow from the Media Session Handler to the 5GMS AF.

The DASH standard includes “Service Descriptors” (ISO/IEC 23009-1 Annex K) in its 4th edition. The concept of Service Descriptors is to provide additional information to the DASH player to influence the “Selection Logic”, e.g. the DASH Player should prefer a certain set of representations within an adaptation set. It is assumed that the DASH MPD can be annotated to give hints for subscription models and different device types.

The 5G System has different means to detect application flows. The Media Session Handler should provide with the dynamic policy activation according information to detect the application data flow(s). It is assumed here, that multiple applications are executed simultaneously on a UE device and each application may independently access the network. Therefore, the Media Session Handler needs to provide (and update) means to detect the application traffic, which should be treated according to the dynamic policy.

The UPF contains a Traffic Detection Function as described in TS 23.501 Clause 5.8.2.4. The SMF provides the Traffic Detection Information for each PDU Session in form of Packet Detection Rules (PDRs), which are derived from the PCC rules. Traffic Detection is called Service Data Flow Detection in the PCC rules (TS 23.503, Clause 6.2.2.2). Service Data Flow Detection is either based on Service Data Flow Filters or on Packet Flow Descriptions (PFDs).

A Service Data Flow Filter for IP PDU Sessions can be the 5-Tuple of Client and Server IP address, protocol and client and server ports. The Service Data Flow Filters for IP PDU session may also contain for IPv4 a Type of Service (ToS) value (incl a mask), for IPv6 a flow label or security parameter indexes. Packet Flow Descriptions (PFDs) can references via the application identifier in the PCC rule (TS 23.501 Clause 5.8.2.8.4 and TS 23.503 Clause 6.3.1). A Packet Flow Description can contain contains significant parts of a URL (e.g. hostname) or a domain name. PFDs can be managed by the 5GMS AF using the “PFD Management API” of the NEF.

Depending on the selected Service Data Flow Filter, the Media Session Handler may either provide 5-Tuples of newly created TCP connections or mark the outgoing traffic e.g. using a flow lable or a tos field value. Alternatively, the Media Session Handler may provide the Server IP address or the server domain name, when all traffic to that server / domain should be treated according to the policy.

Figure 1: end-to-end model

In the following, the parameter propagation for the different use-cases is discussed.

## X.2	Premium QoS

### X.2.1	General

In case of a Premium QoS use-case, the 5GMS Client should activate a QoS bearer with characteristics matching the service needs. It is assumed that the DASH content is prepared for different subscription models, e.g. 4K, HDR, or SD and for different target device types, e.g. SmartPhone or SmartTV. When activating a stream according to the subscription model (e.g. 4K), the 5GMS client activates a dedicated bearer with matching bitrate setting. Note, the client may activate a lower bitrate than supported by the 5G System, e.g. a small screen SmartPhone may activate a different bearer than a large screen device.

The 5GMS Application Provider interacts with the 5GMS AF to setup one or more Policy Templates (using M1). Each Policy Template is identified by a policy template id and contains information, how to activate the according policy within the 5G System (e.g. N5 URLs and parameters). The 5GMS application provider interacts with the DASH content generation function (e.g. an MPD provider) to annotate the DASH MPD with Service Descriptors (using P1). See Step 1 and 2 below.

The 5GMS aware application is configured via M8 (Step 3) with information about the available content catalog (e.g. resolving MPD URLs), the available subscription identifiers (e.g. the user has a 4k subscription or the user has a SD subscription), device type identifiers and network policy identifiers. Each Policy Template is identified by a policy template id and contains information, how to activate the according policy within the 5G System (e.g. N5 URLs and parameters).

The subscription identifiers and the device type identifiers are called Service Description Filters in the following.

It is fore fure study, whether the network policy identifiers are embedded in the MPD Service Descriptors or derived from the Service Descriptors.

The ‘network policy identifier’ can be equal to a Policy Template Id, when the application is aware about its usage (e.g. for QoS streaming or background download).

Figure X.2.1-1: Sequence chart

When a user selects via U1 a video (Step 4), the 5GMSAware Application provides input (via C1) on the selected presentation entry (i.e. MPD URL) together with a Network Policy Identifier (the value indicates here a “Premium QoS” policy, i.e. make the Media Session Handler to request a QoS bearer) and a Service Description Filters (Step 5). The SD Filter is used by the Media Player to identify the usable Service Descriptors from the MPD. The Network Policy Identifier is used by the Media Session Handler to find the according Policy Description containing information on the Dynamic Policy Instantiation method (i.e. procedure and parameters like Policy Template Id).

Step 6: The DASH player fetches the MPD and applies the Service Descriptor Filter (Step 7).

Step 8: The DASH player indicates to the Media Session Handler (M6) that a “Premium QoS” network service should be activated (value of the Network Policy Id). The DASH player provides input on bitrate ranges (which may depend on the device type). The Media Session Handler has received one ore more Policy Descriptions together with matching Service Access Information (via M5_1). When the Media Session Handler has received the policy indication (Step 8), The Media Session Handler uses the Network Policy Id to find the procedure and the parameters to activate the Dynamic Policy Instance (i.e. find the matching Policy Description). Here, the Network Policy Id indicates a Premium QoS policy. The Media Session Handler should activate a dynamic policy in the 5GMS AF, providing packet detection filters of the DASH media flows (audio, video, etc) and to provide the target bitrate of the video.

Step 9: The Media Session Handler activates a Dynamic Policy Instance with the 5GMS AF. The 5GMS AF uses the Policy Template Id to lookup the matching Policy Template in order to create the PCF or NEF API invocation.

### X.2.2	Example parameters

M5_1 Parameters for Policy Descriptions (used by the Media Session Handler)

| Parameter | Type | Purpose | Example Values |
| --- | --- | --- | --- |
| Policy Description | Object |  |  |
| Network Policy Id | String | Identifies the Policy Description | “PremiumQoS4K” |
| Service Access Information URL | URL | References the accosiated service access information |  |

M5_1 Parameters for Service Access Information

| Parameter | Type |  |  |
| --- | --- | --- | --- |
| Service Access Information | Object |  |  |
| Policy Template Id | String | Identifies the Policy Template | “QoS” |
| 5GMS AF URL | URL | Used to invoke the 5GMS AF |  |
| Mandatory Request M5 information | List | Target bitrate, which should be provided by the network for the application | Policy Template Id, Bitrate, Packet Detection Filters |
| M5 Response information | List | Information to the Media Session Handler on the response parameters | Ok (Bitrate accepted), Proposed Lower Bitrate (the requested bitrate cannot be provided) |
| Packet Detection Filter types | [String] | Indication, what Packet Detection Filter types are recommended to use by the media session handler. | 5-Tuple, domain names, ToS marking, etc |

## X.3	(Conditional) zero rating, provided bit rate remains below a threshold (“Binge On” policy)

### X.3.1 General

In case of a (Conditional) Zero Rating, the quality of a video streaming service is not exceeding a certain bit rate threshold. This can be realized by deploying a traffic shaper in the network (e.g. a policing function in the UPF) or by instructing the DASH Player to not exceed a certain bit rate policy threshold. The bit rate policy maybe be network specific, i.e. depending on the 5G System. The following realization assumes the latter realization, i.e. the DASH Player is not exceeding the bit rate policy and the UPF is just monitoring the accordance of the traffic flows. The MPD is annotated in such a way using the DASH Service Descriptors, that the DASH Player can filter the representations that comply with the policy.

The 5GMS Application Provider interacts with the 5GMS AF to setup one or more Policy Templates (Step 1). Each Policy Template is identified by a policy template id and contains information, how to activate the according policy within the 5G System (e.g. N5 URLs and parameters). The 5GMS application provider also interacts with the DASH content generation function (e.g. an MPD provider) to annotate the DASH MPD with Service Descriptors (Step 2). The intention of the Service Descriptors here is that the DASH Player can identify those representation combinations, which not exceed the bit rate requirement.

The 5GMS aware application is configured via M8 (Step 3) with information about the available content catalog (e.g. resolving MPD URLs), the available subscription identifiers (e.g. the user has a 4k subscription or the user has a SD subscription), device type identifiers. Each Policy Template is identified by a policy template id and contains information, how to activate the according policy within the 5G System (e.g. N5 URLs and parameters).

The 5GMSd-Aware application is configured via M8 about the available (Conditional) Zero Rating policy. This includes the Network Policy Ids.

Figure 2: Information Flow

When a user selects (via U1) a video (Step 4), the 5GMS aware Application provides input (via C1) on the selected presentation entry (i.e. MPD URL) and also on the Network Policy Id (value indicates here a (conditional) Zero-Rating policy, i.e. make the Media Session Handler to request the policy threshold parameter from the network). Note, C1 is an abstract interface and indicates, that the 5GMS-Aware application may either first use M6 or M7 for the interactions with the 5GMS Client.

Step 6: The Media Session Handler uses the Network Policy Id to find the procedure and the parameters to activate the Dynamic Policy Instance (here a (conditional) Zero Rating policy). The Media Session Handler has received one ore more Policy Descriptions together with matching Service Access Information (via M5_1). The Media Session handler uses the Network Policy Id as a key to find the correct Policy Description. Here, the Network Policy Id indicates a (Conditional) Zero Rating policy. The Media Session Handler should activate a dynamic policy in the 5GMS AF, providing packet detection filters of the DASH media flows (audio, video, etc) and retriving the bitrate threshold, which shall not be exceeded to comply with the policy. The Media Session Handler receives also some information on the policy enforcement, so that the representation selection logic (bitrate adaptation function) can consider effects of the enforcement scheme.

The Media Session Handler activates the Dynamic Policy Instance on M5, providing a Policy Template Id. Upon positive response, the Media Session handler notifies the DASH Player, providing Service Description Filters (Step 7). The Media Session Handler may receive the Service Description Filters with the response or may lookup the SD Filter values by a response value. Alternatively, the Media Session Handler receives a maximum bit rate with the response and the Media Session Handler derives the Service Description Filter. The Media Session Handler may also receive information about Policy Enforcement, e.g. what type of traffic shaper will throttle the bit rate.

Step 9: The Service Description Filter is used by the Player to filter “usable” Service Descriptors from the MPD. The DASH Access Engine or Selection Logic (see ISO 23009-1 Figure K.1) selects only adaptation sets and representations according to the filter. Here, the DASH Player fetch the MPD after the notification from the Media Session Handler.

### X.3.2	Example parameters

M5_1 Parameters for Policy Descriptions (used by the Media Session Handler)

| Parameter | Type | Purpose | Example Values |
| --- | --- | --- | --- |
| Policy Description | Object |  |  |
| Network Policy Id | String | Identifies the Policy Description | “(Conditional) Zero Rating” |
| Service Access Information URL | URL | References the accosiated service access information |  |

M5_1 Parameters for Service Access Information

| Parameter | Type |  |  |
| --- | --- | --- | --- |
| Service Access Information | Object |  |  |
| Policy Template Id | String | Identifies the Policy Template | “not exceed bitrate” |
| 5GMS AF URL | URL | Used to invoke the 5GMS AF |  |
| Packet Detection Filter types | [String] | Indication, what Packet Detection Filter types are recommended to use by the media session handler. | 5-Tuple, domain names, ToS marking, etc |
| Mandatory M5 Request information | List | Target bitrate, which should be provided by the network for the application | Policy Template Id, Packet Detection Filters |
| M5 Response information | List | Information to the Media Session Handler on the response parameters | Bitrate Threshold (upper bitrate bound, which should not be exceeded) |

## X.4	Background download

### X.4.1	General

In case of a background download use-case, the video asset is acquired in the background, prior to viewing. Many application services offer the capability of acquring a VoD item for later consumption. The 5GMS-Aware Application triggers the Media Session Handler to acquire the item, providing a background download network policy id. Note, here, the DASH Player is handling the acquisition, since the DASH Player contains the MPD processing and the DASH Access engine parts. Other realizations would use a separate background download agent, which is not even try to decode and render the video.

The 5GMS Application Provider interacts with the 5GMS AF to setup one or more Policy Templates (M1). Each Policy Template is identified by a policy template id and contains information, how to activate the according policy within the 5G System (e.g. N5 URLs and parameters).  The 5GMS application provider also interacts with the DASH content generation function (e.g. an MPD provider) to annotate the DASH MPD with Service Descriptors, e.g. to identify, which representation is intended for pro-load (Step 2).

The 5GMSAware application is configured via M8 (Step 3) with information about the available content catalog (e.g. resolving MPD URLs), the available subscription identifiers (e.g. the user has a 4k subscription or the user has a SD subscription), device type identifiers. Each Policy Template is identified by a policy template id and contains information, how to activate the according policy within the 5G System (e.g. N5 URLs and parameters).

The 5GMSd aware application is configured via M8 about the available background download policy. This includes the Network Policy Id which hints a background download policy.

When a user selects (via U1) a video for pre-load (Step 4), the 5GMS-Aware Application provides input (via C1 in Step 5) on the selected presentation entry (i.e. MPD URL) and also on the Network Policy Id (indicating a background download policy, i.e. make the Media Session Handler to request a bearer suitable for background download). Note, C1 is an abstract interface and indicates, that the 5GMS-Aware application may either first use M6 or M7 for the interactions with the 5GMS Client.

Step 6: The Media Session Handler uses the Network Policy Id to find the procedure and the parameters to activate the Dynamic Policy Instance (here a background download policy). The Media Session Handler has received one ore more Policy Descriptions together with matching Service Access Information (via M5_1). The Media Session handler uses the Network Policy Id as a key to find the correct Policy Description. Here, the Network Policy Id indicates a background download policy. The Media Session Handler should activate a dynamic policy in the 5GMS AF, providing packet detection filters of the media flows (audio, video, etc). The Media Session Handler can also receive information on a bitrate policing, e.g. that the bitrate of the background download is actively limited.

The Media Session Handler activates the Dynamic Policy Instance on M5, providing the policy template id and additional parameters. Upon positive response, the Media Session handler notifies the DASH Player to start the pre-load (Step 7). The notification contains a Service Description Filters, which is used by the Player to filter “usable” Service Descriptors from the MPD. The Media Session Handler may receive the Service Description Filters with the response or may look up the SD Filter values by a response value (e.g. derived from a max bit rate indication).

Step 9: The DASH Access Engine / Selection Logic (see ISO 23009-1 Figure K.1) selects only adaptation sets and representations according to the filter (i.e. suitable for pre-load). Here, the DASH Player fetch the MPD after the notification from the Media Session Handler.

### X.4.2	Example parameters

M5_1 Parameters for Policy Descriptions (used by the Media Session Handler)

| Parameter | Type | Purpose | Example Values |
| --- | --- | --- | --- |
| Policy Description | Object |  |  |
| Network Policy Id | String | Identifies the Policy Description | “background download” |
| Service Access Information URL | URL | References the accosiated service access information |  |

M5_1 Parameters for Service Access Information

| Parameter | Type |  |  |
| --- | --- | --- | --- |
| Service Access Information | Object |  |  |
| Policy Template Id | String | Identifies the Policy Template | “backgrounddata” |
| 5GMS AF URL | URL | Used to invoke the 5GMS AF |  |
| Packet Detection Filter types | [String] | Indication, what Packet Detection Filter types are recommended to use by the media session handler. | 5-Tuple, domain names, ToS marking, etc |
| Mandatory M5 Request information | List | Target bitrate, which should be provided by the network for the application | Policy Template Id, Packet Detection Filters |
| M5 Response information | List | Information to the Media Session Handler on the response parameters | Bitrate Threshold (upper bitrate bound, which should not be exceeded) |

X.5

**** Last Change ****