3GPP TSG-<TSG/WG> Meeting # <MTG_SEQ><MTG_TITLE>	<TDoc#>

# <Location>, <Country>,  <Start_Date> - <End_Date>

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <Spec#> | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updated on M5 Dynamic Policy activation API and M1 Policy Template Provisioning API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The document contains changes to the M1 Policy Template API and the M5 Dynamic Policy Invocation API based on the discussion in S4-AHI999 (now S4-AHIa14) and the discussion on available application flow detection mechanisms (in S4-AHIa13). The document is a first draft and may not contain all parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 7.9	Policy Templates Provisioning API

### 7.9.1	Overview

The Policy Templates Provisioning API allows a 5GMS Application Provider to configure a set of Policy Templates within the scope of a Provisioning Session that can subsequently be applied to media streaming sessions belonging to that Application Provider using the Dynamic Policies API specified in clause 11.5. A Policy Template is used to specify the traffic shaping and charging policies to be applied to these media streaming sessions.

A Policy Template, identified by its policyTemplateId, represents a set of PCF/NEF API parameters which defines the service quality and associated charging for the media streaming sessions. The Policy Template is configured as part of the Provisioning procedures with the 5GMS AF and is then used by the 5GMS AF to request specific QoS and charging policies for that session from the PCF or NEF.

### 7.9.2	Resource structure

The Policy Template Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.9.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.9.2-1: Operations supported by the Policy Template Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template | policy-templates | POST | Used to create a new Policy Template resource. |
| Fetch a Policy Template | policy-templates/‌{policyTemplateId} | GET | Used to retrieve an existing Policy Template resource. |
| Update a Policy Template |  | PUT, PATCH | Used to modify the configuration of an existing Policy Template. |
| Delete a Policy Template |  | DELETE | Used to delete an existing Policy Template resource. |

### 7.9.3	Data model

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in Table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Visibility | Description |
| --- | --- | --- | --- | --- |
| policyTemplateId | Integer | 1..1 |  | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 |  | A Policy Template may be in the pending, ready, or suspended state. Only a Policy Template in the ready state may be instantiated as a Dynamic Policy Instance and applied to streaming sessions. |
| apiEndPoint | String | 1..1 | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | MNO Admin | N5: Npcf Policy Authorization Service. N33: AsSessionWithQoS or CHargableParty. |
| externalReference | String | 1..1 |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. |
| QoSSpecification | Object | 0..1 |  | Specifies the network quality of service to be applied to streaming sessions at this Policy Template. |
| qosReference | String |  |  | As defined in clause 5.6.2.7 of TS 29.514. |
| marBwUl | BitRate | 0..1 |  |  |
| marBwDl | BitRate | 0..1 |  |  |
| maxPacketLossRateDl | PacketLossRateRm | 0..1 |  |  |
| maxPacketLossRateUl | PacketLossRateRm | 0..1 |  |  |
| maxSuppBwDl | BitRate | 0..1 |  |  |
| maxSuppBwUl | BitRate | 0..1 |  |  |
| minDesBwDl | BitRate | 0..1 |  |  |
| minDesBwUl | BitRate | 0..1 |  |  |
| mirBwUl | BitRate | 0..1 |  |  |
| mirBwDl | BitRate | 0..1 |  |  |
| desMaxLoss | Float | 0..1 |  |  |
| desMaxLatency | Float | 0..1 |  |  |
| ApplicationSessionContext | Object | 1..1 |  | Specifies information about the application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 | Read-Only | As defined in clause 5.6.2.3 of TS 29.514. |
| sliceInfo | Snssai | 0..1 |  |  |
| dnn | Dnn | 0..1 |  |  |
| aspId | AspId | 0..1 |  |  |
| ChargingSpecification | Object | 0..1 |  | Provides information about the charging policy to be used for this Policy Template. |
| sponId | SponId | 0..1 |  | As defined in clause 5.6.2.3 of TS 29.514. |
| sponStatus | SponsoringStatus | 0..1 |  |  |
| gpsi | [Gpsi] | 0..* |  | List of UEs permitted to instantiate this Policy Template. |

Editor’s Note: The parameter externalReference is for further study. It may be a provisioning parameter of the Media Player and / or a Media Session Handler to assist mapping of external references to a policyTemplateId.

Editor’s Note: The ChargingSpecification object may contain any charging related information, such as sponId or afChargeId.

**** next Change ****

## 11.2	Service Access Information API

### 11.2.1	General

### 11.2.2	Resources

The Service Access Information API is accessible through the following URL base path:

{apiRoot}/3gpp-m5d/v1/service-access-information/{saiSubresource}

The following operations and the corresponding HTTP methods are supported. In each case, the sub-resource path specified in the second column shall be substituted into {sai-subresource} in the above URI template:

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch Service Access Information | { saiSubresource } | GET | Used to acquire the Service Access Information resource for the specified Provisioning Session. |

### 11.2.3	Data model

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformtion resource is specified in Table 11.2.3.1-1 below:

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | Unique identification of the M1d Provisioning Session. |
| StreamingAccess | Object | 0..1 |  |
| mediaPlayerEntry | URL String | 1..1 | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| ClientConsumptionReportingConfiguration | Object | 0..1 |  |
| reportingInterval | DurationSec | 0..1 | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| serverAddresses | Array(URL String) | 1..1 | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| locationReporting | Boolean | 1..1 | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |
| samplePercentage | Percentage | 1..1 | The percentage of streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |
| DynamicPolicyInvocationConfiguration | Object | 0..1 |  |
| serverAddresses | Array(URL String) | 1..N | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| validPolicyTemplateIds | Array(String) | 1..N | A list of Policy Template identifiers which the 5GMSd Client is authorized to use. |
| applicationFlowDescriptionMethods | Array(String) | 1..N | A list of recommended application flow description methods. E.g. 5-Tuple, ToS, 2-Tuple, etc, which should be used by the Media Session Handler to describe the service data flow filters for the to be policed traffic. |
| externalReference | String | 0..1 | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. |
| ClientMetricsReportingConfiguration | Object | 0..1 |  |
| serverAddresses | Array(URL String) | 1..N | A list of 5GMSd AF addresses to which metrics reports shall be sent. (Opaque URL, following the 5GMS URL format.) |
| dataNetworkName | String | 0..1 | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero.When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| samplePercentage | Percentage | 1..1 | The percentage of streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |
| urlFilters | Array(String) | 1..N | A list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |
| metrics | Array(String) | 1..N | A list of metrics which shall be reported. |

### 11.2.4	Operations

This clause defines the behaviour that is expected from the 5GMSd AF when a Service Access Information resource is acquired from the Media Session Handler. The main operation that is performed is to look up or generate the Service Access Information.

**** Next Change ****

## 11.5	Dynamic Policies API

### 11.5.1	Overview

The Dynamic Policies API allows the Media Session Handler to request a specific policy and charging treatment to be applied to a particular application data flow by invoking RESTful operations on the 5GMSd AF at interface M5d. The API defines a set of data models, resources and the related procedures for the creation and management of the dynamic policy request. The corresponding JSON schema for the representation of the resources and operations defined by the API is provided in Annex ZZ.

### 11.5.2	Data model

#### 11.5.2.1	DynamicPolicy resource type

Table 11.5.2.1-1: Definition of Dynamic Policy resource

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| policyTemplateId | String | 1 | Identifies the Policy Template which should be applied to the application flow(s). |
| flowDescription | Object | 0..1 | Refer to subclause 5.3.8 of 3GPP TS 29.214 for encoding. |
| tosTrCl | TosTrafficClass | 0..1 | Type of Service or Traffic Class as defined in TS 29.514 Clause 5.6.3.2-1. |
| provisioningSessionId | String | 1 | Uniquely identifies Provisioning Session, that represents the 5GMS Application Provider. |
| bitrates | Object | 0..1 |  |
| marBwDlBitRate | BitRate | 0..1 | Maximum requested bit rate for the Downlink. |
| marBwUlBitRate | BitRate | 0..1 | Maximum requested bit rate for the Uplink. |
| minDesBwDlBitRate | BitRate | 1..1 | Minimum desired bandwidth for the Downlink. |
| minDesBwUlBitRate | BitRate | 1..1 | Minimum desired bandwidth for the Uplink. |
| mirBwDlBitRate | BitRate | 1..1 | Minimum requested bandwidth for the Downlink. |
| mirBwUlBitRate | BitRate | 1..1 | Minimum requested bandwidth for the Uplink. |
| enforcementMethod | String | 0..1 | Description of the Policy Enforcement Method. The parameter is set by the 5GMSd AF. |
| enforcementBitrate | Integer | 0..1 | Description of the enforcement bitrate. |

### 11.5.3	Resource structure

#### 11.5.3.1	General

All resource URIs of this API should have the following root:

{apiRoot}/3gpp-dynamicpolicies/v1/

All sub-resource paths in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods are supported for the Dynamic Policies API:

Table 11.5.3.1-1: Resources and methods overview

| Resource name | Sub-resource path | Allowed HTTP methods | Meaning |
| --- | --- | --- | --- |
| Dynamic Policies | policies | GET | Forbidden. The 5GMSd AF shall return an error code. |
|  |  | POST | Create a new Dynamic Policy resource. |
| Dynamic Policy | policies/{policyId} | GET | Read a Dynamic Policy resource |
|  |  | PUT | Replace an existing Dynamic Policy resource. |
|  |  | PATCH | Modify an existing Dynamic Policy resource. |
|  |  | DELETE | Delete an existing Dynamic Policy resource. |

#### 11.5.3.3	Dynamic Policies resource

##### 11.5.3.3.1	Introduction

Editor’s Note: To be filled in

##### 11.5.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/policies/

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 11.5.3.3.2-1: Resource URL variables for resource "Dynamic Policies"

| Name | Definition |
| --- | --- |
| apiRoot | Base URL to the 5GMSd AF. |

##### 11.5.3.3.3	Resource Method GET

The 5GMSd AF shall return an Error code.

##### 11.5.3.2.4	Resource Method POST

The POST method creates a new Dynamic Policy resource for a given Media Session Handler. The Media Session Handler shall initiate the HTTP POST request message and the 5GMSd AF shall respond to the message. The 5GMSd AF shall construct the URI of the created resource using that URI.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the Table 5.x.3.2.4-1 and Table 5.x.3.2.4-2.

Table 11.5.3.2.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

Table 11.5.3.2.4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | DynamicPolicy | 1 | Parameters to create a Dynamic Policyinstance. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  |  |  | 201 Created | The Dynamic Policy instance resource was created successfully. The URI of the created resource shall be returned in the Location HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table xxx also apply. |  |  |  |  |

#### 11.5.3.4	Dynamic Policy resource

##### 11.5.3.4.1	Introduction

<some Intro>

##### 11.5.3.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/policies/{policyId}

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 11.5.3.4.2-1: Resource URL variables for resource "Dynamic Policy"

| Name | Definition |
| --- | --- |
| apiRoot | Base URL of the 5GMSd AF. |
| {policyId} | Unique identifier, created by the 5GMSd AF during the create procedure (5.x.3.3.4). |

##### 11.5.3.4.3	Resource Method GET

##### 11.5.3.4.4	Resource Method PUT

##### 11.5.3.4.5	Resource Method PATCH

##### 11.5.3.4.6	Resource Method DELETE

**** Next Change ****