3GPP TSG-WG SA4 AHG Meeting	S4h190833

# Paris, FR, 5 – 6 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR |  | rev | - | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pseudo CR to TS 26.501 on Architecture and Procedures for a Common Ingest Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 5.4 Ingesting Content for Unicast Downlink Streaming

The 5G system defines a common interface for content ingest for downlink media streaming over 5G. The control part of the ingest interface may be performed through the NEF. After the ingest, the content is accessible through a new location URL, which may be associated with a service provider URL through DNS CNAME records. The following example shows how this operation is performed:

The CNAME entry is as follows: cdn.serviceprovider.com. 60 IN CNAME contentdomain.operatorcdn.com. The entry point to the content is as follows: https://cdn.serviceprovider.com/content1/content.mpd The UE resolves the URL domain and gets redirected to the operator’s domain, which is then resolved into the IP address of the MNO CDN node closest to the UE.

The interface supports the ingest of the following types of content:

- Live streaming content

- On demand streaming content

- Static files such as images, scene description files, …

The interface provides an API that allows a Media AS to create/update/delete a Distribution. A Distribution contains all the paramters and configurations to a particular content ingest and distribution setup.

Note: In the current version of the specification, the ingest interface only supports Unicast downlink streaming.

The content ingest procedure is as follows:

The steps are explained as follows:

- In this step, the external Media AS creates a new Distribution for its content through the MNO’s 5G system. It associates the Distribution with a domain name, supplies the certificate for HTTPS access to the content, sets the caching rules per media type, the distribution area, distribution protocol, logging information, … Upon successful configuration, the Media AF will respond with the Distribution id, and the location to which to send the content (if using the push mode).

- The Media AF configures the related Media AS(s) to prepare for media ingest for that particular Distribution. This step may involve instructing the Media AS to set the appropriate caching rules and

- The Media AS may start receiving the media data if using the push mode.

- A UE that desires to consume the media presentation, will acquire the entry point through external means and resolve the domain name of the content.

- The external Media AS responds with a domain name that points to the MNO hosted Distribution.

- The UE starts consuming the media using the MNO hosted domain, which resolves to the closest Media AS to the UE.

- The Media AS may need to fetch the requested media data from a parent Media AS or directly from the origin if it is not already cached locally.

- The Media AS services the requested media content to the UE