3GPP TSG-SA4 MTSI SWG Telco on 5G_MEDIA_MTSI_ext	S4-AHM468

28 March 2019, 16:00-18:00 CET

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | XXXX | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signaling of ANBR Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_MEDIA_MTSI_ext |  |  |  |  |  | Date: |  |  | 2019-03-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of the 5G_MEDIA_MTSI_ext work item aims to specify the following ANBR functionality, as aligned with the agreed conclusions in TR 26.919: - Recommend support of ANBR as an adaptation trigger, as described by clause 10.7 of TS 26.114 - Support for negotiation of radio capabilities on ANBR signalling as described in clause 5.5.6 of TR 26.919. No signaling is currently present to provide end-to-end coordination between the UEs in regards to ANBR support. As observed in Tdoc S4-181240, ANBR procedures in TS 26.114 in the absence of end to end coordination may suffer from unfavourable consequences such as high PLR and poor quality. Enabling such end-to-end coordination would be useful for a UE to learn the ANBR capabilities of its remote UE and may accordingly determine the most suitable media adaptation on its end, e.g., in response to ANBR information received from its local eNB/gNB. Moreover, as also observed in TR 26.919, such signalling can allow the PCF/PCRF when setting GBR<MBR bearers. Furthermore, from a conformance point of view, the LTE_VoLTE_ViLTE_enh test case included in TS 36.523-1 assumes that if a UE is capable of receiving ANBR information then it is also capable of using this ANBR information as an adaptation trigger. Such a conformance point is currently not supported by the existing media handling requirements on ANBR in TS 26.114, where ANBR is an optional/recommended feature, i.e., a UE is free to ignore ANBR information as an adaptation trigger. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SDP-based indication of ANBR-capabilities. If UE signals this SDP capability, then it is capable of receiving ANBR information from its access network and its MTSI client is capable of ANBR as defined in clause 10.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No end-to-end coordination between the UEs on their ANBR-triggered adaptation capabilities. Conformance test case in TS 36.523-1 cannot be satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.X (new), Annex M.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[…]

[163]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[X]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[Y]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

Second Change (NEW)

### 6.2.X	ANBR Support attribute "anbr"

Access network bitrate recommendation (ANBR) is described in clause 10.7. Use of ANBR with dynamic bitrate adaptation is described in clause 10.7.3 and related adaptation of sent and received media is described in clauses 10.7.3.2 and 10.7.3.3, respectively. At the radio signalling level, ANBR signaling capability, also known as RAN-assisted codec adaptation as specified in TS 36.321 [157] for LTE access and TS 38.321 [X] for NR access.

The media-level SDP attribute "anbr" is specified in this clause to indicate ANBR support. If the MTSI client in terminal signals this attribute in the SDP, then it supports ANBR as described in clause 10.7, including the use of ANBR with dynamic bitrate adaptation as described in clause 10.7.3. Moreover, if the MTSI client in terminal signals this attribute in the SDP, then its UE is capable of RAN-assisted codec adaptation specified in TS 36.321 for LTE access and/or TS 38.321 for NR access. For LTE access, inclusion of "anbr" in the SDP indicates whether the UE is able to query and receive ANBR information (for both downlink and uplink ANBR) from its eNB. Likewise, for NR access inclusion of this attribute indicates whether the UE is able to query and receive ANBR information. The "a=anbr" attribute shall only be used on media level. The MTSI client in terminal shall only signal "anbr" in the SDP if the P-CSCF has indicated its ability to handle this SDP attribute, as described in clause E.10 of TS 23.228 [Y].

Informing the remote MTSI client via the SDP about the ANBR support using the "anbr" attribute helps the remote MTSI client to determine the most suitable media adaptation on its end, this is especially valid if the remote MTSI client itself is also capable of performing ANBR-triggered adaptation, e.g., in response to ANBR information received from its local eNB/gNB. As an example, for ANBR-triggered rate up-switching, an MTSI client may adapt its sent bitrate and also perform CMR / RTCP-APP / TMMBR / TMMBN signalling more responsively based on the ANBR information it receives from its local eNB / gNB, if it knows that the remote MTSI client also supports ANBR. In the absence of the knowledge of the ANBR capabilities of the remote UE, an MTSI client may not have dynamic knowledge on the feasibility of the new bitrate based on ANBR over the access network of the remote UE, and this may not be desirable especially in a rate up-switching scenario.

Furthermore, signalling of ANBR capabilities in the SDP via "a=anbr" can also be useful for the PCF/PCRF when setting GBR<MBR bearers.

Here is an example use of the "a=anbr" attribute relative to a media line:

a=anbr

The IANA registration information for the "a=anbr" SDP attribute is provided in Annex M.X.

Third Change (NEW)

# M.X	anbr

Contact name, email address, and telephone number:

3GPP Specifications Manager

3gppContact@etsi.org

+33 (0)492944200

Attribute Name (as it will appear in SDP)

anbr

Long-form Attribute Name in English:

3GPP access network bitrate recommendation (ANBR) support attribute

Type of Attribute

Media level

Is Attribute Value subject to the Charset Attribute?

This Attribute is not dependent on charset.

Purpose of the attribute:

This attribute is used to indicate the UE’s ability to use ANBR as an adaptation trigger and also its ability to receive ANBR information from the access network.

Appropriate Attribute Values for this Attribute:

No values. See clause 6.2.X for detailed usage.

End of document