3GPP TSG-SA4 Meeting on Virtual Reality 	S4-AHV017

# Santa Clara, CA, US, 6 - 8 Dec 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.244 | CR | xx | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 26.244 3GPP file format (3GP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI, Nokia Corporation, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRStream |  |  |  |  |  |  | Date: |  | 2017-12-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To enable support of MPEG OMAF viewport dependent profile with 3GPP file format. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of the reference to the newer version of ISO/IEC 14496-15 and adding of an additional extension of ISO base media file format to the 3GP File Format. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OMAF HEVC Viewport Dependent profile will not be supported by the 3GPP file format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.3, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

==== FIRST  CHANGE====

# 2	References

...

[20]	ISO/IEC 14496-15:2017: "Information technology – Coding of audio-visual objects – Part 15: Carriage of NAL unit structured video in the ISO base media file format".

<Editor’s note`: 14496-15 will be updated soon including Amd. 1, Cor. 2.2 and Cor. 3. For more information refer to the MPEG output documents w16940, w16787, w16938 and File-format Meeting Report.>

…

==== END OF FIRST CHANGE====

==== SECOND  CHANGE====

# 5	Conformance

…

### 5.2.3	Extensions

The following extensions to the ISO base media file format [7] can be used in a 3GP file:

-	streaming-server extensions (see clause 7);

-	asset information (see clause 8);

-	video-buffer information (see clause 9);

-	AVC file format (see clause 5 of [20]), for which the use of sample entry types other than 'avc1' is disallowed;

-	MVC file format (see clause 7 of [20]);

-	H.265 (HEVC) file format (see clause 8 of [20]);

-	H.265 (HEVC) file format (see clause 9 of [20]), for which the use of L-HEVC Configuration record is disallowed, the sample entry types other than 'hvc1' and 'hvc2' are disallowed and track reference types other than 'scal' are disallowed.

-	RTP and RTCP reception hint tracks (see [38]);

-	SRTP and SRTCP reception hint tracks with key management information for SRTP recordings (see [38] and clause 12);

-	Quality metrics timed metadata track (see clause 4 of [53] and clause 16 of this specification).

If SDP information is included in a 3GP file, it shall be used as defined by the streaming-server extensions.

…

==== END OF SECOND CHANGE====

==== THIRD  CHANGE====

## 6.2	Sample Description box

In an ISO file, Sample Description Box gives detailed information about the coding type used, and any initialisation information needed for that coding. The Sample Description Box can be found in the ISO file format Box Structure Hierarchy shown in figure 6.1.

Figure 6.1: ISO File Format Box Structure Hierarchy

The Sample Description Box can have one or more Sample Entries.

Valid Sample Entries already defined for ISO and MP4 include MP4AudioSampleEntry, MP4VisualSampleEntry and HintSampleEntry. Other Sample Entries shall be according to the following:

- AMR, AMR-WB 			AMRSampleEntry

- AMR-WB+					AMRWPSampleEntry

- EVS							EVSSampleEntry

- H.263							H263SampleEntry

- H.264(AVC)					AVCSampleEntry

- H.265(HEVC)				HEVCSampleEntry

- Timed text					TextSampleEntry

- DIMS							DIMSSampleEntry

- CVO timed metadata			CVOSampleEntry

- Location timed metadata	LocationSampleEntry

- Quality metrics timed metadata		QualityMetricsSampleEntry

- Orientation timed metadata	 	OrientationSampleEntry

The format of SampleEntry and its fields are explained as follows:

SampleEntry ::= 	MP4VisualSampleEntry | 
 	MP4AudioSampleEntry | 
 	AMRSampleEntry | 
 	AMRWPSampleEntry |

EVSSampleEntry |
 	H263SampleEntry | 
 	AVCSampleEntry |
 	TextSampleEntry | 
 	DIMSSampleEntry |
 	HintSampleEntry |
 	CVOSampleEntry |
	HEVCSampleEntry |
	LocationSampleEntry |

QualityMetricsSampleEntry |

OrientationSampleEntry

Table 6.1: SampleEntry fields

| Field | Type | Details | Value |
| --- | --- | --- | --- |
| MP4VisualSampleEntry |  | Entry type for visual samples defined in the MP4 specification. |  |
| MP4AudioSampleEntry |  | Entry type for audio samples defined in the MP4 specification. |  |
| AMRSampleEntry |  | Entry type for AMR and AMR-WB speech samples defined in clause 6.5 of the present document. |  |
| AMRWPSampleEntry |  | Entry type for AMR-WB+ audio samples defined in clause 6.9 of the present document. |  |
| EVSSampleEntry |  | Entry type for EVS samples defined in clause 6.14 of the present document. |  |
| H263SampleEntry |  | Entry type for H.263 visual samples defined in clause 6.6 of the present document. |  |
| AVCSampleEntry |  | Entry type for H.264 (AVC) visual samples defined in the AVC file format specification in clause 5 of [20]. |  |
| TextSampleEntry |  | Entry type for timed text samples defined in the timed text specification |  |
| DIMSSampleEntry |  | Entry type for DIMS scene description samples defined in the DIMS specification. |  |
| HintSampleEntry |  | Entry type for hint track samples defined in the ISO specification. |  |
| CVOSampleEntry |  | Entry type for CVO timed metadata track as defined in clause 6.11 of the present document |  |
| HEVCSampleEntry |  | Entry type for H.265 (HEVC) visual samples defined in the H.265 (HEVC) file format specification in clause 8 of [20] or in clause 9 of [20], for which the use of L-HEVC Configuration record is disallowed, the sample entry types other than 'hvc1' and 'hvc2' are disallowed and track reference types other than 'scal' are disallowed. |  |
| LocationSampleEntry |  | Entry type for Location timed metadata track as defined in clause 6.12 of the present document |  |
| QualityMetricsSampleEntry |  | Entry type for Quality metrics timed metadata track as defined in clause 4 of [53] |  |
| OrientationSampleEntry |  | Entry type for Orientation timed metadata track as defined in clause 6.13 of the present document |  |

==== END OF THIRD CHANGE====