3GPP SA4 Video SWG Telco on FS_VRQOE                                                 AHVIC-214

18th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | <CR#> | rev | <Rev#> | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rendered viewport metric |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, InterDigital Communications, Inc., Fraunhofer HHI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2019-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With approval of the work item VRQoE, definition of new QoE metrics for VR services is necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal of new rendered viewport metric partly based on MPEG-I Part 6 WD metrics proposal |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================== CHANGE 1  ==============================

### 9.3.x Rendered viewports

The rendered viewports metric reports a list of viewports that have been rendered during the media presentation.

The client shall evaluate the current viewport gaze every X ms and potentially add the viewport to the rendered viewport list. To enable frequent viewport evaluations without necessarily increasing the report size too much, consecutive viewports which are close to each other may be grouped into clusters, where only the average cluster viewport data is reported. Also, clusters which have too short durations may be excluded from the report.

The viewport clustering is controlled by an angular distance threshold D. If the center (i.e. the azimuth and the elevation) of the current viewport is closer than the distance D to the current cluster center (i.e. the average cluster azimuth and elevation), the viewport is added to the cluster. Note that the distance is only compared towards the current (i.e. last) cluster, not to any earlier clusters which might have been created.

If the distance to the cluster center is instead equal to or larger than D, a new cluster is started based on the current viewport, and the average old cluster data and the start time and duration for the old cluster is added to the viewport list.

Note that as the cluster center might move a bit for each new viewports which is added to the cluster, some viewports which were added early to the cluster might actually be slightly outside the distance of D from the final cluster center. However, these viewports are still considered to be part of the cluster.

Before reporting a viewport list, a filtering based on viewport duration shall be done. Each entry in the viewport list is first assigned an "aggregated duration" equal to the duration of that entry. Then, for each entry E, the other entries in the viewport list are checked. The duration for a checked entry is added to the aggregated duration for entry E, if the checked entry is both less than T ms away from E, and closer than the angular distance D from E.

After all viewport entries have been evaluated and have received a final aggregated duration, all viewport entries with an aggregated duration of less than T are deleted from the viewport list (and thus not reported). Note that the aggregated duration is only used for filtering purposes, and not itself included in the viewport list reports.

The observation points needed to calculate the metrics are:

- OP3 Sensor: Gaze information

- OP5 VR Application: Field-of-view information of the device

The viewport sample interval X (in ms), the distance threshold D (in degrees), and the duration threshold T (in ms) can be specified during metrics configuration as attributes within parenthesis, e.g. "RenderedViewports(X=50,D=15,T=1500)". If values are not specified, the default values used shall be X=1000, D=0, T=0. These default values will result in a report with the viewport logged every 1000 ms (and no clustering or exclusion due to duration).

The rendered viewports metric is specified in Table 1.

Table 1 Rendered viewports metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| RenderedViewports |  |  | List | List of rendered viewports |
|  | Entry |  | Object |  |
|  |  | startTime | Media-Time | Specifies the media presentation time of the first played out media sample when the viewport cluster indicated in the current entry is rendered starting from this media sample. |
|  |  | duration | Integer | The time duration, in units of milliseconds, of the continuously presented media samples when the viewport cluster indicated in the current entry is rendered starting from the media sample indicated by startTime. "Continuously presented" means that the media clock continued to advance at the playout speed throughout the interval. |
|  |  | viewport | ViewportDataType | Indicates the average region of the omnidirectional media corresponding to the viewport cluster being rendered starting from the media sample time indicated by startTime. |

=========================== END OF CHANGE 1  ==========================