Source:	Orange

Title:	Draft CR – Correction of EVS SID update rate

## Document for:	Agreement

## Agenda Item:	7.3

The present document is a continuation of S4-181118. The only change is the revision of Section 3, including draft CR options in Annexes.

## Introduction

The SID update scheme in EVS is described in TS 26.445, clause 5.6.1.1. SID frames in EVS Primary modes are transmitted at fixed or adaptive rate and the fixed/adaptive behaviour is controlled by a command line parameter ('dtx') at the encoder. There are two options defined in the command line:

-dtx D           : Activate DTX mode, D = (0, 3-100) is the SID update rate

where 0 = adaptive, 3-100 = fixed in number of frames,

default is deactivated

-dtx             : Activate DTX mode with a SID update rate of 8 frames

When a value D is supplied, the SID update rate is either adaptive (D=0) or fixed (D = 3 to 100).

When the rate is adaptive (D=0), the actual rate is limited to the range 8-50; this rate is signal dependent, based on estimated background noise characteristics (e.g. SNR).

The objective of this contribution is to trigger discussion on the expected/actual usage of the DTX operation in EVS deployments and to investigate if any specification needs to be updated. This issue has never been openly discussed in SA4 and it may be time to address this topic.

## Discussion

We make the following observations that may help discussion:

- For AMR and AMR-WB, the SID update is deterministic: there is a specified sequence of SID_FIRST and SID_UPDATE with 1, 3 and 8 frame intervals. EVS AMR-WB IO follows the same SID update scheme as AMR-WB for interoperability reasons. For EVS Primary modes, the situation is different.

- There is no SDP parameter in the EVS RTP payload format that may allow controlling the SID update behaviour. Therefore, this feature is out of control at call setup/negotiation and there may be different settings deployed / used in the field.

- EVS with DTX activated was formally tested in 3GPP subjective tests only with the '-dtx' option. The adaptive update feature or fixed update rate other than 8 frames have not been evaluated in selection/characterization tests.

- The possibility that different EVS clients to be configured with different SID update rate settings should not imply any interoperability issues, given that the specified EVS decoder should be able to handle DTX operation, irrespective of the SID update rate.

- The SID update rate may have system impacts, in particular for VoLTE : delay-based schedulers typically take into account the SID transmission period to allocate grants every x ms (where x=160ms in silence state and 20 ms or a multiple for the talk state).

- In a bidirectional communication between two phones A and B, if phone B receives non-SID packets and sends SID packets, and if phone B wants to request a bit rate change by CMR, this CMR request should be sent as soon as possible. In case of adaptive SID transmission, the delay prior to sending a CMR may go up to 1s. This may have strong impacts on the assumed feedback loop for voice.

## Proposal: draft CR options

We propose two possible options:

- Change EVS core specifications (TS 26.445 / 26.449)

See changes in Annexes A1 and A2.

Note that additional corrections to source code (TS 26.442 / TS 26.443) would also be required, however such changes need to be prepared only if option A is agreed.

- Change the usage of EVS in MTSI in TS 26.114 (possibly also TS 26.223 for IMS-based telepresence)

See changes in Annex C.

Note that Option B does not cover the profiling of MTSI in GSMA IR.92. Indeed, IR.92 directly refers to EVS core specifications for EVS support.

Annex A1: Change to TS 26.445

---- beginning of first change ---

### 4.4.3	DTX/CNG operation

The codec is equipped with a signal activity detection (SAD) algorithm for classifying each input frame as active or inactive. The CNG update rate is fixed to 8 frames.

---- end of first change ---

---- beginning of second change ---

### 5.6.1	Overview

This subclause describes the discontinuous transmission (DTX) scheme and the comfort noise generation (CNG) algorithm. The DTX/CNG operation, which is activated on a command line, is used to reduce the transmission rate by simulating background noise during inactive signal periods. The regular DTX/CNG modes are supported for bit rates up to 24.4 kbps. For higher bit rates, the EVS codec supports a less aggressive DTX/CNG scheme that only switches to CNG for low input signal power.

The reduction of the transmission rate during inactive periods is achieved by coding the parameters referred to as comfort noise (CN) parameters. These parameters are used at the decoder to regenerate the background noise as well as possible, by respecting the spectral and temporal content of the background noise at the encoder. In the EVS Codec, the CNG algorithm reproduces high quality comfort noise by choosing between a linear prediction-domain based coding mode (LP-CNG) and a frequency-domain based coding mode (FD-CNG), according to the input characteristics. Each of the two coding modes utilizes a different set of CN parameters. In the LP-CNG mode, four CN parameters are analyzed and encoded: the low-band excitation energy, the low-band signal spectrum, the low-band excitation envelope and the high-band energy, where the high-band energy is only encoded for SWB/FB input. In the FD-CNG mode, the CN parameters consisting of global gain and spectral energies grouped in critical bands. Those parameters are encoded by a vector quantizer for transmission.

When the codec is operated with the DTX/CNG operation, the signal activity detector (SAD) is used to analyse the input signal to determine whether the signal comprises an active or inactive signal (see SAD decision in subclause 6.2). Based on its analysis, the SAD generates a SAD flag, , whose state indicates whether the signal is active (= 1) or merely a background noise (= 0). When = 1, the regular encoding and decoding process is performed, as in the default option. When = 0, DTX functions are run at the encoder that transmit either a silence insertion descriptor (SID) frame or a NO_DATA frame. The SID frame contains the CN parameters, which are used to update the statistics of the background noise at the decoder, whereas the NO_DATA frame is empty. The SID frame is always encoded using 48 bits regardless the actual CNG mode operating.

Further, hangover logic, as described in subclause 6.2, is used to enhance the quality of SID frames. The hangover logic in the SAD algorithm is such that the encoder waits for a certain number of frames before switching from the active signal to inactive signal. If the background noise contains transients that force the encoder to switch from inactive signal to active signal and then back to inactive signal in a very short time period, no hangover is used.

#### 5.6.1.1	SID update

The CN parameters are transmitted at a fixed rate during inactive signal periods. The fixed rate is limited to 8 frames.

-

#### 5.6.1.2	Void

---- end of second change ---

Annex A2: Change to TS 26.449

---- beginning of first change ---

# 4	General

The present document gives an overview for the requirements of the background acoustic noise evaluation, noise parameter encoding/decoding and comfort noise generation for the Enhanced Voice Services (EVS) speech codec during Discontinuous Transmission (DTX) operation.

The present document is mandatory for implementation in all network entities and UEs supporting the EVS codec.

In the case of discrepancy between the EVS comfort noise aspects described in the present document and its ANSI-C code specification contained in [3], the procedure defined by [3] prevails. In the case of discrepancy between the procedure described in the present document and its ANSI-C code specification contained in [4], the procedure defined by [4] prevails.

A basic problem when using DTX is that the background acoustic noise, which is transmitted together with the speech, would disappear when the transmission is cut, resulting in discontinuities of the background noise. Since the DTX switching can take place rapidly, it has been found that this effect can be very annoying for the listener - especially in a car environment with high background noise levels. In bad cases, the speech may be hardly intelligible.

The present document specifies the way to overcome this problem by generating on the receive (RX) side synthetic noise similar to the transmit (TX) side background noise. The comfort noise parameters are estimated on the TX side and transmitted to the RX side at a regular rate when speech is not present. This allows the comfort noise to adapt to the changes of the noise on the TX side.

The Enhanced Voice Services (EVS) speech codec supports two Comfort Noise Generation (CNG) schemes, a linear prediction based CNG (LP-CNG) as well as a frequency domain based scheme (FD-CNG). The selection of the one of the two schemes is performed within the transmit side functions on an input signal bases. The parameters for generating the comfort noise are packed as a Silence Insertion Descriptor (SID) payload.

The update rate of the SID payload is limited to a fixed number of 8 frames.

As the functions of the CNG processing are highly integrated into the speech codec and make use of other coding parameters, The present document only provides an overview of the functions. The relevant references to the algorithmic descriptions are provided in the following.

---- end of first change ---

--- beginning of second change ---

## 5.1	EVS primary CNG operation

The CNG operation on the transmit side is described in figure 1. In DTX operation, the SID updater decides that a SID update is required every 8 frames. If the SID payload is required, the CNG Selector enables either the LP or the FD CNG encoder depending on the signal characteristic of the background noise. The selected CNG encoder calculates the noise generation parameter and creates the related SID payload.

Figure 1: Transmit side Comfort Noise Generator functions

In [2], all TX functions are described in clause 5.6. The algorithmic description of the

-	SID Update mechanism can be found in sub-clause 5.6.1.1 and 5.6.1.2

-	CNG Selector can be found in sub-clause 5.6.1.3

-	LP-CNG encoder in sub-clause 5.6.2

-	FD-CNG encoder in sub-clause 5.6.3.

--- end of second change ---

Annex B: Change to TS 26.114

--- beginning of first change ---

#### 5.2.1.4	Detailed codec requirements, EVS

When the EVS codec is supported, the SID update of EVS Primary shall be limited to a fixed interval of 8 frames.

When the EVS codec is supported, the MTSI client in terminal may support dual-mono encoding and decoding.

When the EVS codec is supported, EVS AMR-WB IO may serve as an alternative implementation of the AMR-WB codec, [125]. In this case, the requirements and recommendations defined in this specification for the AMR-WB codec also apply to EVS AMR-WB IO.

--- end of first change ---