Source:	Nokia Corporation

Title:	Considerations on object-based audio metadata for IVAS

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

The IVAS Design Constrains [1] specify support for an object-based audio format. However, the current definition is still lacking. It is understood that object-based audio should consist of individual streams (that could be mono) and some metadata parameters (TBD).

This input document discusses some potential metadata parameters for the object-based audio format. Furthermore, it is discussed the need to define the format based on use cases relevant for IVAS (where further input can still be expected) and, if possible, ways to provide interoperability with other IVAS audio formats. The latter can be useful to efficiently deal with various combinations of input formats. It is the expectation of the source that it is particularly the object audio that will be used in conjunction with some other audio format at the IVAS encoder input and at the renderer.

# 2. Considerations for the object-audio format

While object-based audio is commonly used in areas such as entertainment and broadcast, using object audio in conversational codecs is new. It therefore seems important to consider in this scope the whole chain from capture of object audio (or other object-based audio generation), via the encoding (with support for an appropriate range of bit rates) all the way to rendering of the object audio either alone or as part of an overall sound scene in combination with some other audio format. Considering each step is likely to provide information on what is possible and useful for this new IVAS audio format.

The source considers that object audio metadata for IVAS can be divided roughly into two categories that will ideally be based on slightly different requirements/considerations. The categories are:

- Rendering metadata. This metadata describes the rendering of the waveform(s). Typical examples include, e.g., spatial direction or position, spatial extent or size, etc. These metadata parameters can most likely be defined based on views of audio experts, results from experiments, etc.

- Control metadata. This metadata describes how the object audio can be used. For example, it may be indicated that an object audio can be switched on/off or its volume relative to an overall sound scene can be freely controlled. The control metadata should preferably be based on needs identified from IVAS-specific use cases.

In addition, some more broadcast style metadata that would perhaps not fit key IVAS use cases could be considered (potentially in either category). A preferred outcome would still be a definition of a single object audio format for IVAS that meets specific requirements identified based on conversational codec use cases.

One key aspect relating to the size of the metadata and, on the other hand, precision/quality of the representation is the metadata parameter update rate. At least some parameters, like those relating to the position of the object, should greatly benefit from exceeding the IVAS framesize update rate (20ms). Nevertheless, it is the opinion of the source that the object audio metadata should be in any case packetized/represented such that it utilizes the same framesize as the codec.

A further consideration is potential IVAS support for 6DoF. It should appear rather obvious that object audio would be among the enablers for such technology. A third category of object audio metadata could be relevant:

- 6DoF metadata. This metadata describes properties and features that enable 6DoF use. For example, this metadata can relate to higher interactivity between the user and the audio environment than what is generally applicable to 3DoF use. The 6DoF metadata should preferably be based on needs identified from IVAS-specific 6DoF use cases such as augmented reality (AR).

It may also be attractive to consider some points of interoperability between the IVAS audio formats. This is a relevant aspect especially for object-based audio, since object audio may often be used in combination with some other input format. Thus, for example downmixing or quantization within the IVAS encoder might benefit if the formats could share some properties. The audio formats for which such properties could be considered include at least the already agreed formats, channel-based audio and Ambisonics (FOA), and formats that are under discussion, such as Ambisonics (HOA), MASA, and HTF.

Final aspects that should be considered for object audio are the number of individual object audio streams and the total number of audio streams as needed to be supported as combinations of the input audio formats.

# 3. Discussion on some potential parameters

Table 1 presents one proposal on metadata for object audio. The metadata set relates to parameters affecting the acoustic rendering of the object audio, e.g., spatial position, gain and so on. This list is not considered complete, and discussion and further input is invited.

Table 1. Proposed metadata for object audio

| Field | Bits | Description |
| --- | --- | --- |
| Direction index | 16 | Directional part of the location of the object sound source in space related to the listener. Spherical representation at about 1-degree accuracy. Range of values: “covers all directions at about 1° accuracy” |
| Distance | 8 | Distance of the sound originating from the direction index in meters on a logarithmic scale. Range of values: for example, 0 to 100 m. |
| Gain | 8 | Mixing gain for the object. Applied in synthesis and intended for allowing input, transmission, and synthesis with optimal SNR. Range of values: [0.0, 1.0] |
| Spatial extent | 8 | Cue for synthesizing the spatial extent of the object. Parameter signifies range from point source to completely surrounding the listener. |

The proposed ‘direction index’ corresponds to what has been agreed for MASA format spatial metadata. Assuming that IVAS will support MASA, it seems useful from a codec designer’s point of view to utilize this existing toolset/method for communicating the object audio stream’s spatial direction to the encoder. Such reuse can, e.g., significantly simplify downmixing in case of having a combination of audio formats as inputs. Similarly, this type of “audio format interoperability” could be considered, e.g., between channel-based audio and object audio. A potential argument against the proposal would be if the 1-degree spatial accuracy is not considered sufficient for object audio.

# 4. Conclusion

This input presented some thoughts and considerations on object-based audio and its metadata for use with the IVAS codec. Some metadata specifically relating to the spatial representation and rendering of the object audio was already presented as “proposal”. However, the source feels that it is probably still early to try to agree on a detailed set of metadata.

The source hopes to see IVAS-specific use cases for defining the IVAS object audio format. This approach should help to build common understanding on what is required for the format especially for practical conversational applications and potentially some more advanced use cases (including 6DoF).

# References

[1]	Tdoc S4-181218: IVAS Design Constraints (IVAS-4), v0.0.6.