3GPP TSG-SA4 Meeting #102	S4-190134

# Bruges, Belgium, January 28 – February 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0460 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use of default EVS SID update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2019-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SID frames in EVS Primary modes are transmitted at fixed or adaptive rate and the fixed/adaptive behaviour is controlled by a command line parameter ('dtx') at the encoder. There are two options defined in the command line: -dtx D : Activate DTX mode, D = (0, 3-100) is the SID update rate where 0 = adaptive, 3-100 = fixed in number of frames, default is deactivated -dtx : Activate DTX mode with a SID update rate of 8 frames When a value D is supplied, the SID update rate is either adaptive (D=0) or fixed (D = 3 to 100). When the rate is adaptive (D=0), the actual rate is limited to the range 8-50; this rate is signal dependent, based on estimated background noise characteristics (e.g. SNR). EVS implementations may use update rates that are not the default behaviour to be used. This would cause extra testing, potential impacts on radio scheduling (if not dynamic) or performance (CMR updates) and even transcoding (if strict compliance with the default behaviour is expected). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The default update rate for EVS SID frames (fixed rate of 8 frames) is mandated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential impacts on radio scheduling (if not dynamic) or performance (CMR updates) and even transcoding (if strict compliance with the default behaviour is expected). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.1.4	Detailed codec requirements, EVS

When the EVS codec is supported, it shall use the default mode of DTX/CNG operation, i.e. the SID update of EVS Primary shall be limited to a fixed interval of 8 frames.

When the EVS codec is supported, the MTSI client in terminal may support dual-mono encoding and decoding.

When the EVS codec is supported, EVS AMR-WB IO may serve as an alternative implementation of the AMR-WB codec, [125]. In this case, the requirements and recommendations defined in this specification for the AMR-WB codec also apply to EVS AMR-WB IO.

- End of changes