Source:	Ericsson LM

Title:	Network modification of AMR-WB mode-set

Document for:	Discussion and Agreement

Agenda Item:	11.11

# 1. Introduction

GSMA RILTE group sent SA4 a liaison on on-path SDP modification of AMR-WB mode-set parameter [1]. This document provides some additional background and proposes a possible way forward in 3GPP SA4.

# 2. Background

This document assumes a transcoding-free scenario, since use of transcoding effectively makes codec usage independent between the involved parties and SDP alignment is then not necessary. The term “transcoding” should here also be taken to mean any type of RTP-level translation, such as replacement of RTP/RTCP header payload type or SSRC numbers.

The AMR / AMR-WB RTP payload format and SDP parameter specification [2] and TS 26.114 [3] specify rules for SDP negotiation, and how to create SDP offers and answers for AMR and AMR-WB.

The AMR / AMR-WB payload format has several parameters that controls the codec operation on RTP. One of those parameters is the “mode-set”, which restricts the active codec mode set to a subset of all modes. The value is a comma-separated list of modes from the set 0, …, 7 (for AMR) or 0, …, 8 (for AMR-WB). If mode-set is specified for the payload type, it must be abided. If mode-set is not present, there is no restriction and all codec modes are allowed for the payload type.

If there is a mode-set parameter in the SDP offer and if that payload type is chosen to also be part of the SDP answer, the received mode-set must be kept unchanged in the SDP answer. If there is no mode-set parameter in the SDP offer, one may be added in the SDP answer. The parameter is bi-directional, i.e., the restricted set applies to media both to be received and sent by the SDP-declaring entity (offerer or answerer). The mode-set in the SDP answer is binding for both offerer and answerer.

To the best of the author’s knowledge, these SDP offer/answer rules for mode-set were chosen to limit the number of options and SDP offer/answer complexity. Since the number of theoretically possible mode-sets for SDP offerer and answerer is quite large, 255 for AMR and 511 for AMR-WB, rule-based handling is the only practical option. Not allowing any changes to a provided mode-set is a clear and simple rule.

However, when both SDP offerer and answerer have mode-set preferences, the current rules would favour the offerer by letting it choose mode-set since it is the initial SDP creator. This imbalance is partly handled in [2] by recommending that an offerer supporting all modes and subsets should not include any mode-set in the SDP offer. That would suggest that mode-set is included in the SDP offer only by entities that cannot restrict it further such as transcoding-free CS gateways, or when the SDP is specifically configured in preparation for connection to such gateways. An SDP answerer would however be allowed to add a mode-set in the SDP answer for a received offer without mode-set, which instead favours the SDP answerer to select mode-set. The case when neither SDP offerer nor SDP answerer can modify their mode-set with different mode-set preferences effectively favours the SDP offerer to choose.

The realization in GSMA that triggered the liaison was that there are not only SDP offering and answering UEs that could have different mode-set preferences, but also offering and answering (and possibly intermediate) networks with additional, different mode-set preferences. This means that both the SDP offer and the SDP answer typically pass multiple back-to-back SIP user agents (B2BUA) with the potential permission and ability to modify SDP before reaching its destination. Currently, no explicit rules exist that would govern this type of on-path modification of SDP. It is fully possible that both originating and terminating networks are limited in terms of mode-set but with different preferences, and there is no description in existing specifications how to resolve that.

The below figures describe a few scenarios with two networks that have different AMR-WB mode-set preferences and that the preference differences can lead to success or failure depending on how SDP modifications are made by the networks.

Figure 1 Reducing mode-set in SDP offer ("Flow 1" of GSMA liaison)

In Figure 1 above, network 1 prefers a larger mode-set than network 2, and network 2 is here allowed to further reduce the mode-set. This results in a joint understanding what mode-set is to be used, expressed by the SDP answer that is seen by both offerer and answerer (and both intermediate networks).

Figure 2 Reducing mode-set in SDP offer ("Flow 2" of GSMA liaison)

Figure 2 describes the case when a UE includes both a payload type without mode-set and a payload type with mode-set in the offer. In this case, network 1 identifies the payload type with the UE-provided mode-set as identical to its preference and keeps only that one. Since network 2 makes further modification, that eventually results in the offering UE observing a violation in the SDP answer of the rule in [2] to not modify a provided mode-set between SDP offer and answer.

Figure 3 Reducing mode-set in SDP offer ("Flow 3" of GSMA liaison)

Figure 3 is very similar to Figure 2, but with the difference that the UE-provided payload type without a mode-set is chosen by network 1 to add a mode-set to, removing the payload type that had a mode-set from the UE. Like Figure 1, this also results in a joint understanding and a workable result.

Figure 4 below provides an example of a failing approach that should therefore be avoided; both networks enforce their mode-set preferences, regardless if mode-set is then reduced or increased, in both offer and answer, which results in disagreement between offerer and answerer what mode-set is to be used and a risk for media (RTP) level incompatibility.

Figure 4 Enforcing mode-set in offer and answer

The CMR in 3-8 range that the answerer thinks are OK to send, are in conflict with the SDP seen by the offerer in network 1. Those CMR could be replaced with (e.g.) CMR 2 by network 1, but that would require inspection and possibly modification of every RTP packet.

To summarize, the approach suggested by the GSMA liaison in [1] is to find a common subset of modes that are acceptable to all parties, by:

- Utilizing and explicitly describing the existing possibility that the network (a B2BUA) may add a mode-set on-path in the SDP offer, in case a mode-set parameter is not already provided.

- Defining that further restricting a provided mode-set is allowed, such that the lowest mode may be increased, and the highest mode may be decreased. In that, they assume that the provided mode-set is a contiguous range of modes (e.g. “0-8”).

Editor’s note 1: As can be seen from above, the mode-set parameter in SDP is in general not a range, but an explicit list of modes that may well be non-contiguous. Therefore, clarification is needed that “increase lowest mode” means “consecutively remove modes from the set lower end”, and “decrease highest mode” means “consecutively remove modes from the set upper end”, and that removing modes from the middle part of an existing set is not allowed.

- Allowing on-path modification of the mode-set in the SDP offer for the case where that mode-set was added by the network and not by the originating UE.

  - To distinguish when a mode-set is added by the UE and is non-modifiable, and when the mode-set is added by a network and is modifiable, a new AMR / AMR-WB payload type parameter (meaning “Provided by the network”, e.g. “np”) is introduced on the SDP “a=fmtp” line.

Editor’s note 2: Some use of the mode-set parameter is due to interworking with CS networks, where the used mode-set is fixed, but where the mode-set upper end in some cases can be further reduced as outlined above, e.g. for transcoding-free interworking between AMR full-rate and AMR half-rate. In that case, SDP is either originated by or terminated in a gateway, which is an MTSI client in network (not MTSI client in terminal) that is acting either directly on behalf of a UE on CS access, or in preparation for a UE that may move to CS access.

  - That new payload type parameter (“np”) would be added by a network when adding a mode-set to a payload type.

  - The new payload type parameter (“np”) should be removed from the SDP by the last network before reaching the terminating UE or a non-3GPP network, to avoid interoperability problems with UE or external networks that don’t understand this parameter.

The description on finding a common subset of acceptable modes provided by GSMA in the liaison and summarized by 1-3 above seems mostly feasible, but should be further complemented with:

- Only SDP offer-to-offer mode-set changes are permitted. The mode-set in SDP answer must never be changed by any B2BUA.

  - Changing the SDP answer in network nodes would make the SDP answerer (the UE sending the SDP answer) and the SDP offerer (the UE receiving the SDP answer) disagree on what mode-set to use in the session. The SDP answer is binding for both SDP offerer and SDP answerer, but the SDP answerer cannot know what to be bound by if it doesn’t have the “final revision” of the SDP answer.

  - An SDP answer with a mode-set that is further restricted when reaching the SDP offerer than when it was sent from the SDP answerer would cause the SDP answerer to use a larger mode-set in RTP packets that what the SDP offerer expects from the received SDP answer.

- When the original offer from the UE contains both payload types without mode-set and payload types with mode-set, the network must not mark a payload type with mode-set as allowed to modify, even if that provided mode-set is identical to a network-preferred mode-set that the network would add to a payload type without mode-set.

  - If a UE-provided mode-set in the SDP offer is marked by a network as allowed to modify, it may be further modified by other networks, which would break the rule in [2] to not modify UE-provided mode-sets, which will be discovered by the offering UE when receiving the SDP answer.

- Increasing the lower end of the mode-set is not permitted.

  - Increasing the mode-set lowest mode would reduce the possibilities to back off to lower modes in case of congestion, and it also prevents transcoding-free operation with CS networks as it makes CMR ineffective during transcoding-free interworking with CS. CMR becomes ineffective because use of lower modes in the mode-set than what is indicated by CMR are generally permitted, which would no longer be sufficient when low modes are removed as the mode-set low end is increased.

- If a further reduction of mode-set to the common modes results in the empty set, such as e.g. finding the intersection of mode-set 0,1,2 and mode-set 8, the entire payload type and all references to it (e.g. by a= lines) must be removed from the SDP.

  - If that was the only payload type on the m= line, transcoding resources should be added to the call instead of removing the payload type, allowing incoming and outgoing SDP offers to be different. An m= line without any payload types violates SDP, is syntactically broken and must be disabled (by setting port to 0), which may in turn cause the entire call to fail.

- It is unlikely that the suggested, new payload type parameter to indicate that the mode-set is network-provided (“np”) can be guaranteed to never reach the UE (MTSI client in terminal, or an MTSI client in network such as a CS gateway) or an external non-3GPP network.

  - The new payload type parameter doesn’t change the existing offer/answer semantics but only impacts what is permitted when making offer-to-offer changes. Therefore, it should strictly not matter if it reaches an MTSI client (UE or CS gateway) that knows the semantics of the parameter. Any SDP changes in the answering MTSI client are offer-to-answer changes that are already regulated by existing text in [2] and [3].

  - From a specification perspective, it should not matter if the new parameter reaches a UE or network that is not aware of this new parameter, since section 8.3.1 “Offer-Answer Model Considerations” of [2] requires that “Any unknown parameter in an offer SHALL be removed in the answer”, which indicates that any unknown parameter will be ignored by a conformant implementation. It is however unclear if this behaviour is enforced or even tested in implementations. It is still possible that some badly designed implementations may break if they receive this so far unspecified and therefore unexpected parameter.

- Any modifications made to AMR-WB mode-set SDP handling should also be made for EVS AMR IO mode.

- Is finding a common, minimum set of mode-set modes to use in the session the best approach, or should the restricted mode-set instead be limited to certain, well-known values (such as “0,1,2” for AMR-WB) to increase the probability for successful interoperation?

# 3. Proposal

- Agree on the SDP-modifying principles described above, amended with the outcome of any discussion during the S4#103 meeting and document that in a CR to TS 26.114 Rel-16 and onwards.

Editor’s note: What category CR would that be? Correction (F), or is the topic substantial enough to require being described as a new feature?

- Send a liaison answer to GSMA RILTE, describing the technical outcome and SA4 planned actions.

- Send a liaison to CT1, CT3, and CT4, describing the new mode-set modification rules for B2BUA, to allow CT to comment and make corresponding modifications in their specifications as needed.

# 4. References

[1]	S4-190286, LS 3GPP SA4 on Codec Mode-Set, GSMA RILTE (RILTE68_104), March 2019

[2]	IETF RFC 4867, RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs, https://tools.ietf.org/html/rfc4867, April 2007

[3]	3GPP TS 26.114, IMS Multimedia Telephony; Media handling and interaction (Release 16), December 2018