Source:	Nokia Corporation

Title:	Proposal for IVAS MASA Channel audio format parameter

Document for:	Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Background

Metadata-assisted spatial audio (MASA) has been proposed as one spatial audio input format that the future IVAS codec shall support. This is currently captured in IVAS Design Constraints [1] as “[Spatial audio, [N] channels and spatial metadata defined by [TBD].]”. Such format is seen crucial for providing uncompromised spatial audio quality from many practical multi-microphone audio capture devices, particularly smartphones. The discussion about adopting MASA remains ongoing with a few open questions. One such question is how to define additional metadata relating to the N channels. This includes specifying number of the channels, N, itself.

At SA4#102, the source presented discussion on the information that should be conveyed as part of the format [2]. In particular, the ‘Channel audio format’ parameter was considered to describe information that is either necessary for encoding MASA format (e.g., number of channels and their meaning), can enable new use cases (e.g., some transport channel definitions), or can help improve encoding or rendering quality in some cases (e.g., channel distance).

In the present input document, the ‘Channel audio format’ is further discussed. This considers feedback from the Bruges meeting as well as development of use cases relating to the more advanced features.

# 2. Number of channels

The number of channels supported by the MASA format is an open question. While it has been primarily considered 1-2 channels for the MASA format (as these directly relate to mobile device capture of spatial audio), it has also been brought up in various discussions that a higher number of channels (including at least 3-4 channels) could be considered for the format. This would allow using, e.g., planar FOA or FOA component signals with MASA spatial metadata, which could be useful for some device categories or in the future. The source however notes that there has been no practical justification for 3-4 channels.

In [2], it was considered other uses for 3-4 channels besides various forms of FOA. In the present discussion, an example use case is presented to illustrate the more creative uses for a higher number of channels based on combinations of more than one mono signals, mono and stereo signals, or more than one stereo signals.

In addition, it is considered an extension of the input format based on practical requirements from the acoustic design of multi-microphone devices.

## 2.1 Use case: Audio focus

In addition to performing a spatial analysis, a spatial audio capture processing can include a beamforming operation. For example, a user can be capturing spatial audio in a noisy environment such as near a busy street. For example, the user can face away from the street and capture two street performers playing instruments. For example, this can be a video capture on a smartphone with spatial audio capture capability. In order to reduce the background noise, the user can apply on their device an audio focusing or beamforming operation, where the audio corresponding to the field of view of the video capture is made louder relative to the background noise from the street. This capture could be provided, e.g., as ‘stereo signal + MASA metadata’ into the IVAS encoder.

In the above example, it is perhaps sufficient to send only this processed capture. In fact, it can be considered that it is not even interesting for the recipient to know there is significant street noise behind the capture point. But what if there is something interesting? Instead of next to a busy street, the user could be capturing the performance in a city park with a more pleasant background with birds singing in the trees and children laughing. The user capturing the scene might still prefer to focus on the performance, but for the recipient it could be preferable to hear the unprocessed scene. To allow this, it seems the capture should be able to provide for the IVAS encoder two stereo signals along with the MASA metadata.

In practice, this type of beamformed audio signal could be a mono focused signal for a corresponding unprocessed mono signal, and a mono or stereo focused signal for a corresponding unprocessed stereo signal. When a single beamforming operation is performed, it is sufficient for the recipient to differentiate between ‘on’ and ‘off’ or, e.g., a slider between these states could be provided in the user interface to allow for a finer granularity in mixing.

The audio input can thus be described as two audio signals that are intended for mixing together at the decoder/renderer based on a user input via a user interface on the UE. This description provides an example use case (user-controllable beamforming or audio focus) and specifies the intention (user-controllable mixing of effect), not the practical solution or how it should be implemented in the encoding.

The MASA format enables this as follows:

- It is used 3 or 4 transport signals

- For three signals (‘Ch = 10’), Transport channel definition is set as ‘01’

- For four signals (‘Ch = 11’), Transport channel definition is set as ‘01’

Additionally, it could be considered, e.g., audio focus using two focus directions or two focus-level pre-set values, where it is used ‘3 mono (mixed)’ or ‘Stereo + 2 mono (mixed)’.

## 2.2 Alternative downmix

The source considers that it can be useful to be able to simultaneously provide both a mono representation and a stereo representation as part of the MASA format. For example, this can be beneficial for bit rate adaptation in the encoder. In principle, a valid mono downmix could be performed in the encoder based on the stereo input. Spatial audio capture on a practical device can however be based on a rather arbitrary multi-channel capture configuration, where the manufacturer of the device designs the audio capture algorithms best suited for their setup. Creating the optimal downmix for stereo and mono can in many cases use different microphone signals. If the downmix is left for the IVAS encoder, this design property is not followed. A simple example is a device with three main microphones, where the three main microphones constitute a row. For stereo, the left and right channels may be selected from the corresponding left and right microphones, while the optimal mono pickup may be based on the center microphone.

It is understood that the creation of the channels for the MASA format can be based on direct use of the microphone capture signals (where some processing is assumed similarly as for regular mono and stereo inputs) or a specific downmix processing that combines microphone capture signals.

In case a device configuration is best suited to provide a single audio channel as part of the MASA format (e.g., the device has a single high-quality microphone with an additional array for spatial analysis), a mono input may be sufficient. In all other cases, it may be a manufacturer choice to provide a single channel (mono), two channels (stereo), or multiple channels (e.g., mono + stereo) in the MASA format.

The MASA format enables this as follows:

- It is used 3 transport signals

- Transport channel definition is set as ‘10’

In addition, this capability could even be combined with the audio focus delivery. The MASA format enables this as follows:

- It is used 4 transport signals

- Transport channel definition is set as ‘10’

# 3. Updates to previous parameter description

Below it is provided some updates to the previous ‘Channel audio format’ discusion in [2].

In addition, it is added for convenience a status field to indicate whether a certain parameter is considered required (to encode/synthesize audio correctly) or only “nice to have” in terms of additional flexibility and/or potentially improving encoding or rendering quality. The status field is coded as follows: R = required, O = optional. If a certain feature is supported, some optional fields become required: O/R.

Square brackets are used in case of 3-4 MASA channels to highlight the extent to which the number of channels affects the proposed ‘Channel audio format’ parameter set.

Number of channels (2 bits)

This parameter field tells how many transport channels are used for the MASA format. This parameter is required in some form to read correct number of channels. As the MASA format consist of channels and metadata, it is most practical to provide the information on number of transport channels as part of the format.

| Bit value | Decoded value | Additional description | Status |
| --- | --- | --- | --- |
| 00 | 1 channel |  | R |
| 01 | 2 channels |  | R |
| 10 | 3 channels | (Intended for planar FOA or additional features) | [R] |
| 11 | 4 channels | (Intended for FOA or additional features) | [R] |

Transport channel definition (2 bits)

This parameter field describes what the transport channels contain. Decoded value depends on the number of channels field. First bit value (00) is the default value for each group, i.e., for 2 channels a L/R stereo is the default and so on.

At least it should be differentiated between regular stereo and binaural audio for 2-channel input, which corresponds to related input format requirements in IVAS-4 [1].

Update: For three transport channels case ‘stereo + mono (alternative)’, text on ‘provision for user input at renderer’ is removed. For four transport channels, the case ‘stereo + stereo (alternative)’ is replaced by a combination of the two three channel cases relating to mixing and alternative channel inputs.

| Ch bit value | Bit value | Decoded value | Additional description | Status |
| --- | --- | --- | --- | --- |
| 00 | 00 | Mono |  | O |
|  | 01 | Reserved |  | O |
|  | 10 | Reserved |  | O |
|  | 11 | Reserved |  | O |
| 01 | 00 | L/R stereo |  | R |
|  | 01 | Binaural |  | R |
|  | 10 | 2 mono (mixed) | The two signals are mixed in synthesis. Provision for user input at renderer. | O/R |
|  | 11 | 2 mono (alternative) | The two signals are alternatives, and only one signal is used in synthesis. Default to be used is the first of the two mono signals. Provision for user input at renderer. | O/R |
| 10 | 00 | Planar FOA | FOA signal with only two horizontal dipoles present | [O/R] |
|  | 01 | Stereo + mono (mixed) | The two signals are mixed in synthesis. Provision for user input at renderer. | [O/R] |
|  | 10 | Stereo + mono (alternative) | The two signals are alternatives (corresponding to stereo and mono downmix of the original capture), and only one signal is used in synthesis. Default to be used is the stereo signal. | [O/R] |
|  | 11 | 3 mono mixed | The three signals are mixed in synthesis. Provision for user input at renderer. | [O/R] |
| 11 | 00 | FOA | Standard FOA | [O/R] |
|  | 01 | Stereo + stereo (mixed) | The two signals are mixed in synthesis. Provision for user input at renderer. | [O/R] |
|  | 10 | Stereo + mono (alternative) + mono (mixed) | The first two signals are alternatives (corresponding to stereo and mono downmix of the original capture), and only one of these signals is used in synthesis. Default to be used is the stereo signal. The third signal is for mixing in the synthesis. Provision for user input at renderer. | [O/R] |
|  | 11 | Stereo + 2 mono (mixed) | The three signals are mixed in synthesis. Provision for user input at renderer. | [O/R] |

Source format (2 bits)

This parameter field describes the format of source that was used to form the MASA format. This offers additional information that can benefit encoding, decoding, and/or rendering. First bit value (00) is the default value.

| Bit value | Decoded value | Additional description | Status |
| --- | --- | --- | --- |
| 00 | Default/Other | Audio originates from unknown format(s) and possibly mixed sources | O |
| 01 | Microphone grid | Audio originates from various (irregular) microphone grids (e.g., smartphones and related devices) | O |
| 10 | Channel-based | Audio originates from premixed channel-based audio (e.g., 5.1) | O |
| 11 | Ambisonics | Audio originates from Ambisonics format | O |

Source format description (3 bits)

This parameter field provides additional information based on the source format bit values. First bit value (000) is the default value and is specified as “unknown”, i.e., there is no specific additional information.

Update: Cardioid description is modified to take into account the ORTF stereo microphone capture system.

| SF bit value | Bit value | Decoded value | Additional description | Status |
| --- | --- | --- | --- | --- |
| 00 | 000 | Unknown |  | O |
|  | 001 | Mixed: microphone + objects | Mixture of microphone source and objects | O |
|  | 010 | Mixed: microphone + channels | Mixture of microphone source and channel source | O |
|  | 011 | Object downmix | Downmix of multiple objects (e.g., talkers) | O |
|  | 100 | Mixed: general | Other mixture or mixture of multiple sources | O |
|  | 101 | Synthetic | Completely synthetic source signal (e.g., computer generated) | O |
|  | 110 | Reserved |  | O |
|  | 111 | Reserved |  | O |
| 01 | 000 | Unknown/Other |  | O |
|  | 001 | Binaural |  | O |
|  | 010 | Omni | With L/R stereo also transport channels are this | O |
|  | 011 | Cardioid ±90 deg. | With L/R stereo also transport channels are this | O |
|  | 100 | Cardioid ±70 deg. | With L/R stereo also transport channels are this | O |
|  | 101 | Cardioid ±55 deg. | With L/R stereo also transport channels are this | O |
|  | 110 | Cardioid ±45 deg. | With L/R stereo also transport channels are this | O |
|  | 111 | Cardioid ±30 deg. | With L/R stereo also transport channels are this | O |
| 10 | 000 | Unknown/Other | Unknown / Other channel-based format | O |
|  | 001 | 1.0 |  | O |
|  | 010 | 2.0 |  | O |
|  | 011 | 5.1 |  | O |
|  | 100 | 5.1+2 |  | O |
|  | 101 | 5.1+4 |  | O |
|  | 110 | 7.1 |  | O |
|  | 111 | 7.1+4 |  | O |
| 11 | 000 | Unknown/Other | Unknown / Other Ambisonics format | O |
|  | 001 | Planar FOA |  | O |
|  | 010 | FOA |  | O |
|  | 011 | HOA2 |  | O |
|  | 100 | HOA3 |  | O |
|  | 101 | HOA4 |  | O |
|  | 110 | HOA5 |  | O |
|  | 111 | HOA6 |  | O |

Channel distance (6 bits)

This parameter field currently applies only when the transport signal contains any stereo signal and the source format is 'Microphone grid’ (i.e., ‘SF = 01’). It describes the distance between the two microphones in the capture. This information can be used at least in certain synthesis scenarios to improve perceived output quality. The values between 0.01 m and 1 m are calculated as an equal multiplicative interval so that there are 60 values from 0.01 m to 1 m. The equation for this can be given as:

ddec=59100B-3100

where ddec is the decoded distance value and B is the bit value as an integer value, i.e., B=3, …, 62. The result is in meters.

| Bit value | Decoded value | Additional description | Status |
| --- | --- | --- | --- |
| 000000 | No effect | Explicit statement that no effect should be applied | O |
| 000001 | 0 m / coincident | No distance between microphones, i.e. they are coincident | O |
| 000002 | < 0.01 m | Distances smaller than 0.01 m | O |
| 000003 | 0.01 m | (Distances formed with equation above) | O |
| … | … | (Distances formed with equation above) | O |
| 111110 | 1 m | (Distances formed with equation above) | O |
| 111111 | > 1 m | Distances larger than 1 m | O |

In the current proposal, ‘Channel distance’ is not specified for the following source formats: Default/other, Channel-based, and Ambisonics. Potentially the same parameter could be defined for some of these source formats, or the 6 bits could be repurposed. Here, the default value for these source formats is bit value ‘000000’.

| Bit value | Decoded value | Additional description | Status |
| --- | --- | --- | --- |
| 000000 | Default |  | O |
| 000001 | Reserved |  | O |
| … | Reserved |  | O |
| 111111 | Reserved |  | O |

# 4. Summary and Proposal

In this input document the ‘Channel audio format’ parameter for the MASA format has been developed further. A potential use case relating to use of 3-4 transport channels in the MASA format has been described. Some clarifications and modifications over the previous discussion [2] have been made.

The source kindly asks to agree on the ‘Channel audio format’ proposal for the MASA format as described in this document (clause 3).

With regards to use of FOA for 3-4 transport channels, the source has noted that there is currently no strong justification for this capability. Nevertheless, this could be a useful part of the format to allow for future differentiation of mobile-device capture or potentially some other form factors where a high-quality Ambisonics representation cannot be obtained. In this case, it needs to be assumed that a manufacturer is able to derive some advantage over a stereo-based MASA representation.

# References

[1]	Tdoc S4-190248: IVAS Design Constraints (IVAS-4), v0.0.7.

[2]	Tdoc S4-190120: On IVAS MASA Channel audio format parameter. Source: Nokia Corporation.