Source: 	Fraunhofer IIS

Title: 	On Output Formats

Document for: 	Discussion and Agreement

Agenda Item: 	7.5 – IVAS_Codec

1. Introduction

At the previous meeting progress on the definition of output formats in the IVAS Design Constraints permanent document [1] was made. One of the open questions is which exact output format shall be supported for which input format. This contribution addresses this question.

2. Discussion

The source agrees that in general the IVAS codec shall support any of the defined rendered output formats for any input formats. However, the source still sees two points for clarification:

Multi-channel output formats: IVAS-4 doesn’t clearly specify which (of the infinite amount of) multi-channel output formats shall be supported. The source proposes to align the multi-channel output formats to the encoder-input formats and leave the optional rendering to any other output format either up to the proponents or up to an external renderer.

Output formats for channel-based Input: As already outlined the source still sees problems with mandating arbitrary output formats for channel-based input formats and sees the need of refining the requirements for these modes, especially wrt. envisioned output and testing. To overcome these issues, it is thus proposed to add a clarification that for channel-based input the output shall be reproduced as closely as possible to the original signal and is proposing to use the original channel-based input as reference signal for testing purposes. This possibly applies to mixed format inputs containing channel-based audio, too.

3. Proposed changes to IVAS-4

The following changes are proposed to the IVAS Codec Design Constraints

In section 3, apply the following change:

*** Start change 1 ***

| Renderer Output Formats | The IVAS codec shall support the following rendered output formats: - Mono - Stereo - Binaural audio - Multi-channel. All channel-based encoder input formats shall be included. Note: For channel-based audio input or mixed scenes featuring channel-based audio input signals, the channel-based input signal in the original format shall be the reference signal or shall be used as reference signal for the channel-based part of the mixed scene for testing. Editor’s Note: Corresponding output formats for each input formats is TBD Editor’s Note: Input on the definition of internal renderer and/or external renderer are invited. |
| --- | --- |

*** End change 1 ***

4. Conclusion

It is proposed to agree on the inclusion of the changes outlined above into IVAS-4.

5. Reference

[1] 3GPP S4-190248 – IVAS Design Constraints (IVAS-4), Version 0.0.7.

[2] 3GPP S4-181219, IVAS Performance Requirements (IVAS-3)