Source:	Qualcomm Incorporated

Title:	Consistency in H.264 Level 1.2 Requirement

Document for:	Discussion and Agreement

Agenda Item:	11.2

# 1 Introduction

Since Release 10, TS 26.114 has the following mandatory requirement in subclause 5.2.2:

An MTSI client in terminal offering video support other than H.264 CBP Level 1.2 shall also offer H.264 CBP Level 1.2.

However, many examples in Annex A in Release 15, such as examples in Tables A.4.13, A.4.16, A.4.19, A.4.20, A.4.21, A.6.3 and etc, do not meet this requirement. Such inconsistency might cause confusions in implementation.

# 2 Discussions

From standard point of view (Refer to [1] and [2]), a device that claims to support H.264 at level X shall support all the levels less than X. However, the above mandatory requirement was introduced since at that time, some deployed terminals rejected video entirely if the offered level was not exactly 1.2.

## 3 Proposals

To resolve the inconsistency, the followings are proposed:

Proposal A: Update the examples in Annex A to meet the requirement; or

Proposal B: Remove the mandatory requirement if we believe those legacy terminals are no longer in service.

# 4 References

[1]	ITU-T Recommendation H.264 (04/2013): "Advanced video coding for generic audiovisual services".

[2]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.