| 3GPP TR 26.801 V0.0.2 (2019-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; UEs Supporting Handset Mode with Non-Traditional Earpieces (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Report on studies with UEs featuring non-traditional earpieces	7

4.1	Challenges on conducting objective tests according to 3GPP existing specifications for UEs featuring non-traditional earpieces	7

4.2	Reports on user studies with UEs featuring Non-traditional earpieces and suitability of existing test equipment	7

5	Conclusion	7

Annex <B>: <Informative annex title for a Technical Report>	8

Annex <C> (informative): Bibliography	9

Annex <D> (informative): Index	10

Annex <X> (informative): Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# Introduction

This technical report investigates on the suitability of 3GPP existing specifications for testing UEs featuring non-traditional earpieces.

UEs featuring non-traditional earpieces may pose challenges for handset mode acoustic testing. For example, a UE may exclusively use a vibrating display to produce sound when operating in handset mode, offering no clearly identifiable centre of an earpiece to position the headset for testing. Additionally, such UE could have its acoustic response affected by the choice of handset positioner mechanism.

This technical report documents studies conducted and concludes with a summary of challenges identified and gap analysis of existing 3GPP specifications.

# 1	Scope

The present document reports on investigations of testing UEs featuring non-traditional earpieces, and identifies related gaps to existing 3GPP specifications and recommended test equipment.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

# 4	Report on studies with UEs featuring non-traditional earpieces

## 4.1	Challenges on conducting objective tests according to 3GPP existing specifications for UEs featuring non-traditional earpieces

Editor’s note: Add reports for measurements of UEs featuring non-traditional earpieces conducted according to TS 26.132 v16.0.0 and outline any challenges or uncertainties faced. Add issues related to applying P.64 to the testing.

### 4.1.1	Handset positioning on head and torso simulator

#### 4.1.1.1 Background

TS 26.132 references ITU-T Recommendation P.64 regarding handset position on artificial ears. However, UEs featuring non-traditional earpieces may not feature a centre of an acoustic port raising the question of how to properly position a handset for testing. Therefore, an update to P.64 would be required for SA4 to reference this Recommendation also for handsets with non-traditional earpieces, if the present study is followed by normative work in 3GPP.

Such update to P.64 may describe how to specify a reference point for handsets where there are no acoustic outlets in the earpiece area of the handset.

This sub-clause suggests an alternative method for positioning the handset that is suitable for UEs featuring non-traditional earpieces.

#### 4.1.1.2. Definition - within the FS_HaNTE study - of handset position for UEs featuring non-traditional earpieces

Within the context of FS_HaNTE, the MECRP is defined in terms of distance from the upper edge of the handset (this is equivalent what is sometimes referred to as “end stop” for positioning fixtures) and the distance from a centre/symmetry line.

Figure 4-1 Definition of MECRP for handsets in FS_HaNTE

ye is allowed to vary significantly to cover also the following case, or more unusual shapes:

Figure 4-2 Example of a handset shape where, due to a curvature, some extra allowance of ye is needed.

The acceptable range of offset of the MECRP should not exceed:

- No limit of ye value.

- ±10 mm along unit vector ze.

#### 4.1.1.3. Template structure - within the FS_HaNTE study - of handset position for UEs featuring non-traditional earpieces

A template for providing handset position for UEs featuring non-traditional earpieces is provided in Tables 4-1 to 4-3.

Table 4-1	MECRP

| Axis | [mm] |
| --- | --- |
| ye |  |
| ze |  |

Table 4-2	Angle Settings

| Angle | Delta from standard angle [°] |
| --- | --- |
| A |  |
| B |  |
| C |  |

Table 4-3	Application Force

| Application force [N] |  |
| --- | --- |

See ITU-T P.64 for details on standard angle.

## 4.2	Reports on user studies with UEs featuring Non-traditional earpieces and suitability of existing test equipment

Editor’s note: Add reports on user studies for UEs featuring non-traditional earpieces here. Conclude whether HATS and existing mounting apparatus are enough for testing and repeatable across vendors.

# 5	Conclusion

# Annex <B>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <C> (informative):
Bibliography

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <D> (informative):
Index

The Index is optional. If it exists, it shall immediately precede the Changes history annex.

Generate the index using MS Word's index field feature.

Annex <X> (informative):
Change history

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 08/04/19 | SA4 #103 | S4-190412 |  |  |  | Initial skeleton TR | 0.0.1 |