3GPP TSG-SA4 Meeting #103	S4-190459

# Newport Beach, CA, USA, 8. - 12. April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (pseudo)CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for 5G Uplink Media Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | <Source_if_WG> |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document introduces a more detailed UE model for Uplink Streaming and also a procedure for uplink streaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 4.3	5G Media Uplink Streaming Architecture

### 4.3.1	Media Architecture

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.3.1-1: Media Architecture for Uplink Streaming

NOTE 1: 	The yellow filled boxes can be defined by SA4. The red lines can be in scope of SA4 specifications. It is ffs, which of the in-scope interfaces and functions will be defined by SA4. The green lines are under SA2 control and SA4 may provide some usage guidelines. The black lines are not in scope of SA4.

NOTE 2: 	It is currently assumed that interfaces between Trusted UL Media Functions (i.e. in the box labelled Trusted UL Media Functions) are not defined by SA4.

NOTE 3: 	Red ovals indicate that the function exposes an API Provider. API invokers may access services, provided by this function.

Functions:

-	5G UL Media Functions (Trusted): Sender of the uplink streaming data.

Editor's note: The 5G UL Media Functions (Trusted) will be broken into smaller subfunctions.

-	External Media Functions (e.g. App): A 5G UL Media Function (Trusted) is controlled by an App, which is implementing the content producer desired behaviour. The App is not defined within 5G Media Streaming.

-	UL Media AS: Receiver of the media stream, originating from the 5G Media Source. The UL Media AS may forward the data to other UL Media ASes and even to External UL Media Functions.

- 	External UL Media Function: External Receiver of the uplink media data.

-	UL Media AF, e.g. Network Assistance (NA AF): A Media Application Function, which provides different kinds of Services.

NOTE 4: 	There may be several UL Media AFs among the Trusted UL Media Functions.

Reference Points:

### 4.3.2	UE Media Functions

Figure 4.3.2-1: UE 5G Media Uplink Streaming Functions

NOTE 1: A UE is a logical device, allowing for tethering or other type of realizations.

Definitions:

Media Streamer Entry: a pointer e.g. a URL that defines an entry point of a media uplink streaming session.

5G Media Uplink Streamer (5GMUS) includes at least: Media Upstream Client, Media Encoder(s), Media Capturing(s) and may include Media Configuration, Remote Control and Assistance Handlers. The 5GMUS receives a Media Streamer Entry. The 5GMUS captures the media on the provided input devices. The 5GMUS exposes some basic controls such as capture, pause, stop etc. to the App. via MUC1 and MUC2. The 5GMUS streamer captures the media on the provided input devices.

Third-Party Uplink Streamer: The Third-Party Uplink Streamer is a part of the application and the application uses MF1, MF2 and MUC2 APIs to exercise selected 5GMUS functions to capture and stream the media content. This type of Streamer is typically a downloaded component.

The following subfunctions are identified as more detailed breakdown of the UE 5G Media Uplink Streaming Functions:

- Operator App. or Third-party App.: application which is out of scope of the present specification and uses the UE 5G Media Uplink Streaming Functions and APIs.

- Media Capturing(s): Devices such as video camera or microphone that transform an analog media signal into digital media data.

- Media Encoder(s): Compress the media data.

- Media Upstream Client: encapsulates encoded media data and pushes it upstream.

Note: this maps to the FLUS media function in the FLUS Source specified in TS 26.238 Uplink Streaming stage 3.

- Assistance Handler: requests and/or subscribes to receive assistance information from one or more Media AF in support of the streaming process.

Note: this is expected to map to the FLUS Assistance function specified in TS 26.238 Uplink Streaming stage 3.

- Media Remote Control Handler: receives control commands from a Media AF.

Note: this is expected to map to the FLUS Remote control function specified in TS 26.238 Uplink Streaming stage 3.

- Media Configuration: configures the Media AS for uplink streaming reception.

Note: this maps to the FLUS control function in the FLUS source specified in TS 26.238 Uplink Streaming stage 3.

Here are the role of the different APIs of the UE 5G Media Uplink Streaming Functions:

MUC1: Control Plane API used to configure, activate and stop the Media Capturing(s), Media Encoding(s) and Media Upstream Client functions.

MUC2: Control Plane API used to control Media Configuration, Assistance Handler, Media Remote Control Handler functions.

MF1: User Plane API between Media Capturing(s) and Media Encoder(s) functions that carries Media data to be encoded. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Encoder(s) function.

MF2: User Plane API between Media Encoder(s) and Media Upstream Client functions to carry encoded Media data. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Upstream Client function.

# 6	Procedures for Uplink Streaming

## 6.1	General

## 6.2	Establish an Uplink Streaming Session

The procedure allows an 5G Media Uplink Streamer to establish a uplink streaming session with a Media AS.

Figure x.2-1: Uplink Streaming Session Establishment

Steps

1: 	During provisioning, the 5G Media Uplink Streamer, the Media AS is provisioned with basic information, such as the Media AF and Media AS addresses. For some devices, only the remote control information is provisioned and all additional parameters are retrieved from Remote Controller.

2: 	The App starts the uplink media streaming.

3: 	The App instructs the 5GMUS to start providing a Media Streaming Entry.

When remote control is provisioning in step 1:

4: 	The transport session is established for the remote control

5: 	The 5GMUS starts waiting for incoming remote control commands

6: 	The 5GMUS receives a remote control command to start upstreaming (either now or with a timestamp). The Remote control command may contain details around the Media AS, etc.

7: 	The 5GMUS establishes the transport session

8: 	The 5GMUS establishes the media streaming session

When client assistance is provisioned:

9: The 5GMUS establishes the assistance channel to the provisioned Media AF(s)

When server assistance is desired (e..g for QoS or charging)

13: The Media AS establishes a assistance session with the Media AF.

## 6.3	Teardown of an Uplink Streaming Session

The procedure defines the termination of an uplink streaming session.

Figure x.2-1: Uplink Streaming Session Teardown

Steps

1: 	An Uplink streaming session is active and should be terminated

When the App terminates the session, e.g. triggered by user input

2: 	The App sends a Stop commands to the 5G Media Uplink Streamer

When remote control is used and the remote control session is established

3:	The 5G Media Uplink Streamer receives a stop command.

4: The 5G Media Uplink Streamer stops the capturing process

5: The uplink media streaming process is terminated

6: The transport session is released

When client assistance was established

7: The Media AF is notified that the media uplink streaming session is terminated.

When server assistance was established

8: The Media AS is notified the Media AF that the media uplink streaming session is terminated.

## 6.4	Providing Network Assistance

The procedure defines the network assistance from a Media AF.

Figure x.2-1: Providing Network Assistance

Steps

1: 	An Uplink streaming session is active. The Assistence Session is established with the Media AF.

2: 	The Media AF determines the need to send an assistance message.

3:	The assistance message is sent.

4: The 5G Media Uplink Streamer takes an appropriate action to the message.

## 6.4	Providing Remote Control

The procedure defines the remote control from a Media AF. Note that the Media AF for remote control may be deployed as a network based function or a UE based function.

Figure x.2-1: Uplink Streaming Session Establishment

Steps

1: 	An Uplink streaming session is active. The Remote Control Session is established with the Media AF.

2: 	The Media AF for Remote Control determines the need to send a remote control message.

3:	The remote control message is sent.

4: The 5G Media Uplink Streamer takes the defined action to the message.