Source: 	Nokia Corporation

Title: 	360-degree Conferencing Use Case and Requirements

Document for:	Agreement

Agenda item: 	11.9

# Introduction

The document describes the use case of a 360-degree conference call.

# Use Case

A group of colleagues are having a meeting in conference room A (see Figure 1). The room consists of a conference table (for physically present participants), a 360-degree camera, and a view screen. Two of their colleagues, Bonnie (B) and Clyde (C) are travelling and join the meeting through a conference call.

- Participants in conference room A use the screen to display a shared presentation and/or video streams coming from Bonnie and Clyde.

- Bonnie joins the conference from her home using a Head Mounted Display (HMD) and a camera that captures her video. She has a 360-degree view of the conference room.

- Clyde joins the conference from the airport using his mobile phone. He also has a 360-degree view of the conference room on his mobile screen and uses his mobile camera for capturing his own video.

Both Bonnie and Clyde can see the screen in the conference room as part of the 360-degree video. They also have the option to bring into focus any of the incoming video streams (presentation or the other remote participant’s camera feed) using their own display devices. The manner in which this focused stream is displayed is a function of their display device and is not covered in this use case.

Within the 3GPP MTSI TS 26.114 and Telepresence TS 26.223 specifications, the above use case can be realized in two possible configurations, which are explained below. The participants are referred to as A, B and C from here onwards.

In the first scenario, shown in Figure 1, the call is set up without the support of any media-aware network elements. Both remote participants, B and C, send information about their viewport orientation to A, which in turn sends them a viewport-dependent video stream from the omnidirectional camera.

Figure 1 360-degree conference call

In the second scenario, the call is setup using a network function, which may be performed by either a Media Resource Function (MRF) [1] or a Media Control Unit (MCU) [2]. In this case, the MRF/MCU receives a viewport-independent stream from A. Both B and C, send viewport orientation information to the MRF/MCU and receive viewport-dependent streams from it. Figure 2 illustrates the scenario. The A/V channel for conversational non-immersive content also flows through the MRF/MCU in the figure. However, it should be possible to maintain this channel directly between the conference call participants.

Figure 2 A 360-degree conference call via MRF/MCU

# Requirements

The following requirements must be met to support the above-mentioned use case:

- Capability for the party that sends 360-degree video to send viewport-dependent and/or viewport-independent streams.

- Timely delivery of the changes in viewport orientation from the remote participants, and appropriate low-delay actions to update the viewport-dependent streams. Any changes in viewport orientation should not lead to latency-prone signalling, such as SIP renegotiations.

- A suitable coordinate system to be used as the standard way of communicating the orientation of the viewport.

# Proposal

We propose that the above-mentioned use case and requirements are moved to Sections 2 and 3 of the Permanent Document [3].

# References

[1] 3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[2] 3GPP TS 26.223: "Telepresence using the IP Multimedia Subsystem (IMS); Media Handling and Interaction".

[3] ITT4RT Permanent Document – Requirements, Working Assumptions and Potential Solutions, Intel, Huawei Technologies Co., Ltd. (ITT4RT Co-Rapporteurs), 3GPP TSG SA WG4 #102 meeting, Bruges, Belgium, Tdoc S4-190241.

Contact: Igor D.D. Curcio, Saba Ahsan, Nokia Technologies, Email: igor.curcio, saba.ahsan@nokia.com

The system supports transmission of full 360 video. However, the use cases may restrict the field of view to enhance user experience.