Source: 	Fraunhofer IIS

Title: 			Proposed Definition of Binaural Audio

Document for: 	Discussion and Agreement

Agenda Item: 	7.5 – IVAS_Codec

1. Introduction

This contribution addresses the open definition of the term “Binaural audio”, as used in IVAS-4 [1]. From the discussion so far occurring it became obvious that the most important aspect of this definition is to avoid an additional spatialization step at the decoding/rendering output of such material. Any additional specification, e.g. wrt exact recording setup or specification of HRTF/BRIR convolution for testing of the IVAS codec is proposed to be carried out in the context of the performance requirements and/or processing functions.

2. Proposed definition

The following definition of “Binaural audio” is proposed:

“Binaural audio: two channel stereo audio (2.0) carrying a spatial image for direct presentation to the left and right ear over a pair of headphones. No additional spatialization (e.g. by an additional HRTF/BRIR convolution) shall be carried out by the codec/renderer.”

It is further on proposed to create a new permanent document “IVAS Terms and Definitions (IVAS-10)”, which includes all terms and definitions for the IVAS work-item.

3. Proposed changes to IVAS-4

The following changes are proposed to the IVAS Codec Design Constraints

In section 3, apply the following change:

*** Start change 1 ***

| Encoder Input Formats | The encoder shall support the following input formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Binaural audio - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - [Spatial audio, [N] channels and spatial metadata defined by [TBD].] [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| --- | --- |

*** End change 1 ***

Further on, add the following sections “X. Definitions” and “Y. References” into a new permanent document “IVAS Terms and Definitions (IVAS-10)”:

*** Start change 2 ***

X. Definitions

For the purposes of IVAS work item, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Binaural audio: two channel stereo audio (2.0) carrying a spatial image for direct presentation to the left and right ear over a pair of headphones. No additional spatialization (e.g. by an additional HRTF/BRIR convolution) shall be carried out by the codec/renderer.

Y. References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

*** End change 2 ***

4. Conclusion

It is proposed to agree on the inclusion of the changes outlined above into IVAS-4.

5. Reference

[1] 3GPP S4-190248 – IVAS Design Constraints (IVAS-4), Version 0.0.7.