3GPP TSG-SA4 Meeting #104	S4-190617

Cork, Ireland, 1-5 July 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.444 | CR | Draft | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EVS Non Bit Exact Float conformance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_FCNBE |  |  |  |  |  | Date: |  |  | 2019-07-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current conformance process in 26.444 relies and bit-exact testing that is not applicable to EVS float standard in 26.443. To be able to useTS 26.443 a non Bit-exact conformance process should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Non bit-exact conformance process description and associated tools and test sequences. The tools and new test sequences are part of the zip containing the test vectors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EVS Float standard 26.443 cannot be widely used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

# 1	Scope

The present document specifies the digital test sequences for the Enhanced Voice Services (EVS) Codec. These sequences test implementation of the EVS Codec (3GPP TS 26.445), Voice Activity Detection (VAD) (3GPP TS.26.451), Comfort Noise Generation (3GPP TS 26.449), Discontinuous Transmission (DTX) (3GPP TS 26.450), Error Concealment of Lost Packets (3GPP TS 26.447), Jitter Buffer Management (JBM) (3GPP TS 26.448), and AMR-WB Interoperable Function (3GPP TS 26.446). In addition, the present document specifies conformance testing.

End of First Change

Second Change

# 7	Conformance Testing

## 7.1 Bit-exact Conformance

In the scripts mentioned in clause 6, bit-exactness between the output sequences and test sequences are performed. If all the test sequences (decoded & bit-streams) are bit exact, then the implementation is considered conformant for 3GPP services. This applies for all implementations based on TS 26.442, TS 26.443 or TS 26.452.

## 7.2 Non-Bit-exact Conformance

### 7.2.1 Overview

In case an implementation based on Floating–point code (26.443) is used, the outputs may not be bit-exact and in this case non bit-exact conformance should be used. This conformance process is defined here.

Using the code in 26.443 a floating-point implementation is obtained by using a compiler with specific platform options and optimizations. This implementation is then tested for conformance using three specific tests:

-	Decoder test comparing the implementation decoder with TS 26.443 decoder.

-	Encoder test comparing the implementation encoder with TS 26.443 encoder.

-	MOS-LQO verification comparing the implementation with TS 26.442 implementation.

All three tests should pass for the implementation to be declared conformant. The tests are described in more details in Annex X.

Figure x1 shows the flow chart of the non bit-exact conformance process.

Figure x1 : Non bit-exact conformance process

### 7.2.2 Running the Tests

All test scripts and tools necessary to run the decoder and encoder test are attached to this zip file. The MOS-LQO test requires both an additional tool and database, as explained in Annex X.3. The reference files for SNR and MLD encoder and decoder tests are also included in this zip file.

The executable and scripts to run the tests are included in T.S 26.444 zip containing the test sequences. Annex X provides more details on the tests. In the case of discrepancy between the procedure described in Annex X and the scripts provided in the zip, the scripts provided prevail.

- Readme_dec_snr.sh batch file describes how to run the decoder test

- Readme_enc_sh batch file describes how to run the encoder test

- Readme_MOSLQO_sh batch describes how to run the MOS LQO test

The implementation will be declared conformant if all tree tests run successfully.

End of Second Change

Third Change

# Annex X: Tools Description

## X.1 Decoder Test

### X.1.1	General Considerations

The reference PCM signals are taken from the decoded floating point test sequences of TS 26.444. The PCM signal under test are obtained by running the floating point bit-stream included in TS 26.444 through the Decoder under Test (Figure x2). The reference decoder is the floating-point code of TS 26.443.

Figure x2: Flow diagram for the decoder test using signal based metrics

All metrics are calculated on the reference PCM signal xREF(t) and the PCM signal under test xTST(t) based on 20ms frames. The frames of the two signals will be time aligned, this means the delay compensation in EVS encoder and decoder remains ON (the default configuration). Furthermore the frame processing is aligned with the encoded frame by adding the decoder delay. Table x1 shows the delay values used for the different sampling frequencies.

Table x1: Delay used for alignment of processing frames with encoded frames

| Sampling frequency | 8000 Hz | 16000 Hz | 32000 Hz | 48000 Hz |
| --- | --- | --- | --- | --- |
| Delay (samples) | 10 | 37 | 74 | 111 |

The number of samples N for a 20ms frame size is defined by N=fs∙0.02, where fs represents the sampling rate.

The PCM signals xREF(t) and xTST(t) should be scaled between -1 and 1.

### X.1.2 Metrics

#### X.1.2.1 RMS Error Threshold

The RMS method is derived from the decoder conformance used in ISO/IEC 14496-26 [10]. The RMS error is calculated for each 20ms frame and compared to a threshold according to:

20∙log10(xREF-xTST)2N< TRMS RMS=20∙log10(xREF-xTST)2N< TRMS

The value chosen for the RMS error threshold is to assume change on the last bit of the audio signal:

TRMS=20∙log102-K-112 with K=15K=1

#### X.1.2.2 Signal to Noise Ratio (SNR)

The segmental SNR method is derived from the decoder conformance used in ISO/IEC 14496-26 [10]. For each 20 ms segment, the following values need to be calculated:

Energy of reference signal: EREF=xREF2

Noise energy: EN=(xREF-xTST)2

Signal to noise ratio snr=10log10EREF+EPSEN+EPS with EPS=10-5

As EVS is a switched codec containing a LPC based speech coder and a MDCT based transform coder, the SNR values vary significantly depending on the used coding mode. Therefore, a constant threshold for the SNR is not suitable but instead, a reference value per frame and test vector should be specified. The SNR should be compared against the thresholds by

WSNRf,v<(TSNRf,v-SNRHEADROOM)∙ PREF(f,v)   snrf,v ≥(TSNRf,v-SNRHEADROOM)  where f is a 20 ms frame index and v is the test vector index

The set of SNR reference values is included in the zip file.

#### X.1.2.3 Spectral Distortion

The spectral distortion method can be conducted on a 20 ms frame base by the following steps:

Calculate the absolute FFT spectrum of xREF and xTST using a Hanning window

XREFk=327681000n=0N-1whannn∙xREFn∙e-j2πnkN     for k=0..N-1

XTSTk=327681000n=0N-1whannn∙xTSTn∙e-j2πnkN     for k=0..N-1

with whann(n)=12-12cos2πN+1∙(n-1)   for n=0..N-1

The 32768 is due to MATLAB scaling and to align to 16 bit PCM C-code. This scaling is dependent on the input value range.

For all spectral bins the distortion d is calculated according to the following pseudo code:

cnt=0
d=0
for k=1..N/2-1
    if (XREFk==0 && XTSTk==0)
        X_Y = 1;
        Y_X = 1;
    else
        if (XREFk==0)
            X_Y = 0;
            Y_X = 2;
        else if (XTSTk==0)
            X_Y = 2;
            Y_X = 0;
        else
            X_Y = (XREFk * XREFk) / (XTSTk * XTSTk);
            Y_X = (XTSTk * XTSTk) / (XREFk * XREFk);
        end
    end
    COSH = (X_Y + Y_X - 2)/2;   
    d = d + COSH;
    cnt = cnt+1;
end
d = d/cnt;

The distortion value d is to be compared against a threshold. The frame will be considered as passed if d<TSD  and snr≥maxSNR

with

maxSNR=CDMAXSNR,  if CDMAXSNR>Tsnr-SNRHEADROOM-CDSNRHEADROOMTsnr-SNRHEADROOM-CDSNRHEADROOM,  else

### X.1.3 Analysis Flow and Reporting

The three metrics are computed in a specific order, as shown in Figure x2. Once a frame passes a metric, the process is stopped and the next frame is analysed. The SNR metric is computed on the frames failing the RMS error criteria. Similarly the Spectral Distortion metric is computed on the frames failing the SNR criteria.

Figure x3: Flow chart for decoder tool

In a file one or two frames could slightly be above the threshold. To avoid relaxing the threshold, a constraint on the number of frames failing per file has been added as an additional criteria.

if number_of_frames_failing =< THRESH_GOOD_FRAMES_TO_PASS * number_of_frame_in file, the test signal will be considered equivalent to the reference signal.

All the test sequences need to pass for the implementation to be conformant.

In addition to the number of fail/pass test sequences, the statistics from the three methods should be displayed. Table x2 shows an example of reporting.

Table x2: Template for result presentation

|  | RMS | WSNR | Spectral Distortion |
| --- | --- | --- | --- |
| Number of frames tested |  |  |  |
| Number of frames passing |  |  |  |
| Number of frames failing |  |  |  |
| Ratio of frames passing |  |  |  |
| Ratio of frames failing |  |  |  |

As part of conformance criteria, thresholds are set for the ratio of frames passing with RMS and WNR tests (Ratio_RMSframespassing_and RatioWSNRframespassing respectively).

The list of the thresholds used in decoder test are summarized in table x3 with example values.

Table x3: List of thresholds

| Thresholds | Description | Example value |
| --- | --- | --- |
| SNRHEADROOM | Headroom compare to the Tsnr threshold | 3 dB |
| CDSNRMAX | Limit of SNR for the spectral distortion test | 0 dB |
| CDSNRHEADROOM | Headroom compare to Tsnr threshold for the spectral distortion test | 10 dB |
| Tsd | Threshold for the spectral distance | 6.6 |
| THRESH_GOOD_FRAMES_TO_PASS | Factor for number of failing frame per file | 0.005 |
| Ratio_RMSframespassing | Minimal percentage for frames passing RMS error test | 47% |
| RatioWSNRframespassing | Minimal percentage for frames passing WSNR test | 95% |

## X.2 Encoder Test

### X.2.1 General Consideration

The MLD metrics is used to test the floating-point encoder implementation. Figure x4 shows the flow diagram of the proposed encoder conformance test:

Figure x4: Flow diagram for the encoder test using MLD Loudness Difference metric

All encoder test sequences from TS 26.444 will be encoded using the float encoder implementation under test. The bit-stream obtained will be then decoded using the 3GPP reference float decoder from TS 26.443 to obtain the test signals. The test signals will then be compared with the reference signal from TS 26.444. Since the loudness tool in the presented form operates on 48 kHz sample rate only, additional resampling is applied before processing.

### X.2.2 Metrics

The Loudness Difference (LD) is used as to assess the encoder implementation. The procedure is adopted from the loudness calculation of PEAQ [11] using the Filter bank-based ear model. The following steps need to be processed:

-	Filterbank (Annex 2 section 2.2.5 of [11]):

-	subsample factor F changed to 16 for higher time resolution.

-	Outer and Middle Ear Filtering (Annex 2 section 2.2.6 of [11])

-	Frequency Domain Smearing (Annex 2 section 2.2.7 of [11])

-	Rectification (Annex 2 section 2.2.8 of [11])

-	Time Domain Smearing 1 - Backward Masking (Annex 2 section 2.2.9 of [11])

-	Adding of Internal Noise (Annex 2 section 2.2.10 of [11])

-	Time Domain Smearing 2 - Forward Masking (Annex 2 section 2.2.11 of [11])

-	Loudness (Annex 2 section 3.3 of [11]):

-	This section defines the specific loudness patterns Nk,n for k subbands and n time samples

-	The specific loudness patterns are calculated for:

-	reference signal Nref[k,n]

-	signal under test Ntest[k,n]

-	Loudness Difference (LD):

-	The loudness difference Ndiff[n] is calculated as follows:

-	Ndiffn= k=140Nrefk,n-Ntestk,n

The LD is computed with a granularity of 2ms. To get a MLD value every 20ms, 10 segments are combined using the maximum value of those 10 segments.

Note:	In the context of this test, the "loudness difference" is calculated as difference in some values (subtraction, not division).

The LD should be below a threshold ThMLD based on a reference value LDref plus some headroom.

[

The headroom could be defined as fixed value, currently set to 1.

ThMLDn=1+LDrefn

]

Then for each file a MLD could be defined as

MLD=minThMLDn-Ndiffn

The test file will be considered equivalent to the reference file if the MLD is positive or equal to 0, i.e. the Loudness Difference doesn’t exceed the threshold for all the frames.

All the test sequences need to pass for the implementation to be conformant.

LDref has been obtained using these [4] implementations:

[

- clang-6.0 x86_64 -O2

- aarch64-linux-gnu-gcc-8 -march=armv8-a -O2

- gcc-7 i686 -O3

- MS Visual Studio 2017 /O2 (reference executable included in the TS 26.443 ZIP file)

]

For each frame the maximum LD value of the [4] implementations then defines a corridor, the ‘refline’. This then leads to a profile for each EVS test vector, which contains on a 20ms frame basis an allowed LDref value relative to the reference. Allowed differences in implementations under test (IuTs) are thus limited to the tolerable differences by the different compilers used for the refline generation.

## X.3 MOS-LQO Test

### X.3.1 General consideration

For this test, POLQA™ software should be licensed, and the audio database ITU.P.501 Annex B & C should be downloaded. POLQA version 2.4 is currently used for the test.

This test was used in EVS characterization reported in TS 26.952

For this test, four combinations of encoder/decoder are used (3GPP EVS encoder/decoder executables are taken from TS 26.442):

a)	3GPP fixed-point encoder and 3GPP fixed-point decoder (FX/FX),

b)	floating-point Encoder under Test and floating-point Decoder under Test (FL/FL),

c)	3GPP fixed-point encoder and floating-point Decoder under Test (FX/FL),

d)	floating-point Encoder under Test and 3GPP fixed-point decoder (FL/FX).

The MOS-LQO scores are computed for each of the four cases using the decoded files and the original test files.

The test files are based on ITU-T P.501 Annex B & C to be compliant with POLQA tool. 30 files representing various talkers and languages are used for each test conditions, and the average MOS-LQO scores are reported.

The scenario a) is considered the reference score. For the three other scenarios (b, c and d), the difference in MOS-LQO of a) are then computed:

-	a) - b)

-	a) - c)

-	a) - d)

The difference a) - b) assesses the encoder + decoder floating-point implementation, the difference a) - c) assesses the decoder implementation and a) - d) assesses the encoder implementation.

Figure x5 represents the flow diagram to obtain the MOS-LQO in the three scenario.

Figure x5: Flow diagram to obtain the MOS-LQO in the three scenario

### X.3.2 Test Files

The test files are based on ITU-T P.501 Annex B & C. 30 files representing various talkers and languages. The files are listed below:

an1f1s1 => AnnexC//P501_C_chinese_f1_FB_48k.wav

an1f1s2 => AnnexC//P501_C_chinese_f2_FB_48k.wav

an1f1s3 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//female 1.wav

an1f1s4 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//female 2.wav

an1f1s5 => AnnexC//P501_C_english_f1_FB_48k.wav

an1f2s1 => AnnexC//P501_C_english_f2_FB_48k.wav

an1f2s2 => AnnexC//P501_C_finnish_f1_FB_48k.wav

an1f2s3 => AnnexC//P501_C_finnish_f2_FB_48k.wav

an1f2s4 => AnnexC//P501_C_french_f1_FB_48k.wav

an1f2s5 => AnnexC//P501_C_french_f2_FB_48k.wav

an1f3s1 => AnnexC//P501_C_german_f1_FB_48k.wav

an1f3s2 => AnnexC//P501_C_german_f2_FB_48k.wav

an1f3s3 => AnnexC//P501_C_italian_f1_FB_48k.wav

an1f3s4 => AnnexC//P501_C_italian_f2_FB_48k.wav

an1f3s5 => AnnexC//P501_C_japanese_f1_FB_48k.wav

an1m1s1 => AnnexC//P501_C_chinese_m1_FB_48k.wav

an1m1s2 => AnnexC//P501_C_chinese_m2_FB_48k.wav

an1m1s3 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//male 1.wav

an1m1s4 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//male 2.wav

an1m1s5 => AnnexC//P501_C_english_m1_FB_48k.wav

an1m2s1 => AnnexC//P501_C_english_m2_FB_48k.wav

an1m2s2 => AnnexC//P501_C_finnish_m1_FB_48k.wav

an1m2s3 => AnnexC//P501_C_finnish_m2_FB_48k.wav

an1m2s4 => AnnexC//P501_C_french_m1_FB_48k.wav

an1m2s5 => AnnexC//P501_C_french_m2_FB_48k.wav

an1m3s1 => AnnexC//P501_C_german_m1_FB_48k.wav

an1m3s2 => AnnexC//P501_C_german_m2_FB_48k.wav

an1m3s3 => AnnexC//P501_C_italian_m1_FB_48k.wav

an1m3s4 => AnnexC//P501_C_italian_m2_FB_48k.wav

an1m3s5 => AnnexC//P501_C_japanese_m1_FB_48k.wav

### x.3.3 Test Cases

The differences are computed for various test conditions:

-	All the codec modes of EVS

-	All the bandwidths of EVS

-	All the bit-rates of EVS, including bit-rate switching

-	DTX ON and OFF

-	Various levels: -26 dB, -36 dB, -16 dB

-	Various noise conditions

-	Various impairment conditions

The files have been processed according to EVS-7c (EVS processing plan) for the various test conditions [6].

In all, 941 test conditions are assessed, representing 225,600 second of speech, or a little bit more than 62 hours.

### X.3.4 Thresholds and Criteria

From the MOS-LQO differences of the test condition, the average, 95%, 99% and Maximum are computed for all bandwidths combined, as well as for each set of bandwidth condition. The number of test condition for each bandwidth and the total are summarized in Table x6.

Table x6: Number of test conditions per bandwidth

| Bandwidth | NB | WB | WBIO | SWB | FB | All |
| --- | --- | --- | --- | --- | --- | --- |
| Number | 136 | 236 | 216 | 192 | 161 | 941 |

An implementation will be considered passing the MOS-LQO verification if all the average, 95 percentile, 99 percentile and maximum MOS-LQO differences are below the thresholds proposed in Table x7 for all conditions.

Table x7: Thresholds for MOS_LQO difference

| All | Average | 95% | 99% | Max |
| --- | --- | --- | --- | --- |
| A-B | 0.002 | 0.04 | 0.07 | 0.09 |
| A-C | 0.002 | 0.02 | 0.04 | 0.07 |
| A-D | 0.002 | 0.04 | 0.08 | 0.09 |
| NB | Average | 95% | 99% | Max |
| A-B | 0.009 | 0.07 | 0.08 | 0.09 |
| A-C | 0.002 | 0.02 | 0.04 | 0.06 |
| A-D | 0.011 | 0.07 | 0.09 | 0.09 |
| WB | Average | 95% | 99% | Max |
| A-B | 0.002 | 0.04 | 0.07 | 0.08 |
| A-C | 0.002 | 0.02 | 0.04 | 0.06 |
| A-D | 0.002 | 0.04 | 0.07 | 0.09 |
| WBIO | Average | 95% | 99% | Max |
| A-B | 0.002 | 0.02 | 0.06 | 0.08 |
| A-C | 0.002 | 0.02 | 0.03 | 0.07 |
| A-D | 0.002 | 0.02 | 0.03 | 0.08 |
| SWB | Average | 95% | 99% | Max |
| A-B | 0.002 | 0.04 | 0.06 | 0.09 |
| A-C | 0.003 | 0.03 | 0.04 | 0.04 |
| A-D | 0.002 | 0.04 | 0.07 | 0.08 |
| FB | Average | 95% | 99% | Max |
| A-B | 0.006 | 0.04 | 0.07 | 0.08 |
| A-C | 0.005 | 0.04 | 0.05 | 0.06 |
| A-D | 0.005 | 0.04 | 0.06 | 0.08 |

End of document