3GPP TSG-SA4 Meeting #104	S4-190679

01 July – 05 July 2019, Cork, Ireland

Agenda item: 	10.7

Source: 	Fraunhofer HHI

Title: 	Content generation and HTML5 MSE playback tools for TS 26.118 Advanced Video Media profile

Document for	Information

# Introduction

One of the aspects included in the objective for VR Streaming Interoperability and Guidelines in [S4-190562] is to collect content generation tools for the usage scenarios defined in TS 26.118 [1]. The Advanced Video Media (AVM) profile as defined in TS 26.118, supports viewport dependent transmission and rendering with the user viewport at a higher resolution than that achievable with a 4K ERP for VR360 content. With this contribution, Fraunhofer HHI presents content generation tool which conforms to the AVM profile and its usage guidelines. A JavaScript application supporting HTML5 MSE playback for VR360 content is also announced.

The AVM profile enables usage of content generated according to the Flexible H.265/HEVC operation point where multiple representations can be downloaded and played back together. The multiple streams are combined together into a single bitstream. Thus, a single video bitstream is decoded on the client’s end device. This technique allows to increase the resolution within the viewport. Projection formats like Equirectangular Projection (ERP) and Cubemap Projection (CMP) are supported as well. ISO/IEC 23090-2 Omnidirectional Media Format (OMAF) defines HEVC-based viewport-dependent media profile (Annex D.6.4 in [2]) which allows to stream HEVC tiles of different resolutions. The HEVC-based viewport-dependent media profile aligns with the AVM profile as defined in TS 26.118.

The compliant tools for content generation as well as the HTML5 Media Source Extension (MSE) playback are publicly available on GitHub (https://github.com/fraunhoferhhi/omaf.js) for evaluation, research and standardisation purposes. A successful demonstration of the tools was carried out at the Second VR ecosystems and standard workshop in Sony Studios, Culver City, CA, USA.

# Fraunhofer HHI TS 26.118 Advanced Video Media profile implementation

The software available in the repository implements the tools supporting the AVM profile in addition to OMAF. The tools can be used for generation of live and on-demand streaming of VR360 content. However, the capability for ‘live streaming’ is limited to the format generation part (a static MPD and corresponding DASH formatted segments), while the full live functionality, i.e. live encoder feed directly connected to the generator tool, is expected to be released in the future updates. The Fraunhofer HHI AVM implementation has two functionalities which are as follows:

2.1. File Creation for AVM profile with HEVC encoded Motion-Constraint Tile Sets

The scheme as described in the clause 5.2.4. of TS 26.118 is implemented in ‘avm_mode’ branch in the GitHub repository:

https://github.com/fraunhoferhhi/omaf.js/tree/avm_mode

The supported platforms are macOS and Linux. The content generation process for AVM profile is carried out in the following five steps:

- The ERP raw file (YUV) is converted to a high-resolution CMP raw file (YUV). ‘TApp360Convert’ tool is used for projection format conversion of 360 videos [3].

- A low-resolution CMP raw file is created by downscaling the high-resolution CMP raw file.

- Both high-resolution and low-resolution CMP raw videos are split into twenty-four tiles. The size of a high-resolution tile is 768x768. The size of a low-resolution tile is 384x384. The task of splitting the YUVs is done using FFmpeg.

- Each tile is encoded as a Motion-Constraint Tile Set (MCTS) using an HEVC encoder. The HEVC encoders supported by our implementation are as follows:

  - HM reference encoder. ‘TAppEncoder’ HEVC encoder version 16.19 is available as a binary in the repository.

  - Kvazaar [4]

- The encoded HEVC bitstreams are packaged into AVM compliant files. In the AVM profile [1], the sample entry type 'hvc2' is allowed which makes it possible for a client to use extractors. Each extractor track carries a preselection property descriptor. The preselection property descriptor holds a set of track ID values for the dependent ‘hvc1’ tracks. A total of twenty-four ‘hvc2’ tracks are generated. The preselection descriptor in each ‘hvc2’ track holds trackIDs for twelve high-resolution tiles and twelve low-resolution tiles. The duration of each media segment is equal to frame duration of nine frames.

Binaries for AVM compliant content are available in the repository. An example usage of the content generation tool is shown below where the Kvazaar encoder is used to encode 100 frames of ‘raw_video.yuv’ with QP 32 using a maximum of 12 threads with ‘AVM’ mode enabled:

./create_omaf_files.py -s 1-5 -i raw_video.yuv -f 100 -fr 30 -q 32 -t 12 --mode avm  --codec 1 -o outputDir

The flexibility in the file generation tool may be expected in the future updates. More information on the usage of the tool can be found on the GitHub page as mentioned above.

2.2. JavaScript player for AVM profile

The JavaScript player is able to play the content generated using AVM scheme with projection formats such as:

- Equirectangular Projection  
Note: The content with ERP projection type can be prepared using the tools   provided by Nokia. [S4-190437]

- Cubemap Projection

The requirements for the JavaScript player are:

- Support for MSE

- Support for hardware-based HEVC decoding

- WebGL API for rendering

Downloader

Media Engine

Renderer

Orientation

MPD Parser

Player

Scheduler

DASH content

MSE 
Source buffer

Media / Meta data
Control data

The JavaScript player implementation for AVM profile is visualised in Figure 1.

Figure 1. Block diagram for the JavaScript player

The ‘Player’ module is the core of the entire application. It connects all the other modules with each other and controls them. ‘Downloader’ module generates HTTP requests to the server. ‘MPD Parser’ parses the DASH manifest file (MPD) as well as parses the dependencies for each extractor track. According to the user’s viewing orientations, an extractor track is selected. The player makes a request to download the extractor track as well as the dependent tracks. ‘Scheduler’ module controls the ‘Downloader’ module and decides when a request for the next segment should be made based on the current buffer status of the ‘Player’. The ‘Media Engine’ module receives the downloaded media segment data from the ‘Downloader’ module as an input and parses the AVM-related metadata on the file format level. However, the support for VRStream media segments is not yet present in the HTML5 MSE. Therefore, the downloaded media segment data are repackaged into an MSE-compliant format. The repackaging logic uses GPAC’s MP4Box tool ‘mp4box.js’ which is further extended to support VR-Stream specific boxes. Figure 2 shows the repackaging process and the flow of the downloaded media segments through the ‘Media Engine’ to the MSE SourceBuffer.

Figure 2. Repackaging by Media Engine

Each segment consists of multiple ‘hvc1’ tracks (one track for each HEVC encoded tile) and one ‘hvc2’ track with HEVC extractor NAL units (as defined in ISO/IEC 14496-15). The extractor track is required to generate a single HEVC bitstream in order to use a single decoder instance. An extractor track holds in-stream data structure using a NAL unit header which is useful for extracting data from other tracks. ‘Media Engine’ resolves the logic of extractors and packages an MSE complaint file with a unique track ID. Therefore, irrespective of change in the extractor track ID, the MSE SourceBuffer receives segments having the same unique track ID. Thus, to the MSE SourceBuffer it always looks like the segments are coming from the same video source. The ‘Renderer’ module uses the region-wise packing metadata to construct a video texture from the decoded frame. The video texture is rendered on a canvas through the WebGL API and three.js library.

The JavaScript player implementation was tested on Safari browser with support of HEVC video playback through HTML5 MSE API on a 2016 MacBook Pro. A screenshot of the JavaScript player running in Safari browser is shown in Figure 3.

The JavaScript player provides additional features such as:

- ability to change maximum buffer limit

- ability to disable or enable viewport track switching

- logging information

- rendering information like FPS and rendered projection

- Yaw and Pitch values

- Extractor track information

Currently, the JavaScript player supports video media playback for a Live media streaming service, i.e. only static MPDs are supported. The support for Video on Demand service may be expected in the future releases.

Figure 3. Screenshot of the JavaScript player

# Test vectors

The test vector for the following sequence is available at: https://datacloud.hhi.fraunhofer.de/nextcloud/s/2CrbSpdLFpZNDed

Sequence name   : Trolley360_8192x4096_30fps_300frames_8bits.yuv

Projection Type 	: Full 360-degree ERP raw video sequence

Resolution 			: 8192x4096

Frame rate 			: 30fps

Pixel Format		: 4:2:0 format

Bit depth 			: 8

Duration 			: 300 frames

# 4 Conclusion

In this contribution, Fraunhofer HHI announces the public availability of the tools for content generation and playback for 360-degree VR streaming conforming to the Advanced Video Media profile defined in TS 26.118 and provides a test vector for the Advanced Video Media profile aligned with the objectives of Draft SID on VR Streaming Usage and Implementation Guidelines (S4-190562). 3GPP members are welcome to use these tools for research and standardization purposes.

5 References:

[1] TS 26.118: Technical Specification Group Services and System Aspect; 3GPP Virtual reality profiles for streaming applications

[2] ISO/IEC 23090-2: "Coded representation of immersive media -- Part 2: Omnidirectional media format".

[3] 360-degree video conversion library [Online] Available at https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-5.0/ [Accessed 17 June 2019]

[4] Ultra Video Group. 2019. Kvazaar: An open-source HEVC encoder. [ONLINE] Available at: http://ultravideo.cs.tut.fi/. [Accessed 17 June 2019]