Source:	Nokia Corporation

Title:	On testing IVAS MASA with Reference

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

For IVAS codec testing, it remains important to follow the traditional principle of using an unencoded signal as a reference to which the encoding algorithm is compared. This will allow for evaluating the performance of the compression. However, for the IVAS codec it is being discussed the support of certain input audio formats that cannot be listened to directly. For example, scene-based audio such as Ambisonics and MASA (metadata-assisted spatial audio) fall into this category.

The current input document discusses how the IVAS codec can be tested with an unencoded reference when the MASA format input is being used.

# 2. Reference testing

It is discussed testing aspects relating to generation of reference for various signal inputs and their use in testing, e.g., in [1] and [2].

The basic principle for codec comparison against Reference (REF) and other Codec(s) under Test (CuT) is shown in Figure 1. The Direct signal provides to the listener an unencoded/uncompromised reference quality. Depending on the test type, the listener may be asked, e.g., to rate the perceived degradation of the CuT condition against said reference.

According to Figure 1, an audio capture may be directly used in its original format (e.g., mono, stereo) or alternatively transformed into a codec input (e.g., Ambisonics). There may thus be included a format generation step. For example, a mono audio is well understood without a specific format generation, while in the case of Ambisonics or MASA, the captured audio goes through a specific format generation for obtaining the codec input. The input audio signal is fed separately into each CuT encoder. For the “reference path”, no encoding is performed. The reference and the decoded audio by the CuT may need a rendering into an audio format that can be listened to. It is understood that the CuT could include a rendering, however an external rendering could be used.

Figure 1: General model of reference generation and IVAS evaluation.

# 3. Testing MASA with unencoded reference

Scene-based audio including Ambisonics and MASA can generally be evaluated according to a similar high-level setup. However, the formats as such are different, which means that the format generation and rendering algorithms will also be different. Figures 2 and 3 present suitable schemes for MASA evaluation. In Figure 2, an external rendering module is used, which can then be shared between the unencoded reference and the CuT conditions. This evaluation approach allows for a comparison of the compression algorithms, as the rendering step is common. However, it should be taken care that the common renderer is of sufficient quality to not hide effects caused by the CuT.

Figure 3 illustrates the evaluation based on the required IVAS renderer, i.e., the renderer included with the CuT. The intention of this test can be seen as to show the capability of the codec with optimized renderer compared to an uncompressed common reference. It should be noted that this test type should allow CuT to exceed the common reference condition in quality to encourage novelty in both codec and rendering.

Figure 2: Input/Reference generation and IVAS evaluation for MASA format using external renderer.

Figure 3: Input/Reference generation and IVAS evaluation for MASA format using IVAS renderer.

# 4. Summary

It was discussed in this contribution examples on how reference testing can be carried out for IVAS in case of MASA format input. In view of the source, testing with reference is important to evaluate the encoding performance of the candidate algorithms.

In general, it can be important to test the candidate solutions using both external and (internal) IVAS renderers in order to evaluate different aspects. While the external renderer test allows for comparing the quality of the compression algorithms, it does this only in terms of that particular rendering. When testing using the (internal) IVAS renderer, it should be possible for the CuT to score higher than the REF condition.

# References

[1]	S4-190370: “Discussion on input format testing”, Qualcomm Technologies, Inc.

[2]	AHEVS-470: “On the Importance of the Reference for IVAS Testing”, Fraunhofer IIS