3GPP TSG-SA WG#4 Meeting #104	S4-190762

# Cork, Republic of Ireland, 1-5 July 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | <CR#> | rev | <Rev#> | Current version: | 16.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FLUS Remote Control Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_FLUS |  |  |  |  |  | Date: |  |  | 25.6.2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The FLUS remote control procedures were missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The FLUS remote control procedures are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remote control procedures are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.3.2.10	Session establishment for Remote Control

When remote control is needed, the Remote Control Target in the FLUS Source is provisioned with the relevant information to establish one or more F-RC sessions. The Remote Control Target creates the F-RC sessions and then listens for incoming messages.

Figure 5.3.2.10-1: FLUS F-RC Session creation

1. The F-RC session is established triggered by the Remote Control Target.

2. On successful creation, the Remote Control Target receives a positive acknolwedge.

The Remote Control Target starts waiting for remote control related messages.

#### 5.3.2.11	FLUS Source capability discovery

This procedure allows discover capabilities of the FLUS Source.

Figure 5.3.2.11-1 shows the sequence diagram of the FLUS Source capability discovery message exchange using F-RC.

The set of FLUS Source capabilities is specified in clause 5.3.5.

Figure 5.3.2.11-1: Get current FLUS source capabilities

1. The Remote Controller functions sends the FLUS Source capability request the Remote Control Target of the FLUS Source.

2. The Remote Control Target provides a list of FLUS Source capabilities in response.

#### 5.3.2.12	Remote FLUS Source configuration creation

When the FLUS Source has established a F-RC session, the Remote Controller function can create FLUS Source Configurations. FLUS Source Configuration properties and in particular FLUS media instantiation selection is added in subsequent procedures.

Figure 5.3.2.12-1 shows the sequence diagram of the remote FLUS Source Configuration establishment message exchange.

Figure 5.3.2.12-1: Remote FLUS Source configuration creation

1. The FLUS Source Configuration is created. The Remote Controller function provides a valid access token.

2. On successful creation, the Remote Control Target acknowledges the creation. FLUS Source Configuration properties (such as the FLUS Session information to identify the FLUS Sink resources) are fetched and modified with subsequent transactions.

### 5.3.2.13	Get FLUS Source configuration properties

This procedure allows a Remote Controller function to obtain the current FLUS Source configuration.

Figure 5.3.2.13-1 shows the sequence diagram of the FLUS Source Configuration properties retrieval message exchange.

Figure 5.3.2.13-1: Get current FLUS Source Configuration properties

1. The Remote Controller function sends along with the session property request, the access token.

2. The Remote Control Target of the FLUS source provides the FLUS Source Configuration properties in response.

#### 5.3.2.14	Update FLUS Source configuration

The procedure allows a Remote Controller function to update the current FLUS Source configuration.

Figure 5.3.2.14-1 shows the sequence diagram of the Remote FLUS Source Configuration update message exchange.

Figure 5.3.2.14-1: Remote FLUS Source Configuration update

1. The Remote Controller function modifies any or all of the properties of the FLUS Source Configuration resource.

2. The FLUS source updates the resource.

#### 5.3.2.15	FLUS source status report request

This procedure allows a Remote Controller function to obtain the current status of the FLUS source. The current status is reported as a list of its capabilities with the current status of each one. Each parameter may be elaborated with more information, e.g. when on battery power the charge level in per-cent may be reported.

Figure 5.3.2.15-1 shows the sequence diagram of the FLUS source status report request message exchange.

Figure 5.3.2.15-1: Get current FLUS source status report

1. The Remote Controller function sends the FLUS source status request to the FLUS source.

2. The Remote Control Target of the FLUS source provides its current status as a list of its capabilities with the status of each one in response.

#### 5.3.2.16	Terminate a FLUS Source configuration

This procedure allows a Remote Controller function to terminate a FLUS session in a FLUS Source. All active media sessions and streams will be terminated automatically with the termination of the FLUS session.

Figure 5.3.2.16-1 shows the sequence diagram of the Remote FLUS session termination message exchange.

Figure 5.3.2.16-1: Remote FLUS session termination

1. The Remote Controller function sends the terminate FLUS session command. The access token is provided as input.

2. The Remote Control Target of the FLUS source terminates the FLUS session, including all active media streams. The Remote Control Target may discard the FLUS session configuration.

3. The Remote Control Target acknowledges the execution of the request.

*** Next Change ***

### 5.3.5	FLUS source characterisation, capabilities and configuration properties

FLUS source devices are characterized so that the basic FLUS-related functionality can be made known to the FLUS sink.

The following FLUS source properties are specified:

-	Available video format(s);

-	Available audio format(s);

-	Available ancillary stream format(s): subtitles / captions, content metadata – for that which is not embedded in the media stream(s);

-	Connectivity: RAN, wired – and which system/s;

-	Remote control - one of: manual, remote, none – and which system/s;

-	Mobility - one of: fixed, on foot, ground vehicle, airborne vehicle, surface water vehicle, underwater;

-	Power – one of: Battery, battery with autonomous charging, mains.

The Remote Controller function uses the FLUS source capability discovery message exchange as specified in sub-clause 5.3.2.11 to retrieve the capabilities of the FLUS source and its attached media capture device(s).

The Remote Controller function uses the FLUS source status report request message exchange as specified in sub-clause 5.3.2.15 to retrieve the current status of the FLUS source device. The status is returned in the form of the capabilities of the FLUS source populated with the applicable setting to indicate the current state of the corresponding parameter.

Some parameters can contain a particular capability setting, e.g. the current media format being transmitted. Other parameters contain one of the possible settings, possibly adding auxiliary information, e.g. when running on battery power, the current charge level and estimated operating time before needing to re-charge is indicated. When charging, the charge level and estimated time to full charge could be indicated.

In the table below, the following assertions are made:

-	Table header: C stands for Create FLUS Source configuration procedure, G is for Get FLUS Source Configuration properties procedure, U is for Update FLUS Source configuration properties procedure and T is for Terminate FLUS Source configuration procedure. "I", and "O" respectively denote "request" (going Into the FLUS sink), and response (going Out of the FLUS Source).

-	Optional ("O") means that the property may or may not be sent/received during a REST transaction. It does not necessarily mean that the property is optional. It is possible, for example, that a session is not yet active because the FLUS Source has not set the property in any previous update transaction using the PUT or PATCH HTTP method, as opposed to representing a hint on the importance of the property for the FLUS Source.

-	A property marked as optional (O) in a request message may be present in the request. When not present in the request body, the property, if present in the FLUS Source, will not be updated.

-	A property marked as optional (O) in a response message is only present in the response when a value is assigned or changed by the FLUS Source.

-	A property marked as mandatory (M) in a response message is always present in the response. The FLUS Source provides defaults, which may be modified subsequently by the content provider.

-	A blank cell in the table means "forbidden" (the property cannot be added to the request or returned by the FLUS Source, depending on the transaction direction).

Table 5.3.6-1: List of FLUS Sink Configuration properties

| Property Name | Property Description | C I | C O | G I | G O | U I | U O | T I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| id | Identifier of the FLUS Source Configuration resource. Note that "id" is only provided within an HTTP body during the Create FLUS session response. Otherwise, "id" should be present in the message URL to identify the resource in the FLUS Source. Type; Unit; Default. Integer; None; N/A. |  | M |  |  |  |  |  |
| fu_instantiation | Identifier of the FLUS media instantiation that is used by this FLUS session. Vendor specific enumeration values shall start with "vnd-" followed by a unique vendor name and optionally followed by additional characters. The F-U instantiation shall be provided as a globally unique URN. Type; Unit; Default. URI; None; All. |  |  |  | M | O |  |  |
| entrypoint_URL | Entry point URL information (e.g., SIP URL) for establishing the F-U connection to start the media streaming. Details on the Entrypoint URL is F-U instantiation specific. |  |  |  |  |  |  |  |

*** Next Change ***

Annex A (informative):
Example deployment scenarios for FLUS sub-functions

# A.0	General

This informative Annex illustrates several deployment examples for FLUS sub-functions. The intention is to show different co-location options.

# A.1	Deployment with minimal FLUS sub-functions

## A.1.1 	Introduction

This clause describes deployments with minimal FLUS sub-functions. Only F-U and F-C end-points are deployed.

Editor’s Note: Add a general description of the API end-point and the presence of implementation specific application functions, which use the APIs.

## A.1.2 	Deployment with a Media Source and a Control Source co-located in the same device

This clause contains a deployment, where the Control Source and the Media Source are deployed on the same device. The Media Source is provisioned via a local API with the selection F-U instantiation and the Media Sink ingest information.

Figure A.1.2-1: Deployment with Minimal FLUS Subfunctions

It is assumed that the workflow is started from the Control Source. For example, a human user uses the Control Source functions (via the exposed API).

It is assumed that the FLUS sub-functions expose not-3GPP defined APIs and that application functions control the line-up of sub-functions and their configuration.

## A.1.3 	Deployment with a separate control source sub-function

This clause contains a deployment, where the Control Source and the Media Source are deployed on different devices. The Control Source is located on a CTRL device.

Figure A.1.3-1: Deployment with minimal FLUS sub-functions

It is assumed that the workflow is started from the Control Source. For example, a human user uses the Control Source functions (via the exposed API). It is assumed that the Media Source has a local GUI so that it can be provisioned via the selection F-U instantiation, the Media Sink ingest information and not defined additional configurations.

It is assumed that the FLUS sub-functions expose not-3GPP defined APIs and that application functions control the line-up of sub-functions and their configuration.

# A.2	Deployments with FLUS remote control sub-functions

## A.2.1 	Introduction

This clause describes deployments where FLUS remote control sub-functions are present.

## A.2.2 	Deployment with stand-alone Control Source and Remote Controller sub-functions

This clause contains a deployment, where the Control Source and the Remote Controller are deployed on the same device (called CTRL device). The Remote Control Target is co-located with the Media Source on the same device (the FLUS Source).

Figure A.2.2-1:

The CTRL device is reachable from the FLUS Source and can reach the FLUS sink. The CTRL device is used as main configuration and control device. The FLUS Source can be configured at least with the address of the Remove Controlled function within the CTRL device.

The Remote Control Target is provisioned with the address of the Remote Controller. Once the F-RC session is established, the FLUS Source can be provisioned via F-RC.

It is assumed that the FLUS Source contains an application function, which can convey the configurations from the Remote Control Target to the Media Source and the Capture Device. The F-U instantiation and the Media Sink selection is configured via F-RC. The F-RC interface can provide more additional, implementation specific properties, which support the provisioning of the Capture Device.

## A.2.3 	Deployment with a Remote Controller co-located with a Control Sink sub-function

This clause contains a deployment, where the Remote Controller is co-located with the Control Sink on the same device. The Remote Control Target is deployed with the Media Source on the same device (the FLUS Source).

The FLUS Source is provisioned with the Remote Controller information via a local API. The FLUS Source receives the provisioning information of the selected F-U instantiation and the Media Sink ingest information via the Remote Control Interface.

Figure A.2.3-1:

It is assumed, that the CTRL device is used as main control function. The implementation may extend the F-C information so that all necessary configurations can be relayed.

Check with Charles thing

Add an API to Remote Controller

Clarify missing information, which are realized by “framework usage”

## A.2.4 	Deployment with a Control Source co-located with a Remote Control Target sub-function

This clause contains a deployment, where the Control Source is co-located with the Media Source on the same device. The FLUS Source is provisioned with the Remote Controller information via a local API. The FLUS Source receives the provisioning information of the FLUS Sink, the selected F-U instantiation via the Remote Control Interface.

Figure A.2.4-1: Stand Alond End-Points

*** Last Change ***