3GPP TSG-SA4 Meeting #104	S4-190791

# Cork, Ireland, 1st Jul 2019 - 5th Jul 2019                                       revision of S4-190787

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.929 | CR | 0002 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VR QOE metrics updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_QoE_VR |  |  |  |  |  | Date: |  |  | 2019-06-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is only FOV of the device reported in VR QOE metrics，but the user may adjust FoV to achieve a better experience. For example, the HMD can provide 120 degree FOV, but the user feels dizzy and uncomfortable for 120 degree FOV is too large for him. Then he adjusts the FOV to a smaller value. So the rendered FOV indicates user’s action during the VR playback and is also needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the rendered FOV in VR QOE metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can’t get the whole user’s QOE information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.1,10.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================== Start of  change=================================

# 10	VR device impact on QoE

## 10.1	Introduction

Compared with traditional streaming video, the key feature of VR service is to create immersive experience and enable smooth interactivity between user and the environment, in which VR device would play an important role. This contribution proposes device information relevant to user experience of VR service. All the device property information could be collected by OP5 of the reference model described in clause 6.1.

## 10.2	QoE metrics relevant with VR device

### 10.2.1	Field of View

One of the factors that contribute to the uniqueness of 360 video experience is the level of immersion induced by the wider FoV of HMD, which represents the extent of observable environment at any given time. A wider FoV could help provide a more authentic feeling of immersion. Thus, FoV of the HMD is an important parameter that helps evaluate to what extent a VR device could help create immersive experience.

The user may adjust the FoV for a programmable HMD device to achieve a better experience. The adjustment may depend on the VR content or the user's preference. A record of the rendered FoV may provide information about the user's most preferred FoV during the VR playback.

### 10.2.2	Resolution

Resolution here is defined as for per eye. An appropriate screen resolution would provide the best and comfortable experience.

### 10.2.3	Refresh Rate

Refresh rate is the number of times per second the display grabs a new image from the graphic processing unit. Lower refresh rate would contribute to processing latency and lead to VR sickness, i.e. viewing glitches on the screen. While higher refresh rate adds to the sense of presence in virtual worlds.

### 10.2.4	Decoder capability

The support of codec profile and level is an important property that decides the content types it could decode.

### 10.2.5	Detailed QoE metrics

The QoE metrics relevant with VR device as listed in Table 10.2.5.1 is necessary for assessment of device impact on user experience, and suggested to be collected by VR client. And since these metrics are generally static for VR device, the metric is only logged at the start of each QoE reporting period.

Table 10.2.5.1: QoE metrics relevant with VR device

| Key |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- |
| DeviceInformationList |  |  |  | List | A list of device information objects. |
|  | Entry |  |  | Object | A single object containing new device information. |
|  |  | resolution |  | Object | Display resolution for each eye |
|  |  |  | videowidth | Integer | Number of pixels in display width |
|  |  |  | videoheight | Integer | Number of pixels in display height |
|  |  | refreshRate |  | Integer | The number of times in a second that a display hardware updates its buffer |
|  |  | decoderCapability |  | Set | Codec profile and level the device supports |
|  |  | fieldofview |  | Object | Information of end device FoV capability. |
|  |  |  | horizontalFoV | Integer | Horizontal FoV of the device in degrees. |
|  |  |  | verticalFoV | Integer | Vertical FoV of the device in degrees. |
|  |  | RenderedFOV |  | Object | Information of the rendered FOV |
|  |  |  | hRenderedFOV | Integer | Horizontal rendered FOV in degrees |
|  |  |  | vRenderedFoV | Integer | Vertical rendered FOV in degrees |