3GPP TSG-SA WG4 Meeting #105 	S4-190847

# Ljubljana, Slovenia, 12 - 16 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | draft | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Session Handling for 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 05/08/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No control session was available No live session was considered |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add control session Add live session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 5.2	Establish a Unicast Media Downlink Streaming Session

## 5.2.1	Progressive Download of On-Demand Content

This procedure describes the establishment of a Unicast Downlink Streaming Session for On-Demand Content. Generally, a streaming session may use DASH format, 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats. This procedure assumes a progressive download content.

Figure 5.2-1: High Level Procedure for progressive download for on-demand content

Steps:

1: The App triggers the Service and Content Discovery procedure.

The Content Discovery procedure only involves the App and the external Application Server and hence is out of scope of the present document.

2: A Media Content is selected.

3: The App triggers the Media Session handler to initiate the playback. The media entry URL entry is provided to the Media Session handler.

4: The Media Session Handler communicates with the media AF to establish a control session (based on content metadata and client information).

5: The Media AF notifies the Media Session Handler about the establishment of the control session and provides transport session parameters.

The 5GMS Player establishes the transport session.

6: The Media Session handler notifies the 5GMS Player on the established control session and provides the transport session information.

7: The 5GMS Player establishes the transport session

8: The 5GMS Player requests the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

9: The 5GMS Player receives the initialization information of the progressive download content.

10: Optional: 5GMS Player acquires a DRM License from an External AS.

11: The 5GMS Player configures the playback and decryption pipeline for media playback.

12: The 5GMS Player continuously receives and plays back the media content.

### 5.2.2	Joining a DASH Streaming Session of a Live Content

It is assumed that several live sessions are offered and the UE joins an ongoing live session.

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.2-2: High Level Procedure for DASH content

Steps:

1: The App triggers the Service and Session Discovery procedure. The Session Discovery procedure only involves the App and the external Application Server.

2: A media session is selected.

3: The App triggers the Media Session handler to join the media session. The Media Player Entry is provided to the Media Session handler.

4: The Media Session Handler initiates a control session taking into account the session parameters and the client information. It determines for example the number of needed transport sessions for media acquisition.

5: The Media Session Handler notifies the 5GMS Player on the established control session.

6: The 5GMS Player establishes the transport session referenced by the Media Player Entry.

7: The 5GMS Player requests the manifest

8: The media access client receives the manifest

9: The 5GMS Player processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

10:	The 5GMS Player requests and receives initialization information

11: Optional: the 5GMS Player acquires the necessary DRM information, for example a DRM License.

12: The 5GMS Player configures the playback and rendering pipeline.

13: The 5GMS Player requests media segments according to the manifest at the live edge.

14: the 5GMS Player receives media segments and puts the information into the according media rendering pipeline.

15: Previous steps are repeated according to the manifest information.