Source:	Dolby Laboratories, Inc.

Title:	Suggested updates to server-based spatial voice conferencing usage scenario

Document for:	Agreement

Agenda Item:	7.5

- Introduction

At the SA4#104 meeting, the description of the server-based spatial voice conferencing usage scenario of IVAS was progressed based on input [1] and edits made during the EVS SWG session. Due to lack of time the edits could not be finalized, and it was invited to progress the description of the usage scenario further such that it can be completed at the SA4#105 meeting.

This contribution introduces some further edits of the usage scenario description of server-based spatial voice conferencing, based on the output version of the SA4#104 meeting included in the latest version of Pdoc IVAS-9 [2]. The source proposes to implement the suggested edits in IVAS-9 and to agree on the usage scenario.

      - Server-based spatial voice conferencing

Usage Scenario Name

Server-based Spatial Voice Conferencing

Description

Figure 3 illustrates one typical realization of the spatial conferencing scenario. It demonstrates the fact that conferencing situations may consist of many endpoints with significantly different capabilities, for instance in terms of supported codec, bit rate, audio bandwidth, capture and render. Some of the endpoints may be 3GPP UEs, some may be PSTN or generic VoIP clients. Among the 3GPP UEs, not all will necessarily support IVAS but only legacy codecs like AMR, AMR-WB or EVS. There is also some capability variety among the IVAS-enabled UEs. In this specific scenario, all participants are connected through a call server (MRFP) that handles mixing, forwarding, and floor control functions. Whilst other conferencing architectures are possible, the use of a centralized server architecture is practical when a diverse set of endpoints with very different capabilities participate in the same conference. Figure 3. Illustration of a multi-party conference call for a spatial conferencing service. The following text discusses in detail the functionality of the nodes of the spatial conference scenario depicted in Figure 3. 1: Call server: While multi-party calls are sometimes envisioned as peer to peer, practically speaking, when many different types of endpoints need to be serviced as part of the same conference, a server-based architecture becomes attractive. In this usage scenario, conference participants join using a range of endpoints, from PSTN lines, mobile connections, mobile apps (over the top), computer apps, and room systems. Since PSTN connections will continue to represent a significant connection type, transcoding efficiency in the server is still of relevance for the operation of such a service, making decode and re-encode efficiency important. While a Selective Forwarding Unit (SFU) style conferencing server is efficient and may look very appealing, this is infeasible when transmitting to PSTN endpoints and other endpoints that can only receive a single mixed stream. The endpoints may have a wide range of rendering capabilities and the service will need to support endpoint rendering spanning from mono, stereo, headphone rendering (binaural), to multi-channel room systems. Likewise, the conference needs to support diverse audio format capabilities upstream. Clients most commonly use mono as upstream audio format, but endpoints with stereo or multi-channel (e.g., 1st order B-format) capture exist today. The server must be capable of managing these various formats, mix in a common domain, and forward appropriately. 2: Standard PSTN/other PLMN endpoint, whether from an IP-PBX, home phone, or any endpoint residing in a different operator network (in case the interconnect involves transcoding at network edge): This provides the most ubiquitous access and fallback for connectivity and is supported as a matter of course. This is likely to represent a significant fraction of conference attendees for the foreseeable future, though it is trending downward with time. The key element here is that the PSTN/other PLMN endpoint must receive a mono downmix of the signal and encode in the appropriate format. Thus, the concept of a stream forwarding unit is not applicable here – the server must mix in some manner and re-encode to the required PSTN/PLMN interconnect codec (G.711 being mandatory and always a fallback). 3: Legacy UE running a mono codec (EVS, AMR-WB, AMR-NB): While currently it is uncommon for mobile service providers to terminate AMR-NB or AMR-WB on a conference server, such termination is both feasible and under consideration by conference service providers. This can be viewed as a higher quality mono endpoint than is feasible with G.711 transcoding and connect. A shift to an EVS mono codec does not fundamentally change the fact that mono up and mono down, while of good quality, does not provide a spatial conferencing experience to the legacy UE. However, it is worth noting that in a server-based architecture, mono sources can be spatially mixed into an audio scene that is delivered to a spatial endpoint. Thus, a user on a spatial endpoint, conversing with multiple mono endpoints, can still receive a spatial experience. 4: UE: This represents the target of IVAS in the conferencing scenario. The access over a mobile network (LTE, 5G) provides extensive coverage. Other access techniques (WiFi, even OTT via mobile access) may be included. Typically, the audio output is binaurally rendered to headphones but could also be stereo rendered for the increasing number of mobile phones and tablets that have stereo speakers. Mono playout may also frequently occur. Thus, IVAS-enabled UEs should support and apply one out of multiple render types (binaural, stereo, mono). Since the UE is typically connected over a mobile network, there are likely to be bandwidth limits, both upstream and downstream, which may be time varying. According to the nature of mobile communications, the transmission may be prone to errors. Thus, bandwidth efficiency and error robustness are important in this usage scenario. 5,6: Conference room clients. These two represent the same endpoint type (endpoint in conference room or home), where the endpoint has the capability of spatial capture and spatial playout. The clients may be with or without 3GPP connectivity, natively connected or over the top. The playout could be 1,2,3,..N channel loudspeaker render. The reason that two endpoints are shown is to emphasize the fact that conferencing with two or more rooms is a very common usage scenario and mixing and merging of multiple spatial captures at the server can represent a usage scenario where the computational complexity is relatively high. The key thing to recognize here is that spatial capture and coding may require a significantly higher bandwidth (both upstream and downstream) than other examples, so bit rate efficiency is critical. 7. Computer client: Typically with headset, running a conferencing client and with or without 3GPP connectivity. This may be an application running on the computer, connected over the top via a reasonably high bandwidth upstream and downstream link. In this example, the computer is playing out through a headset, so that the signal should be binaurally rendered. Capture is typically mono because in this usage scenario there is usually only one talker. 8. Stereo device: Devices like 5,6 or 4 but with stereo capture and stereo render capability. Stereo capture might be obtained using microphone pairs in various possible configurations such as XY, M/S, ORTF or A/B. 9. Content ingest: This is an example of pre-recorded or live streaming content that might be fed into and delivered over the system. This could be served for instance while the conference has not yet started or is on hold. Announcements, jingles, etc. to conference participants also fall into this category. Typically, it involves upstream transmission to the conference server only (i.e., there is no downstream audio path from the server to this node). 10. Recording/analytics: This is an example of a recording function of the conference. Audio analytics features like automatic transcription, speaker annotation, conference control keyword spotting, etc. could also be part of this function. Typically, unless this functionality is realized physically in the call server, it involves downstream transmission without strict bit rate limitations.

Categorization

Type: Immersive Degrees of Freedom: 3DoF Delivery: Interactive, Conversational Media Components: Audio-only, Audio-Visual Devices: Legacy UE, UE, PSTN phones, Call server, Desktop/Laptop computer, Conference systems, Content ingest server, Recording / analytics server

Preconditions

TR 26.980 describes many elements of conferencing using a server (including describing a mixing server and stream forwarding server and combinations of the two). However, the emphasis here is that a spatial conferencing service will have endpoints with different spatial capabilities and thus a key element of the system (and server) is to ensure that all endpoints can deliver high quality audio output to their users, according to their capabilities and the constraints imposed by the network and by the operator offering the service. To meet this general precondition, server-based mixing is practical. While server-based mixing is often a precondition to support a broad array of endpoints, the server design that maximizes flexibility and quality is one that combines both mixing and stream forwarding. For example, it may be beneficial to stream forward a small number of mono voice endpoints and at the same time mix other endpoints. This is realized with codecs allowing both a main audio stream (which is mixed and may be immersive) as well as substreams (which are forwarded but not mixed) and are typically mono streams. Though in practical systems, there are also server designs that require multiple immersive substreams to be sent in parallel. In the industry, it is common to formulate a conferencing service using geographically cascaded servers to form a single “virtual” server. This geographical cascade allows geographically localized participants to speak with short round trip times across their local server while geographically distant participants experience a longer round-trip time. This approach also reduces long backhaul traffic because each local server may mix some traffic, prune some traffic, and selectively forward some traffic. To this end, multiple audio substreams are assembled into common frames for between-server traffic. In these cases, it is common for a secondary server to mix some of the previously forwarded traffic into the main stream for delivery to endpoints, to drop some traffic, and to retain some traffic as independent substreams. Ideally, substreams should be extracted or inserted by parsing of the codec bit stream rather than doing a full decode.

QoS/QoE Considerations

The following considerations related to QoS/QoE of the server-based spatial voice conferencing usage scenario can be made: - In general, it can be expected that spatial multi-party conferencing will be dominated by mono, two-channel, and 1st order B-format representations. The 1st order B-format representation in the considered usage scenario may typically omit the vertical component because that direction is not particularly useful in audio voice conferencing. - Mono coding performance: High quality/rate performance should be provided for voice (clean, noisy, reverberant), generic audio and music. It should be assumed that noise reduction to a certain degree is applied in most cases. “Far voice” or capture and encoding of reverberant speech is a frequent occurrence and measures like applying a sufficiently high bit rate should be taken to avoid coding artefacts introduced by room reverberation. Concurrent talkers (overtalking) is a frequent occurrence and, accordingly, measures should be taken to maintain high quality when two or more voices are mixed and encoded. - Two-channel coding performance: High quality/rate performance should be provided for voice (clean, noisy, reverberant), generic audio and music captured with various setups and in various environments. Capture in conference rooms is a relevant scenario for which it should be assumed that noise reduction to a certain degree is applied and that multiple talkers talking on top of each may frequently occur. Thus, mixed voices coming from different angles are common and should be coded with high quality. Various relevant microphone layouts should be supported, such as XY, M/S, ORTF, A/B. “Far voice” or capture and encoding of reverberant speech is a frequent occurrence and measures like applying a sufficiently high bit rate should be taken to avoid coding artefacts introduced by room reverberation. High quality should also be achieved for ambient sounds, including speech, music or nature sounds, and not only for the (foreground) voice. It should be ensured that the codec does not create onerous artefacts from common audio events in real rooms or real environments. - Spatial coding performance: High quality/rate performance should be provided for voice (clean, noisy, reverberant), generic audio and music captured over a planar B-Format microphone system. Capture in rooms for conferencing typically have noise reduction to a certain degree applied, but there may be multiple simultaneous talkers from different room directions. The spatial coding of reverberant speech and speech in noise should not cause significant degradations. Reverberant speech capture is an extremely common (or just normal) situation in spatial capture of speech. It should be ensured that annoying artefacts introduced into the ambiance by the encoding process are avoided, e.g. by using a sufficiently high bit rate. High quality should also be achieved for ambient sounds, including speech, music or nature sounds, and not only for the foreground voice. The codec should not create onerous artefacts from common audio events in real rooms or real environments. High quality/rate performance should also be achieved for coding of B-format content with additional vertical component. - If supported by the codec, for complexity reasons it may be beneficial for receiving endpoints with mono only rendering capabilities to use coding modes that allow high-quality mono decoding of multiple-channel representations (two-channel or spatial) without the need to decode and downmix the multiple-channel representation. - The service should be set up such that it supports high-quality two-channel rendering (binaural/stereo) derived from mono, two-channel or spatial ingests . - The service should be set up such that it supports high-quality multi-channel (>= 3 channels) (spatial) rendering derived from mono, two-channel or spatial ingests. - High complexity decodes and encodes on the server may result in low scalability (and high cost) for the server-based spatial voice conferencing service. Encoding and decoding complexity of the used codec modes may thus be an important criterion when setting up the service.

Feasibility

Spatial audio conferencing systems already exist using Internet-based (IP) protocols, therefore implementation as a 5G service is feasible.

Potential Standardization Status and Needs

It is expected that there will be a specification need for the handling and interaction of the media (audio/video) associated with the conferencing server as well as of the service setup parameters.

# References

[1]	S4-190669: “IVAS-9 comments”, Ericsson LM

[2]	S4-190834: “IVAS Usage Scenarios (IVAS-9)”, Editor