Source:	Fraunhofer IIS

Title:	IVAS Output Formats

For:	Agreement

Agenda Item:	7.5

- Introduction

At the last SA4#104 meeting a first agreement on input to output format mapping was reached, with some formats not yet being covered. Following the previous proposals by the source and also other contributions, a completion of the table is proposed within this document.

More specifically, it is proposed to amend the table by the following Encoder Input Formats:

- Scene-based audio

- Object-based audio

- Multi-channel 7.1+4 and 5.1+4

- Combinations of input formats

For the multi-channel formats 7.1+4 and 5.1+4, it is proposed to follow the same philosophy as for the agreed formats 7.1 and 5.1. For all other formats it is proposed to include all possible (input) multi-channel formats (5.1, 7.1, 5.1+4, 7.1+4), binaural audio, stereo and mono. For combinations of formats (not including Binaural audio), or a superset of the Rendered Output Formats of the respective Encoder Input Formats (logical OR) is proposed. For combinations of formats including Binaural Audio, Binaural Audio is proposed as rendered output format.

- Proposed change to IVAS-4

| Rendered Output Formats | The IVAS codec shall support the following rendered output formats for the corresponding input format: Encoder Input Format; Rendered Output Format. Multi-channel 7.1; Multi-channel 7.1, Binaural Audio, Stereo, Mono.. Multi-channel 5.1; Multi-channel 5.1, Binaural Audio, Stereo, Mono.. Binaural Audio; Binaural Audio. Stereo; Stereo, Mono. Mono; Mono. Scene-based audio, first-order (FOA) and up to [N]-order Ambisonics; Multi-channel (5.1, 7.1, 5.1+4, 7.1+4), Binaural audio, Stereo, Mono. Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]; Multi-channel (5.1, 7.1, 5.1+4, 7.1+4), Binaural audio, Stereo, Mono. [Multi-channel 7.1+4; Multi-channel 7.1+4, Binaural audio, Stereo, Mono]. [Multi-channel 5.1+4; Multi-channel 5.1+4, Binaural audio, Stereo, Mono]. Combinations of the above Encoder Input Formats, not including Binaural Audio; Superset of the above Rendered Output Formats of the respective Encoder Input Formats (logical OR). Combinations of the above Input Formats, including Binaural Audio; Binaural Audio. Editor’s note: Specification of rendered output formats for the remaining input formats is needed. |
| --- | --- |

- Summary and Conclusion

It is proposed to include the changes outlined in Section 2 into the IVAS design constraints IVAS-4 [1].

- References

[1] 3GPP S4-190839, IVAS Design Constraints (IVAS-4), v0.1.0