Source:	HEAD acoustics GmbH, Intel

Title:	Status of RR-Test for FS_ANTeM

Document for:	Discussion

Agenda Item:	9.7

# Introduction

For device testing in handset mode, the reproduction accuracy of the system according to ETSI ES 202 396-1 [1] was evaluated across labs with a round-robin test [2]. Also, several variants of the noise field simulation systems according to ETSI TS 103 224 [3] were taken into account: the eight-channel microphone array as well as binaural equalization are considered here, too.

This contribution presents the current status of the round robin test.

# Status of RR-Test

The conducted measurements followed the test plan of the round robin test as specified in [2], no update or modification was necessary. Up to five noise field simulations were tested per bandwidth, device and room.

Due to several issues in logistics, the devices arrived late at HEAD acoustics GmbH. Four measurement rooms should be investigated, but only two were completely covered. Measurements in the third room are ongoing, the fourth room could not be evaluated within the given time frame - but the remaining measurements are certainly scheduled.

So far, more than 3300 measurements were collected, each with a duration of at least 80 seconds. This corresponds to more than 73h of recording time. Even though values for S-MOS and N-MOS can easily be calculated and reported, consistency checks as well as organisation and visualization of data need some more additional time.

# Noise field simulations

An overview about the investigated noise field simulations is provided in Table 1. The short names in the first column are used in the following clauses to differentiate between the systems. More detailed descriptions are given in clause 5.3.2 of TR 26.921 [5]. Eight noise types according to Tables 1 and 2 of the test plan [2] were used for each noise field simulation to generate the recordings.

Table 1: Overview of noise field simulations

| Noise field simulation (short name) | Equalization Points | Noise source from |
| --- | --- | --- |
| ES202 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-4.0 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-4.1 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-8.0 | Ear Mics (2) | ES 202 396-1 (binaural) |
| TS103-HS | Mics of array (8) | TS 103 224 (8 Channels) |

# Devices

The devices according to Table 2 are currently available for testing. In addition to the test plan, one more device was tested in some labs.

Table 2: Devices for round robin test

| Device ID | Manufacturer | Year | VoLTE possible | Test in NB | Test in WB | Test in SWB |
| --- | --- | --- | --- | --- | --- | --- |
| DUT 01 | A | 2018 | Yes (only for EVS- SWB) | Yes | Yes | Yes |
| DUT 11 | B | 2018 | Yes | Yes | Yes | Yes |
| DUT 08 | C | 2015 | No | Yes | Yes | No |
| DUT 09 | D | 2012 | No | Yes | Yes | No |
| DUT 10 | E | 2014 | No | Yes | Yes | No |
| DUT 05 | F | 2018 | Yes | Yes | Yes | Yes |
| DUT 04 | F | 2017 | Yes | Yes | Yes | Yes |
| DUT 07 | G | 2018 | Yes | Yes | Yes | Yes |
| DUT 06 | H | 2018 | Yes | Yes | Yes | Yes |
| DUT 39 | I | 2017 | Yes (not for EVS- SWB) | Yes | Yes | No |

Four out of these ten devices could not be evaluated in SWB mode due to several reasons (at least in conjunction with test equipment):

- DUT 09 and 10 do not provide LTE functionality (no VoLTE possible).

- DUT 08 offers VoLTE calls in the settings, but none of the codecs worked.

- DUT 39 supports VoLTE calls, but EVS-SWB codec seems not available at all.

# Rooms

A detailed description of the participating labs/rooms can be found in S4-190954 [4].

Due to internal restructuring and time constraints, room #6 could only be used two evaluate two devices (DUT 01 & 11).

# Additional measurements

After a submission of a liaison statement [9], four devices of the round robin test were kindly provided by CTIA CPWG, sub-working group “Audio”. In return, they asked to provide some basic measurements according to TS 26.132 [10] in NB, WB and SWB mode. These additional measurements were addressed described in the latest test plan [2] and were collected in advance to the ambient noise testing.

Sending Direction:

- DUT Delay (compensated for test equipment delay)

- Frequency Response

- Loudness Rating

- Receiving Direction:

- Predicted speech quality acc. to ITU-T P.863 V2.4

Receiving Direction:

- DUT Delay (compensated for test equipment delay)

- Frequency Response

- Loudness Rating (at nominal level)

- Predicted speech quality acc. to ITU-T P.863 V2.4 (at nominal level)

The results of these additional measurements will be reported after finalization of the whole round robin test. A dedicated contribution is expected for SA4#106.

# Conclusion

Due to several logistic issues, the round robin test could not be finalized in time for SA4#105, but testing is currently ongoing. Thus, intermediate results could not yet be verified and cross-checked in order to investigate possible inconsistencies between labs and/or devices. Final results will be presented at SA4#106 and the time plan has to be adjusted accordingly.

# References

| [1] | ETSI ES 202 396-1 v1.7.1, „Speech Processing, Transmission and Quality Aspects (STQ); Speech quality performance,“ 10/2017. |
| --- | --- |
| [2] | 3GPP S4-190683, „Test plan for a Round-Robin-Test - comparison of noise field simulations for handset mode v0.2,“ HEAD acoustics GmbH, Intel, Cork, Ireland, 05 July 2019. |
| [3] | ETSI TS 103 224 v1.3.1, „Speech and multimedia Transmission Quality (STQ);. A sound field reproduction method for terminal testing including a background noise database,“ 07/2017. |
| [4] | 3GPP S4-190954, „Improved description of labs participating in RR-Test,“ HEAD acoustics GmbH, Intel, Ljubljana, Slovenia, 16 August 2019. |
| [5] | 3GPP TR 26.921, „Investigations on ambient noise reproduction systems for acoustic testing of terminals,“ in Release-16. |
| [6] | ETSI TS 103 106 v1.3.1, „Speech quality performance in the presence of background noise: Background noise transmission for mobile terminals-objective test methods,“ 04/2014. |
| [7] | ETSI TS 103 281 v1.2.1, „Speech quality in the presence of background noise: Objective test methods for super-wideband and fullband terminals,“ 01/2018. |
| [8] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release 16.0.0. |
| [9] | 3GPP S4-190554, „LS on Round-robin test plan for FS_AnTEM (To: CTIA),“ TSG SA WG4, Newport Beach, USA, 17 April 2019. |
| [10] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release 16.0.0. |
| [11] | 3GPP S4-181500, „New SID on ambient noise test methodology for evaluation of acoustic UE performance (FS_ANTeM),“ HEAD acoustics GmbH, Qualcomm Incorporated, Deutsche Telekom AG, Intel, HuaWei Technologies Co., Ltd, Busan, South Korea, 25 November 2018. |