3GPP TSG-SA WG4 Meeting #106 	S4-191179

# Busan, Korea, October 21-25

Source:				Fraunhofer IIS

Title:					Evaluation of potential IVAS Reference Codecs – SBA Format

Document for:		Discussion

Agenda Item:		7.5

## 1.	Introduction

At SA4#105, the list of potential legacy codecs for the IVAS performance definition and evaluation was amended. To progress discussion on the IVAS performance requirements, this contribution presents listening test results for SBA conditions (FOA), where the performance of 4x EVS and 2x enhanced AACplus approx. 64 kbit/s is compared. For reference, also a non-3GPP communication codec is included.

## 2.	Listening Test

### Test Setup

To evaluate the performance of the reference conditions, a MUSHRA listening test with the following conditions was conducted:

| Label | Description |
| --- | --- |
| hiddenRef | Hidden Ref (FOA), uncoded |
| LP3k5Hz | 3.5 kHz Anchor (FOA), uncoded |
| EVS_FOA | FOA, EVS SWB @ 4x 16.4 kbit/s (= 65.6 kbit/s) |
| EVS_AFORMAT | A-Format, EVS SWB @ 4x 16.4 kbit/s (= 65.6 kbit/s) |
| eAACp_A | A-Format, enhanced AACplus @ 2x 32.0 kbit/s (= 64.0 kbit/s) |
| Other_FOA | FOA, non 3GPP Codec @ 2x 32.0 kbit/s (= 64.0 kbit/s) |
| Other_AFORMAT | A-Format, non 3GPP Codec @ 2x 32.0 kbit/s (= 64.0 kbit/s) |

All conditions were rendered to and played back on a 7.1+4 loudspeaker layout according to the CICP19 configuration described in MPEG-H Audio [2]. The FOA components were all in ACN-SN3D format and rendered on loudspeakers using the same FOA renderer namely the MPEG-H 3D Audio Ambisonics renderer [2] combined with an ACN-N3D format conversion.

For A-format conditions, the FOA signals were first transformed into A-Format signals before being coded. The decoded A-format signals were then transformed back to FOA ACN SN3D before being rendered to loudspeakers. The 4 A-Format signals are defined as the 4 cardioid virtual-microphone signals defined at the positions given by the following table:

| azimuth | elevation |
| --- | --- |
| + α | 0 |
| - α | 0 |
| 0 | + α |
| 0 | - α |
| Where α = acos(1/sqrt(3)) |  |

There were 12 samples tested coming from different sources. The items have been selected to obtain a varied representation of possible 3D audio scenes where the focus was on microphone array recordings and user generated content, according to the focus of the IVAS WI. A brief description for each sample is given below.

| Name | Description |
| --- | --- |
| Item 1 (synthetic) | 3 point-sources (voice, piano and clap) positioned in horizontal plane |
| Item 2 (synthetic) | 2 speakers positioned in horizontal plane at 0 and 180 degrees |
| Item 3 (synthetic) | 2 speakers positioned in horizontal plane at +90 and -90 degrees |
| Item 4 (synthetic) | 4 speakers positioned at 35 degrees of elevation and +/- 45 and +/-135 degrees of azimuth |
| Item 5 (recorded) | EigenMike recording of 4 participants natural conversation in English |
| Item 6 (recorded) | EigenMike recording of 4 English speakers |
| Item 7 (recorded) | EigenMike recording of 4 German speakers |
| Item 8 (recorded) | EigenMike recording of 4 German speakers with background street noise |
| Item 9 (recorded) | Percussions |
| Item 10 (recorded) | Classical music recording |
| Item 11 (recorded) | Live recording of crowd cheering with applause |
| Item 12 (recorded, post-produced) | 3D sound effects |

### Results

A total of 6 expert listeners participated in the test. The evaluation of the test results was performed using 95% confidence intervals and Student-t distribution.

The following observations could be made from the results of the test

- Over all items, EVS at 4x 16.4 kbit/s (A-Format) is the best performing condition. It is ranked good.

- Over all items, EVS at 4x 16.4 kbit/s (FOA) is seen on par with enhanced AACplus at 2x 32.0 kbit/s (A-Format)

- Over all items, in this test the A-Format representation seems to have a slight advantage over coding FOA coefficients separately. The source however acknowledges that this might be dependent on item and possibly also bitrate, which would also explain earlier results which hint towards different conclusions.

- Over all items, the included non-3GPP codec does not show significant advantages over the 3GPP codecs.

## 3.	Conclusion

A listening test was carried out comparing reference codecs for coding of FOA at approx. 64 kbit/s using EVS, enhanced AACplus and a non-3GPP codec. The solution based on EVS was outperforming the other conditions. It is proposed to consider these results when completing the IVAS performance requirements.

## 4.	References

[1] 3GPP S4-191064, IVAS Performance Requirements (IVAS-3), v0.0.4
[2] ISO/IEC 23008-3, Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 3: 3D audio