## Source:	Fraunhofer IIS

## Title:	Comments on 26.117

## Document for:	Discussion

## Agenda Item:	8.8

# Summary

This contribution addresses some open points in 26.117, where there was some discussion on EVS for 5GMS3, with the text for EVS being in an early phase. Some additions are proposed to the text for EVS mono, text for EVS dual-mono is also proposed (but still being kept in square brackets).

Changes to clause 2 – add reference:

[X]	3GPP TS 26.452: "Codec for Enhanced Voice Services (EVS); ANSI C code; Alternative fixed-point using updated basic operators".

Changes to clause 3:

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Bitstream: A media bitstream that conforms to an audio/speech encoding format and certain Operation Point.

[ Dual-mono: A variant of 2-channel stereo encoding where two instances of a mono codec are used to encode a 2-channel stereo signal. ]

Operation Point: A collection of discrete combinations of different content formats and the encoding format.

Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

New populated clause 6.2.5 (still in square brackets) – 6.2.4 as a template included for convenience:

### 6.2.4	EVS

#### 6.2.4.1	Bitstream Encoding Requirements

The following requirements apply to the EVS Operation Point:

- The sampling frequency shall be one of the following: 8, 16, 32, 48 kHz.

- The bitstream shall be encoded according to either TS 26.442 [14], TS 26.443 [15], or TS 26.452 [X] encoding functions.

- The EVS Primary mode shall be used

- A single mono channel shall be encoded, i.e. one speech frame per (file format) sample

Note that the bitstream produced by the EVS encoder consists of 20ms encoded speech frames.

#### 6.2.4.2	Receiver Requirements

Receivers conforming to the EVS Operation Point shall support the EVS speech media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

### [6.2.5	EVS Dual-Mono

#### 6.2.5.1	Bitstream Requirements

The following requirements apply for the EVS-Dual-Mono Operation Point.

- The sampling frequency shall be one of the following: 8, 16, 32, 48 kHz.

- The bitstream shall be encoded according to either TS 26.442 [14], TS 26.443 [15], or TS 26.452 [X] encoding functions.

- The EVS Primary Mode shall be used for dual-mono

- The channelcount field in the EVSSampleEntry shall be set to "2".

- Speech frames for left and right channel shall be time-aligned

Note that the bitstream produced by the EVS encoder consists of 20ms encoded speech frames. Two EVS encoders are required to generate the speech frames for the left and right channel independently.

The packetization of EVS dual-mono is described in [16] (see clause A.2.6.2).

#### 6.2.5.2	Receiver Requirements

Receivers conforming to the EVS-Dual-Mono Operation Point shall support two instances of the EVS speech media decoding capability according to clause 5.2 and shall support playback of the decoded signal in stereo.

]