## Source:	Fraunhofer IIS

## Title:	On Complexity and Delay Constraints for IVAS

## Document for:	Discussion & Approval

## Agenda Item:	7.5

# 1	Introduction

Besides required features the major points to be resolved in IVAS-4 [2] are design constraints on complexity and algorithmic delay. Algorithmic delay has been a single figure in previous codec standardization projects (e.g. 88wMOPS for EVS), that defined the balance of audio quality vs. delay, aiming at highest QoE for users of the service. Complexity on the other hand has often been the gross term that can be subdivided into computational complexity and memory demands. Contrary to algorithmic delay, complexity is only indirectly of relevance to operators and users of the service (by cost of terminals) but mainly affects implementers (not just in terms of cost but also in terms of product design). This contribution is to some extent a response to [1] (which raises some good points) but may differ in its conclusions.

# 2	Proposal for Delay

It could be assumed that, similarly to existing 3GPP communication codecs, a session’s configuration remains static within the boundaries of the session negotiation (using SDP offer/answer exchange). Thus, some parameters may well be mutable within a session (e.g. bit rate), while others will be steady, if not a new session is negotiated (e.g. the input format won’t normally change), having an impact on QoE and algorithmic delay requirements.

During the session setup/renegotiation it will be transparent to all entities having access to the SDP offer/answer exchange what parameters are set. This concept enables operators to monitor and control the sessions in their network and ensure that customers get a high QoE. Still there may be factors outside the operator’s control, as e.g. how a headset is connected.

Delay is one parameter of relevance for QoE. On the other hand, it could be assumed that the IVAS codec is not just a multi-rate and multi-bandwidth codec but also a multi-format codec, supporting a high variety of input formats and output formats (as pass-through, rendered for a reproduction setup or as input to an external renderer). This additional functionality could well lead to different delay paths in an IVAS candidate but even more so leave some paths in the unclear, e.g. the delay contribution of a vendor-specific external renderer.

This analysis suggests that a single algorithmic delay figure might not be appropriate, given the factors of formats and rendering – as already observed during EVS standardization (where mono and stereo coding had different requirements on algorithmic delay). It is however not a deal-breaker for carriers, as the delay can still be controlled by a service provider during SDP negotiation, when the formats are negotiated.

It is therefore proposed to define different levels for algorithmic delay, depending on e.g. input and output formats or rendering options.

This contribution adds potential ways how to define limits for algorithmic delay, which could be based on a table.

Table 1: Limits for algorithmic delay (in ms) of Encoder Input / Decoder Output Format Combinations

| IN OUT | Mono | Stereo | Bin-aural | 5.1 | 7.1 | 5.1+4 | 7.1+4 | HOA1 | HOA2 | HOA3 | 1 Obj | 2 Obj | … | N Obj |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mono | 32* | TBD | - | - | - | - | - | - | - | - | - | - | - | - |
| Stereo | TBD | TBD | - | - | - | - | - | - | - | - | - | - | - | - |
| Binaural | - | - | TBD | - | - | - | - | - | - | - | - | - | - | - |
| 5.1 | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - | - | - | - |
| 7.1 | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - | - | - |
| 5.1+4 | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - | - |
| 7.1+4 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - |
| HOA1 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - |  | - | - | - |
| HOA2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | TBD | - | - | - | - | - |
| HOA3 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | TBD | - | - | - | - |
| 1 Obj | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | TBD | - | - | - |
| 2 Obj | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | TBD | - | - |
| … | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | TBD | - |
| N Obj | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | TBD |

*) EVS

3	Proposal for Complexity

Complexity should be limited independently of content; operation modes may however well impact complexity limits. While bit rate and audio bandwidth may play a minor role, it’s especially the support for many input and output formats (and rendered vs. non-rendered etc.) envisioned with IVAS having potentially major impact to complexity. This however may again not be such a big deal, as a UE being available to create high-order scene-based audio and being able to render to multiple loudspeakers could be largely different in its allowed complexity for its IVAS codec compared to a stereo-only UE.

The analysis for algorithmic delay to some extent also applies to complexity (i.e. the transparency during SDP negotiation what the session will contain), it’s however mainly the implementors of IVAS that are affected by the complexity. This has already been foreseen in the WID for IVAS:

- The solution is expected to be implementable on a wide range of UEs to address various needs in terms of balancing user experience and implementation complexity / cost.

In the source’s opinion the wide range of UEs would mainly span devices that support different input formats, output formats or different rendering options (internal vs. external). A high-end UE vendor supporting some of the more complex input or output formats would be less concerned by higher IVAS complexity compared to a vendor of an entry-level IVAS UE that only supports e.g. stereo input and output.

This analysis suggests that a single complexity figure might not be appropriate, given the factors of formats and rendering – as already observed during EVS standardization (where mono and stereo coding had different requirements on complexity). It is however not a deal-breaker for UE vendors, as the complexity can still be controlled by the offering and answering UE during SDP negotiation, when the formats are negotiated.

It is therefore proposed to define different complexity levels, depending on e.g. input and output formats or rendering options.

This contribution adds potential ways how to define limits for algorithmic complexity, which could be based on a table.

Table 2: Limits for Complexity (in wMOPS) of Encoder Input / Decoder Output Format Combinations

| IN OUT | Mono | Stereo | Bin-aural | 5.1 | 7.1 | 5.1+4 | 7.1+4 | HOA1 | HOA2 | HOA3 | 1 Obj | 2 Obj | … | N Obj |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mono | 88* | TBD | - | - | - | - | - | - | - | - | - | - | - | - |
| Stereo | TBD | TBD | - | - | - | - | - | - | - | - | - | - | - | - |
| Binaural | - | - | TBD | - | - | - | - | - | - | - | - | - | - | - |
| 5.1 | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - | - | - | - |
| 7.1 | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - | - | - |
| 5.1+4 | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - | - |
| 7.1+4 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | - |
| HOA1 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - |  | - | - | - |
| HOA2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | TBD | - | - | - | - | - |
| HOA3 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | TBD | - | - | - | - |
| 1 Obj | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | TBD | - | - | - |
| 2 Obj | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | TBD | - | - |
| … | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | TBD | - |
| N Obj | TBD | TBD | TBD | TBD | TBD | TBD | TBD | - | - | - | - | - | - | TBD |

*) EVS

# 4	Summary

The source proposes to focus on the finalizing the required feature set for the IVAS codec, while keeping algorithmic delay and complexity in mind to avoid conflicting constraints that provide low QoE or suffer from an unrealistically high implementation burden. Finally, delay and complexity design constraints would need to be set, likely not as a single figure but e.g. depending on the combination of input and output format.

It is proposed to add Tables 1 and 2 to IVAS-4 as a template for algorithmic delay and complexity. The source also invites to propose detailed limits for the different combinations of input and output format.

[1]	3GPP AHEVS-472 – On delay and complexity design constraints for IVAS, Orange

[2]	3GPP S4-190450, IVAS Design Constraints (IVAS-4), v0.0.10