| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.928 | CR | pseudo | rev | - | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposed Conclusions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GXR |  |  |  |  |  | Date: |  |  | 14/01/2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This document assumes the document in S4-200041 and S4-200042 and S4-200043 is agreed. |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[3]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[4]	ARCore, https://developers.google.com/ar/

[5]	ARKit, https://developer.apple.com/arkit/

[6]	3GPP TR 22.842: "Study on Network Controlled Interactive Service in 5GS".

[7]	3GPP TR 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]		3GPP TS 23.501: "System Architecture for the 5G System ".

[9]	Schuemie, Martijn J., Peter Van Der Straaten, Merel Krijn, and Charles A.P.G. Van Der Mast. “Research on Presence in Virtual Reality: A Survey.” CyberPsychology & Behavior, Vol. 4, No. 2. April 2001.

[10]	Ching, Teo Choong. “The Concept of Presence in Virtual Reality.” Medium. 27 August 2016.

[11]	Sparks, Matt. “Don't Break the Spell: Creating Presence in Virtual Reality” Learning Solutions Magazine, 17 July 2017.

[12]		3GPP TS 26.501: "5G Media Streaming Architecture".

[13]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[14]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction ".

[15]	3GPP TR 22.891 "Feasibility Study on New Services and Markets Technology"

[X]	"Cloud Gaming: Architecture and Performance", Ryan Shea and Jiangchuan Liu, Simon Fraser University; Edith C.-H. Ngai, Uppsala University; Yong Cui, Tsinghua University; IEEE Network-July/August 2013.

[Y] 	M. Claypool and K. Claypool. Latency and player actions in online games. Communications of the ACM, 49(11):40–45, 2006.

[Z]	 Quax, P., Monsieurs, P., Lamotte, W., De Vleeschauwer, D., and Degrande, N. Objective and subjective evaluation of the influence of small amounts of delay and jitter on a recent first person shooter game. In Proceedings of 3rd ACM SIGCOMM workshop on Network and system support for games (New York, NY, USA, 2004), NetGames ’04, ACM, pp. 152–156.

[W] 	Chen, K.-t., Huang, P., Wang, G.-s., Huang, C.-y., and Lei, C.-l. On the Sensitivity of Online Game Playing Time to Network QoS. Proceedings of IEEE INFOCOM 2006 00, c (2006).

[V]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications"

[T]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)"

[S]	S. Schwarz et al., "Emerging MPEG Standards for Point Cloud Compression," in IEEE Journal on Emerging and Selected Topics in Circuits and Systems, vol. 9, no. 1, pp. 133-148, March 2019.

# 7	Potential Standardization Areas

## 7.1 	General

This clause documents and clusters potential standardization areas in the context of this Technical Report.

## 7.2 	XR-Centric Device Architectures

As documented in clause 4.5, XR centric devices are the key enablers for XR services. Key aspects for XR devices are:

- Rendering-centric device architectures using sophisticated GPU functionalities, see clause 4.4.

- Support for Tracking, in particular inside-out tracking in the device

- Heavily power-constrained at least for certain form factors for different form factors

- Support for multiple decoding formats and parallel decoding of video streams

- Based on standards developed predominantly in Khronos, such as glTF, Vulkan, OpenXR, etc.

In addition, device characteristics can be quite different. Hence, the device types developed in clause 4.8 serve as a starting point for different device types. A more formal definition of XR devices types is considered useful.

## In any work considered in 3GPP, end-points compatible to Khronos-based graphics and XR functionalities should be considered. A framework for interfacing device centric XR functionalities with 5G System and radio functionalities is a relevant standardization effort.

## 7.3 	Extensions to 5G Media Streaming for XR/6DoF Media

Streaming XR and 6DoF is considered in several use cases evaluated in this Technical report. With the establishment of 5G Media Streaming in Rel-16 in TS 26.501 and the stage-3 specifications, extensions of 5G Media Streaming to support XR experiences is useful. Whereas MPEG develops several new formats and and codecs for different immersive formats, also many only partial standardized formats exist. However, all of them have in common that they rely on existing and emerging device architectures that make use of existing video codecs and GPU rendering. In addition, the use of multiple video codecs in parallel is commonly applied. XR/6DoF Streaming is based on CDNs and HTTP delivery, however new functionalities are required.

Extensions to 5G Media streaming may be done in ordered to support the delivery of different XR/DoF media in 5G Systems. Relevant aspects are:

- Additional media decoding capabilities including higher profile and levels, the use of multiple decoders in parallel to interface with rendering architectures, as well more flexible formats

- Support for more flexible delivery protocols allowing parallel download of different objects and parts of objects

## Potentially new 5QIs and radio capabilities to support higher bitrate streaming

## 7.4 	Simple Split Rendering for Online Gaming

Split Rendering is a promising technology to support online gaming in power- and resource constrained devices. Split rendering requires the use of edge computing as the pose-to-render-to-photon is expected to be below 50ms. Rendering of frame buffers in the network allows to support XR devices with existing codecs using 5G System and radio capabilities. Relevant aspects for simple split rendering include:

In the context of the this architecture, the following potential standardization needs are identified:

- A simple XR split rendering application framework

- Regular 2D video encoders and decoders that are capable encode and decode 2K per eye as well as 90 fps

- Pose information in the uplink at sufficient frequency

- Content Delivery protocols that support split rendering

- Radio capabilities that support split rendering

## Edge computing discovery and capability discovery based on work in SA2 and SA6

## 7.5 	XR conversational applications

Tbd.

## 7.6 	HMD-based Augmented Reality

# More work on AR is necessary, especially for power constrained devices.

## 7.7 	Traffic Characteristics for XR Services

As identified in the course of the development of this document, there is significant interest in 3GPP radio and system groups on the traffic characteristics for XR services. This effort should be a prime work for 3GPP to collect realistic traffic characteristics for typical XR services. Of specific interest for other groups in 3GPP is a characterization of traffic of an XR service in the following domains:

-	Downlink data rate ranges

-	Uplink data rate ranges

-	Maximum packet delay budget in uplink and downlink

-	Maximum Packet Error Rate,

-	Maximum Round Trip Time

- 	Traffic Characteristics on IP level in uplink and downlink in terms of packet sizes, and temporal characteristics.

Such characteristics are expected to be available for at least the following applications

-	Viewport independent 6DoF Streaming

-	Viewport dependent 6DoF Streaming

- 	Simple split rendering for online cloud gaming

## -	XR conversational services

## 7.8 	Social VR

Combination of XR experiences with the real world is an essential aspect for excellent user experiences. Social VR combines and renders signals originating from different sources into a single user experience. Social VR is expected to integrate multiple XR functionalities such a 6DoF streaming with XR conversational services.

# 8	Conclusions and Proposed Next Steps

Based on the details in the report, the following is proposed:

In the short-term:

- Develop a flexible XR centric device reference architecture as well as a collection of device requirements and recommendations for XR device classes based on the considerations in clause 7.2. Device classes should include VR device for 6DoF streaming and XR online gaming (XR5G-V4), as well as AR devices (XR5G-A1, XR5G-A4 and XR5G-A5).

- Develop a framework and basic functionalities for Simple Split Rendering for Online Gaming according o the considerations in clause 7.4.

- Document typical XR traffic characteristics in TR26.925 based on the initial considerations in this report, in particular clause 7.7 and support other 3GPP groups in design systems for XR services and applications.

- Address simple extensions to MTSI to support basic XR conversational services based on the considerations in clause 7.5

In the mid-terms:

- Address simple extensions to 5G Media Streaming to support 6DoF Streaming based on considerations in clause 7.3.

- Based on the work developed in the shorter time frame, address the considerations in clause 7.8 on Social VR

3GPP should also stay in close coordination with other in 3GPP on edge computing and rendering as well communicate with experts in MPEG on the MPEG-I project as well as with Khronos on their work on OpenXR, glTF and Vulkan/OpenGL.