3GPP TSG-S4 Meeting #107	S4-200078

# Wroclaw, Poland,  20th Jan.  - 24th Jan. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Collaboration Scenarios for 5G Media Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 14/01/2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR suggests a set of collaboration scenarios, which illustrate different deployments of the 5G Media Streaming related network function. The intention is to illustrate the different capabilities of Release 16 functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Collaboration Scenarios for 5G Media Streaming are not explained and there could be confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex A (informative): Usage Guidelines for collaboration scenarios

This annex describes a set of collaboration scenarios and deployment options of the 5G Media Streaming Architecture. The intention is to illustate different deployment options.

Note, that the scenarios focus on the ownership of functions. Salaibility realizations such as a Content Delivery Network (CDN) are not illustrated. As result of the scalability considerations, the M4d serving 5GMSd AS and / or M5d serving 5GMSd AF may

-	Consist of multiple (physical) servers, which may be addressed using a single FQDN. A load balancer forwards the client requests to one of these server. Forwarding may be via HTTP redirects or transparent towards the client.

-	Consist of multiple (physical) servers, where different servers, or different groups of servers, may be addressed with different FQDNs. The client may be made aware of this via the manifest (i.e., listing multiple base URLs).

-	Be addressed with different FQDNs (MNO AS and content provider AS), where the client is made aware via the manifest.

-	Be addressed with a single FQDN. For example, the MNO AS is mostly transparent and acts as a proxy/cache.

# A.1. Collaboration 1

This collaboration scenario represents a typical OTT collaboration scenario, where the 5GMSd AF and AS are deployed in an external Data Network.

Figure A.1-1: Collaboration 1

The Interfaces M1d’ and M2d’ may be similar to 3GPP defined interfaces. The interface M4d follows 3GPP specifications.

NOTE: 	Although a single 5GMSd AS exposes the M2d’(Ingest) and M4d (Downlink Streaming) API, different physical servers (with different FQDNs) may expose the APIs. Furthermore, a large number of serving nodes, each with its own FQDN may offer the M4d service, following Content Distribution Network (CDN) scaling principles.

# A.2. Collaboration 2

This collaboration scenario represents a MNO CDN scenario, where the MNO’s Content Delivery Network is used for ingest and delivery of the content.

Figure A.2-1: Collaboration 2

# A.3. Collaboration 3

Figure A.3-1: Collaboration 3

NOTE 1: The M5d API may be exposed using a different FQDN than the M1d’ API exposing function.

NOTE 2: The M5d API may be exposured using several FQDNs, e.g. for different M5d assistance services.

# A.4. Collaboration 4

This collaboration scenario depicts a content hosting function in the external data network, e.g. using a third party CDN in collaboration with MNO offerent assistance and network services. The provisioning API (M1d’) and ingest API (M2d’) may follow 5GMSA specifications.

Figure A.4-1: Collaboration 4

# A.5. Collaboration 5

# This collaboration scenario depicts a collaboration similar to Collaboration 4 with the difference, that the external content hosting function (5GMSd AS) is provisioned from a 5GMSd AF, which is located in a trusted Data Network. It is expected that a 5GMSd AF and 5GMSd AS from different providers are interconnected using an M3d interface.

Figure A.5-1: Collaboration 5

# A.6. Collaboration 6

This collaboration scenario depicts a collaboration similar to Collaboration 4 and Collaboration 5 with the difference, that the trusted content hosting function (5GMSd AS) is provisioned from an external 5GMSd AF. It is expected that a 5GMSd AF and 5GMSd AS from different providers are interconnected using an M3d interface.

Figure A.6-1: Collaboration 6

# A.5. Collaboration 7

This collaboration scenario represents a MNO CDN scenario (like in Collaboration 2), where the MNO’s Content Delivery Network is used for ingest and delivery of the content. In addition, additional 5GMSA features are used, which require interactions with th PCF-

Figure A.7-1: Collaboration 7