3GPP TSG-S4 Meeting #107	S4-200081

# Wroclaw, Poland,  20th Jan.  - 24th Jan. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and clarifications for Dynamic Policy Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 10.1.2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This draft CR corrects the dynamic policies procedure. It introduces a Policy Template concept, which is used to store all semi-static API parameters for N5 / N33 API calls. It introduces a new Domain Model, which clarify the different API parameters and link the M1d Provisioning configuration with M5d dynamic policy parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Media Streaming (5GMS) Aware Application: Application in the UE, provided by the 5GMS Application Provider, that contains the service logic of the 5GMS application service, and interacts with other 5GMS Client and Network functions via the interfaces and APIs defined in 5GMS.

NOTE 1: Functionality of the 5GMS Aware Application is outside the scope of this specification.

NOTE 2: The 5GMS Aware Application is associated with the delivery of either a downlink or uplink related 5GMS service, and is referred to as the 5GMSd Aware Application and 5GMSu Aware Application, respectively.

5G Media Streaming Client for downlink (5GMSd Client): UE function that includes at least a 5G Media Streaming Player and a Media Session Handler for downlink streaming and that may be accessed through well-defined interfaces/APIs.

5G Media Streaming Client for uplink (5GMSu Client): Originator of 5GMSu service that may be accessed through well-defined interfaces/APIs.

5GMSu Media Streamer: UE function that enables uplink delivery of streaming media content to an Application Server function of the 5GMS Application Provider, and which interacts with both the 5GMSu Aware Application for media capture and subsequent streaming, and the Media Session Handler for media session control.

NOTE 3: The 5GMSu receives a Media Streamer Entry to initiate an uplink streaming session.

NOTE 4: The 5GMSu captures the media on the provided input devices. The 5GMUS exposes some basic controls such as capture, pause, and stop to the App.

Dynamic policy: a Dynamic PCC Rule (cf. TS 23.503[4]) for an uplink or downlink application flow during a media session.

Egest Session: a uplink media streaming session from the 5GMSu AS towards the 5GMSu Application Provider.

Ingest Session: a session to upload the media content into a 5GMSd AS

Policy Template: A collection of (semi-static) PCF / NEF API parameters, which are specific to the Application Service Provider and also the resulting PCC Rule.

Policy Template alias: an identification of the desired policy template, which is used by 5GMSd AF to select the appropriate PCF/NEF API towards 5G System, so that the PCF can compile the desired PCC Rule.

media player entry: a document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file.

media streamer entry: a pointer, e.g. in the form of a URL that defines an entry point of a media uplink streaming session.

presentation entry: a document or a pointer to a document that defines an application presentation e.g. an HTML5 document as defined in e.g. TS 26.307 [6].

5GMSd Media Player:  UE function that enables playback and rendering of a media presentation based on a Media Player Entry and exposing some basic controls such as play, pause, seek, stop to the App.

NOTE 5:	It is expected to include a Media Access Client, Media Decoders, Media rendering/presentation, and possibly also DRM Client and Metrics Collection and Reporting. The 5GMS Player's Media Access Client receives a Media Player Entry. The 5GMS player renders the media on the provided output devices, such as a display in case of video.

NOTE 6:	The 5GMSd Player is functionally similar to the combination of a TS 26.247 [7] 3GP-DASH client and a TS 26.234 [8] PSS media decoder and renderer.

service access information: set of parameters and addresses (including 5GMSd AF and 5GMSd AS addresses), which are needed to activate the reception of a streaming session.

Service and Content Discovery: Functionality and procedures provided by the 5GMS Application Provider to the 5GMS Aware Application that enables the end user to discover the available streaming service and content offerings and select a specific service or content item for access.

NOTE 7:	The Service and Content Discovery functionality and procedures are outside the scope of this specification.

Service Announcement: Procedures conducted between the 5GMS Aware Application and the 5GMS Application Provider such that the 5GMS Aware Application is able to obtain 5GMS service access information, either directly or in the form of a reference to that information.

third-party player: A Third-Party Player is a part of an application and this Player uses APIs to exercise selected 5GMS Media Functions to play-back the Media content.

NOTE 8:	Such APIs are for example defined in TS 26.307 [6] when using the Media Source Extensions for media playback. This type of Player is downloaded by or built-in to an application, or it is downloaded with the Presentation Entry (e.g. as a JavaScript library).

third-party uplink streamer: The third-party uplink streamer is a part of an application and the application uses APIs to exercise selected 5GMSu functions to capture and stream the media content.

NOTE 9:	This type of Streamer is typically implemented as downloadable software.

**** Next Change ****

### 4.2.3	Service Access Information

The Service Access Information is the set of parameters and addresses, which are needed by the 5GMSd Client to activate the reception of a downlink streaming session.

The Service Access Information may be provided together with other service announcement information using M8d. Alternatively, the 5GMSd Client fetches the Service Access Information from the 5GMSd AF.  The Service Access Information contains at least the baseline information. Additional information is added, depending on offered features.

Table 4.2.3-1: Parameters of baseline service access information

| Parameters | Description |
| --- | --- |
| Media player entry | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| URL signing information | …. |
| 5GMSd Application Service Configuration Id | Unique identification of the M1d Provisioning Session. |

When consumption reporting is activated, then the parameters from Table 4.2.3-2 are present.

Table 4.2.3-2: Parameters for consumption reporting configuration

| Parameters | Description |
| --- | --- |
| Report interval | Identify the interval when the consumption report messages are sent by the 5GMS Media Session Handler. |
| Server address | A list of 5GMSd AF addresses where the consumption reporting messages are sent by the 5GMS Media Session Handler. |
| Location reporting | Identify whether the 5GMS Media Session Handler provides the location data to the Media AF (in case of MNO or trusted third parties) |

Table 4.2.3-3: Parameters for dynamic policy invocation configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses which offer the APIs for dynamic policy invocation sent by the 5GMS Media Session Handler. (Opaque URL, following the 5GMSA URL format) |
| Policy Template aliases | A list of policy template aliases, which the 5GMSd Client is authorized to use. |

Table 4.2.3-4: Parameters for metrics configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses to which metric reports shall be sent. |
| DNN | The DNN which shall be used when sending metrics report. If not specified, the default APN shall be used. |
| Reporting interval | The sending interval between metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |
| Sample percentage | The percentage of clients that shall report metrics. If not specified, all clients shall send reports. |
| Streaming source filter | A list of URL patterns for which metrics reporting shall be done. If not specified, reporting shall be done for all sessions. |
| Metrics | A list of metrics which shall be reported. |

Table 4.2.3-5: Parameters for Network Assistance configuration

**** Next Change ****

## 5.3	Provisioning Session for Media Streaming

The present clause describes the procedure to provision the features using the 5GMS System. The 5GMS Application provider.

Figure 5.3-1: High Level Procedure for provisioning the 5GMS System for downlink streaming sessions

Steps

1.	The 5GMSd Application Provider discovers the address (URL) of the 5GMSd AF for Session Provisioning. The address maybe be the result of a manual SLA negotiation procedure or a self-on-boarding procedure.

2.	The 5GMSd Application Provider authenticates itself with the system.

3.	The 5GMSd Application Provider creates a provisioning session for a Downlink Streaming Session, providing its 5GMSd Application Provider ID as input. 5GMSd Application Provider queries the capabilities and authorized features.

4. 	The 5GMSd Application Provider provisions the unicast downlink streaming session. A set of authorized features are activated, such as QoE Reporting, Consumption Reporting, Dynamic Policy, Network Assistance and Content Hosting. When Content Hosting is offered and selected, the 5GMS Application provider selects the ingest format. 
When Dynamic Policy is offered and selected, the 5GMS Application provider selects a set of policies, which can be invoked for the unicast downlink streaming session. The UE becomes aware about the selected policies in form of a list of valid policy type indications.
When Consumption Reporting is offered and selected, the 5GMS Application provides provider input on the desired reporting interval. When the 5GMSd Application Provider delegated the service access information handling to the 5GMS System, then also location reporting is selected or de-selected. 
When QoE reporting is offered and selected, the 5GMS Application provider provides configuration input on the QoE post processing. When the 5GMSd Application Provider delegated the service access information handling to the 5GMS System, then more detailed QoE reporting is configured.

5.	When content hosting is desired, the 5GMSd AF interacts with the 5GMSd AS to allocate resources and receive the address for 5GMSd AS (M2d address). The 5GMSd AF selects the desired ingest format.

6.	The 5GMSd AF compiles the service access information. The Service Access Information contain access details and options such as 5GMSd Application Configuration Id, M5d (Media Session Handling) Addresses for Consumption Reporting, QoE Reporting, Dynamic policy, Network Assistance, etc. When content hosting is provisioned, then also M4d (Media Streaming) information such as the DASH MPD is included.

7.	The 5GMSd AF provides the results to the 5GMSd Application Provider.

- When the 5GMSd Application provider selected the full service access information, then the results are provided in form or addresses and configurations for M2d (Ingest), M5d (Media Session Handling) and M4d (Media Streaming)

- When the 5GMSd Application Provider delegated the service access information handling to the 5GMS System, then a reference to the service access information (e.g. an URL) is provided. The Media Session Handler fetches the full service access information later from the 5GMSd AF.

8.	The 5GMSd Application provider can start ingesting content, using the M2d API address. In case of progressive download or on-demand DASH Sessions, the 5GMSd Application provider uploads the content assets. In case of Live DASH streaming sessions, the content provider starts ingesting the live content.

9.	The 5GMSd Application Provider executes Service Announcement and updates the UEs.

Optional

10.	The 5GMSd Application Provider may update the provisioning session.

According Provisioning

11.	The 5GMSd AF may send event related or periodic notifications to the 5GMS Application Provider.

According to Schedule or upon request

12. 	The 5GMSd Application Provider may manually terminate the Provisioning Session (at any time). All associated resources are released. Content may be removed from the 5GMSd AS. The 5GMSd Provider may configure a schedule for Provisioning Session termination.

13.	The 5GMSd AF sends a notification upon Provisioning Session terminations.

The 5GMSd AF may request the creation or reuse of one or more network slices for the distribution of the content of the provisioned session.

If more than one network slices are provisioned for the distribution of the content of a session, the list of allowed S-NSSAIs shall be conveyed to the target UEs (e.g. through URSP or through M5d or M8d).

NOTE: The 5GMSd AS(s) serving the content are only accessible through the DNN(s) used by the network slice(s) provisioned for the distribution of that content.

**** Next Change ****

## 5.7	Establish a Unicast Media Downlink Streaming Session with 5GMSd AF interactions for dynamic policy updates

### 5.7.1	General

This clause describes the provisioning for dynamic policy invocation and the establishment of a unicast downlink streaming session using the dynamic policy API. The establishment of the unicast media downlink streaming session uses 5GMSd AF interactions, as described in Steps 4 to 8 of Clause 5.1.

The dynamic policy API allows separate handling of application flows within the same PDU Session. The Media Session Handler provides a Policy Template alias together with a Flow Description, when requesting a dynamic policy. An example Policy Template alias value is “background_data”.

The Media Session Handler provides needed information to the 5GMSd AF, so that the 5GMSd AF can combine the semi-static parameters from the Policy Template (which is associated with the Policy Template alias) with the API parameters to trigger a dynamic PCC rule update using the NEF or PCF.

The 5GMSd AF may trigger for example the AF session with required QoS procedure (See TS 23.502 [3], Clause 4.15.6.6) for updating QoS related dynamic PCC rules. For changing charging related rules, the 5GMSd AF may trigger the Change the chargeable party during the session procedure (See TS 23.502 [3], Clause 4.15.6.5).

### 5.7.2	Provisioning

The provisioning for the dynamic policy procedure follows generally the procedure from Clause 5.3. Specifically, the Dynamic Policy feature is activated. As result, the 5GMSd Application Provider provisions one or more Policy Templates.

The domain model of M1d and M5d APIs is depicted in Figure 5.7.2-1. Realization of the dependencies between M1d and M5d data entries are up to implementation. Note that multiple M5d 5GMSd AF or M4d 5GMSd AS nodes may reference to the same M1d resource.

Figure 5.7.2-1: Domain Model for M1d and M5d resources

A Policy Template is identified by a Policy Template alias and contains semi-static parameters, including the API entry for the PCF / NEF interactions. The list of provisioned Policy Template aliases is communicated as valid Policy Template aliases to the Media Session Handler. The Media Session Handler uses one of the valid Policy Template aliases when invoking a dynamic policy.

### 5.7.3	Progressive Download of On-Demand Content

This procedure describes the establishment of a unicast media downlink streaming session with 5GMSd AF interactions to dynamic policy updates. A streaming session may use 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.7-1: High Level Procedure for progressive download for on-demand media

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMSd Application Provider

- 	The service announcement information contains detailed information for the policy related interactions. Specifically, the information includes URLs for the 5GMSd AF, an identifier of the 5GMS Application Service Configuration and a list of authorized Policy Type indications for that specific application.

-	The 5GMSd Application Provider has agreed an SLA with the operator defining the possible QoS levels and their charging rates (See TS 23.503, Clause 6.1.3.22). The PCF is configured with the corresponding QoS parameters and charging information.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Content Discovery procedure. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information. The configuration parameters are listed in Table 5.7.4-1

The Service and Content Discovery procedure only involves the App and the external Application Server and hence is out of scope of the present document.

2: A Media Player Entry is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information (See Step 1), the Media Session Handler interacts with the 5GMSd AF to acquire the whole service access information

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The Media Player establishes the transport session, e.g. the TCP connection.

7: The MediaPlayer notifies the Media Session Handler about the Flow description(s) (in [3]), e.g. the 5-Tuple.

8:	The Media Session Handler requests to apply a dynamic policy to the media session. The request includes at least the 5GMS Application Service Configuration Id, the Flow description(s) and the Policy Template alias (see Table 5.7.4-1), which should be applied to the described transport session. The 5GMSd AF uses the Policy Template for the related procedure and to identify the related network function.

9: This step applies when the 5GMSd AF resides in the trusted data network. Depending on the Policy Template, the step is executed:

- When the Policy Template relates to QoS, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue with step 10.

- When the Policy Template relates to a different charging scheme, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue at step 10.

10: This step applies when the 5GMSd AF resides in the external data network. Depending on the Policy Template, the step is executed:

- When the Policy Template relates to QoS, the 5GMSd AF may use the Nnef_AFsessionWithQoS service as defined in TS 23.502, Clause 5.2.6.9. The complete call flow is described in TS 23.502, Clause 4.15.6.6.

- When the Policy Template relates to a different charging scheme, the 5GMSd AF may use the Nnef_ChargeableParty service as defined in TS 23.502, Clause 5.2.6.8. The complete call flow is described in TS 23.502, Clause 4.15.6.4 and Clause 4.15.6.5. The Policy Template may contain the AF Identifier, Sponsor Information (values, based on SLA negotiation), Background Data Transfer Reference ID. The Flow Description is provided by the Media Session Handler at API invocation.

11: The Media Player sends the request for the progressive download content.

12:	The Media Session Handler queries the status of the dynamic policy invocation. The response contains status information (policy accepted, rejected, etc) and information on policy enforcement.

13: The Media Session Handler notifies the Media Player according to the response from the 5GMSd AF (See Table 5.6.4-2).

14: The Media Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

15: The Media Player configures the rendering pipeline for media playback.

16: The Media Player notifies the media session handler, providing the transport session information and some media content related information.

17: Optional: Media Player acquires a DRM License from an External AS.

18: The Media Player receives media content and puts it into the rendering pipeline.

19: The Media Player continuously receives and plays back the media content.

### 5.7.4	DASH Streaming

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.7-2: High Level Procedure for DASH content

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMSd Application Provider.

-	The service announcement information contains detailed information for the policy related interactions. Specifically, the information includes URLs for the 5GMSd AF, an identifier of the 5GMSd Application Provider Service Configuration  and a list of authorized Policy Type indications for that specific application.

-	The 5GMSd Application Provider has agreed an SLA with the operator defining the possible QoS levels and their charging rates (See TS 23.503, Clause 6.1.3.22). The PCF is configured with the corresponding QoS parameters and charging information.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Content Discovery procedure. The Content Discovery procedure only involves the App 5GMSd aware Application and the external Application 5GMSd Application ProviderServer. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information. The configuration parameters are listed in Table 5.7.4-1.

2: A media content item is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information (see Step 1), the Media Session Handler interacts with the 5GMSd AF to acquire the whole service access information

5: The Media Session Handler triggers the MediaPlayer to start the session.

6: The Media Player establishes the transport session for acquiring the MPD referenced by the Media Player Entry.

7: The MediaPlayer requests the MPD

8: The media access client receives the MPD

9: The MediaPlayer processes the MPD. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the MPD information to initialize the media pipelines for each media stream. The MPD should also contain information to start the DRM client initialization, when DRM is used.

10: The MediaPlayer notifies the media session handler about the MPD. The notification may contain parameters from the MPD

11: Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

12: The Media Player configures the media rendering pipeline.

13: The Media Player establishes the necessary transport sessions for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

14: The media session handler is notified about the established transport sessions for the content by providing the description of the application flows e.g. 5-Tuples.

15:	The Media Session Handler requests to apply a dynamic policy to the media session. The request includes at least the 5GMS Application Provider Service Configuration Id, the Flow description(s) and the Policy Template alias (see Table 5.7.4-1), which should be applied to the described transport session. The 5GMSd AF uses the Policy Template for the related procedure and to identify the related network function.

16: This step applies when the 5GMSd AF resides in the trusted data network. Depending on the Policy Template, the step is executed:

- When the Policy Template relates to QoS, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue at step 17.

- When the Policy Template relates to a different charging scheme, the 5GMSd AF may either directly interact with the PCF or may use a NEF service

- when directly interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization Service as defined in TS 23.502, Clause 5.2.5.3).

- when interacting via the NEF with the PCF, continue at step 17.

17: This step applies when the 5GMSd AF resides in the external data network. Depending on the Policy Template, the step is executed:

- When the Policy Template relates to QoS, the 5GMSd AF may use the Nnef_AFsessionWithQoS service as defined in TS 23.502, Clause 5.2.6.9. The complete call flow is described in TS 23.502, Clause 4.15.6.6.

- When the Policy Template relates to a different charging scheme, the 5GMSd AF may use the Nnef_ChargeableParty service as defined in TS 23.502, Clause 5.2.6.8. The complete call flow is described in TS 23.502, Clause 4.15.6.4 and Clause 4.15.6.5. The Policy Template may contain the AF Identifier, Sponsor Information (values, based on SLA negotiation), Background Data Transfer Reference ID. The Flow Description is provided by the Media Session Handler at API invocation.

18: The Media Session Handler queries the status of the dynamic policy invocation. The response contains status information (policy accepted, rejected, etc) and information on policy enforcement.

19: The Media Session Handler updates the configuration of the Media Player according to the response from the 5GMSd AF (See Table 5.6.4-2).

20: The Media Player requests initialization information. The 5GMSP repeats this step for each required initialization segment.

21: The Media Player receives the initialization information.

22: The Media Player requests media segments according to the MPD.

23: the Media Player receives media segments and puts the information into the according media rendering pipeline.

24: Previous steps are repeated according to the MPD information.

### 5.7.5	Parameters for dynamic policy invocation configuration

Table 5.7.5-1: Status information dynamic policy

| Parameters | Description |
| --- | --- |
| Status | Status of a dynamic policy. A dynamic policy have the status granted or rejected. The status of the policy may change while the streaming session is active. |
| Policy Enforcement | Information around the policy enforcement system. |

**** Next Change ****