## Source:	Fraunhofer IIS

## Title:	IVAS Stereo Speech Item Generation

## Document for:	Discussion & Agreement

## Agenda Item:	7.5

# 1	Introduction

While there are already proposals available for almost all aspects relevant for the Stereo operation of IVAS, the processing and content types still seem vague. Relevant contributions are e.g. [1] and [2], where the content types and stereo microphony are discussed. While [1] suggests using filtering of speech using IRs, the source has found that this would not suffice to test the real-world performance of a stereo codec. Thus, the source developed a recording procedure using a setup with 8 microphones to generate stereophonic items for a single talker. Due to the high number of microphones and potential combinations, different stereophonic items in A-B and M/S configuration can be generated. The single talker items can then be mixed with other single talker items, similarly as described in [1], to have subsequent or overlapping talkers in the final stimuli.

# 2	Recording Procedure

Clean Speech stereo items can be recorded by picking from a number of microphone configurations as e.g. outlined in [2]. The configuration in today's UEs, using a pair of the omni-directional MEMS microphones, is an A-B configuration, with rather small microphone distances. Other multi-microphone configurations are what could be used in meeting rooms, with more distant microphones. Note that most conferencing phones with and without satellites only capture in mono (sometimes attempting to isolate the currently active talker), even though many microphones would be available.

The procedure outlined here was developed to have an efficient method for getting recorded stereo speech items in multiple microphone configurations and from different angles. Another requirement has been high-quality recordings with low background noise levels and low reverberation, which can be accommodated by using high-quality microphones and conduct the recording in an acoustically treated room. This is crucial, as there is significant distance of the talkers to the microphones, and if one wants to create a high-quality clean speech signal after the fact of lazy recording, noise reduction and dereverberation tend to introduce artefacts that are often similar in nature to coding artefacts, making listening tests very challenging for naïve subjects if already the DIRECT signal is deteriorated.

The resulting setup is as follows:

- Recording in an acoustically treated room / studio using low-noise recording equipment

- 7 omni-directional low-noise boundary microphones on a table (>2m length) at -1m, -15cm, -5cm, 0cm, 5cm, 15cm, 1m.

- 1 figure of eight microphone, just above the center microphone

- Talkers seated in 1.5m distance from the center microphone, different positions on an arc at 0, 15, 30, 45, 60, 75, 90deg, reading from a sentence list (e.g. the phonetically balanced Harvard sentences) with sufficient gaps between sentences

Figure 1 illustrates the recording setup.

Figure 1: Top View of the recording setup with seven omni- and one figure-eight microphone. Talkers are seated on an arc, facing to microphone "1"

The recording process gives eight tracks of audio for each recorded sentence, that can be used to derive A-B microphony setups.  The following configurations are recommended to generate A-B setups for 5cm, 10cm, 15cm, 20cm, 30cm, 100cm, 200cm microphone distance. Microphone 2 (figure eight) would be used together with the center microphone to generate M/S (with an appropriate gain of Mic2, see example command lines below).

Table 1: Microphone selection for A-B configurations

| d (cm) | Mic1 | Mic2 |
| --- | --- | --- |
| 5 | 3 | 1 |
| 10 | 3 | 4 |
| 15 | 5 | 1 |
| 20 | 5 | 4 |
| 30 | 5 | 6 |
| 100 | 7 | 1 |
| 200 | 7 | 8 |

Those configurations can e.g. be generated using the "sox" tool:

sox remix 8ch.wav ms.wav "1,2v+3 1,2i+3"

sox remix 8ch.wav d5.wav "3 1"

sox remix 8ch.wav d10.wav "3 4"

sox remix 8ch.wav d15.wav "5 1"

sox remix 8ch.wav d20.wav "5 4"

sox remix 8ch.wav d30.wav "5 6"

sox remix 8ch.wav d100.wav "7 1"

sox remix 8ch.wav d200.wav "7 8"

NOTE: 	The setup covers some A-B microphone distances of relevance; some distances are not centered around microphone 1, in case centering around microphone 1 was not possible, the more distant microphones are used. The distances are the result of a compromise to fit within eight recording channels for which hardware is widely available.

As a result, single talker stereophonic items are generated, all panned to one side only (e.g. all talkers would come from the center or left). To get items from the other side, left and right channel are simply swapped. Depending on item availability, different spreads, e.g. 90 degrees or 60 degrees could be defined then.

# 3	Item Selection

Similarly, as in other exercises, good coverage of talkers, sentences, plus now microphone configurations and spreads should be achieved. The following setups are proposed to in the end have 24 different stimuli to exercise candidates, with four sentences per category. A sentence is defined by the talkers and their respective (ideally unique) sentence. Each category is defined by the microphone configuration and the two angles for the talkers (which result in a certain spread).

Table 2: Example sentence configuration

| Sentence | Talker 1 | Talker 2 |
| --- | --- | --- |
| s1 | f2 | m2 |
| s2 | f1 | m1 |
| s3 | m2 | f1 |
| s4 | m1 | f2 |

Table 3: Example category configuration

|  | d | Mic1 | Mic2 | Ang1 | Ang2 |
| --- | --- | --- | --- | --- | --- |
| a1 | 5 | 3 | 1 | 60 | -90 |
| a2 | 10 | 3 | 4 | 90 | -30 |
| a3 | 30 | 5 | 6 | 45 | -45 |
| a4 | 100 | 7 | 1 | 45 | -75 |
| a5 | 200 | 7 | 8 | 75 | -75 |
| a6 | (M/S) | 2 | 1 | 15 | -15 |

# 4	Item Mixing

One male single talker item and one female single talker is always mixed, which results in stereo items consisting of two sentences by two different talkers from different gender. A preamble and postamble of 0.5s is used. The 1st talker has a preamble of 0.5s, followed by the spoken sentence and afterwards a sufficiently long pause to cover the extent of the final mixed item. The second talker has a postamble of 0.5s, preceded by the spoken sentence and a prior preamble to also cover the extent of the final mixed item. The preamble and postamble should be the recording noise when the talkers pause and not digital silence to avoid unnatural noise onsets in the mixed items.

Depending on the desired mixed item length (extent) the two sentences could be overlapping or be subsequent. A typical spoken sentence for P.800 style items is ~2-3s in length. Thus, if the mixed item length is set to 4.5s, a short overlap of ~0.5s is typically achieved (this may depend on the used corpus though). This is depicted in Figure 2.

Figure 2: Mixing two single sentences with short overlap

If the mixed item length is set to a larger value (6s in the depicted case in Figure 3), the sentences no longer overlap.

Figure 3: Mixing two single sentences without overlap

# 4	Summary

The source presented a recording procedure for generating recorded stereophonic clean speech stimuli. Whilst other procedures could be developed, the presented one has already been used by the source with success and it was found it is rather flexible and the effort is reasonable. To have a common procedure of item generation, the source decided to present it to SA4, to allow other interested parties to generate their own stimuli.

The resulting items are suitable for P.800 style listening tests. A similar recording procedure could be applied for background noise recordings, which is not addressed by this contribution yet but planned for a future update.

The source also expressed previously, that P.800 DCR should be the listening test methodology of choice, where possible.

Thus, in summary the source proposes to agree on the method above for P.800 style clean speech recordings, and to agree on P.800 DCR as the listening method.

References:

[1]	3GPP AHEVS-509, IVAS processing functions for stereo and binaural audio, Orange

[2]	3GPP S4-190961, Content Types for IVAS Testing, Fraunhofer IIS

[3]	3GPP S4-191307, "IVAS Design Constraints (IVAS-4)", v0.2.0