3GPP TSG-SA4 Meeting #107 	S4-200230

# Wroclaw, Poland, 20-24 January 2020														rev of S4-200148

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.346 | CR | <CR#> | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of HLS in the SA profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ENENSYS, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DAHOE |  |  |  |  |  | Date: |  |  | 2019-12-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As mentioned in the WID : To allow the delivery over MBMS of HLS services and hybrid HLS/DASH services, should be specified: - how to announce a hybrid HLS/DASH service, as the maximum cardinality of the release 12 appService element is at most one, while two would be required; - how to announce and deliver the HLS manifests (master and media playlists) in MBMS Application Services; |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ The USBD schema is extended to allow more occurrences of r12:appService. 2/ Profiles 1a and 1b and extended to support HLS and hybrid DASH/HLS services. 3/ Usage of r12:appService is added in the profile. It is optional for DASH service (if an unified MPD is used), and usage of r9:MediaPresentationDescription remains compulsory, for backward compatibility. 4/ usage of r12:broadcastAppService.basePattern is made possible, to indicate the ressources available over MBMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 5.2.2.9, J.1, L.2.1, L.2.3, L.2.4, L.2.5, L.3, M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE:

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service; Stage 1".

[3]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[5]	3GPP TS 25.346: "Introduction of Multimedia Broadcast/Multicast Service (MBMS) in the Radio Access Network (RAN); Stage 2".

[6]	IETF STD 0064/RFC 3550 (July 2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick, V. Jacobson.

[7]	IETF STD 0006/RFC 0768 (August 1980): "User Datagram Protocol", J. Postel.

[8]	IETF STD 0005/RFC 0791 (September 1981): "Internet Protocol", J. Postel.

[9]	IETF RFC 3926 (October 2004): "FLUTE - File Delivery over Unidirectional Transport", T. Paila, M. Luby, R. Lehtonen, V. Roca, R. Walsh.

[10]	IETF RFC 3450 (December 2002): "Asynchronous Layered Coding (ALC) Protocol Instantiation", M. Luby, J. Gemmell, L. Vicisano, L. Rizzo, J. Crowcroft.

[11]	IETF RFC 3451 (December 2002): "Layered Coding Transport (LCT) Building Block", M. Luby, J. Gemmell, L. Vicisano, L. Rizzo, M. Handley, J. Crowcroft.

[12]	IETF RFC 5052 (August 2007): "Forward Error Correction (FEC) Building Block", M. Luby, M. Watson, L. Vicisano.

[13]	IETF RFC 3695 (February 2004): "Compact Forward Error Correction (FEC) Schemes", M. Luby, L. Vicisano.

[14]	IETF RFC 4566 (July 2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[15]	IETF RFC 4570 (July 2006): "Session Description Protocol (SDP) Source Filters", B. Quinn, R. Finlayson.

[16]	Void.

[17]	IETF RFC 3048 (January 2001): "Reliable Multicast Transport Building Blocks for One-to-Many Bulk-Data Transfer", B. Whetten, L. Vicisano, R. Kermode, M. Handley, S. Floyd, M. Luby.

[18]	IETF RFC 2616 (June 1999): "Hypertext Transfer Protocol -- HTTP/1.1".

[19]	IETF STD 0066/RFC 3986 (January 2005): "Uniform Resource Identifier (URI)".

[20]	3GPP TS 33.246: "3G Security; Security of Multimedia Broadcast/Multicast Service (MBMS)".

[21]	OMG: "Unified Modeling Language (UML), version 1.5" (formal/03-03-01).

[22]	W3C Recommendation 28 October 2004: "XML Schema Part 2: Datatypes Second Edition".

[23]	IETF RFC 5234 (January 2008): "Augmented BNF for Syntax Specifications: ABNF", D. Crocker and P. Overell.

[24]	3GPP TS 26.290: "Audio codec processing functions; Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec; Transcoding functions".

[25]	3GPP TS 26.304: "Floating-point ANSI-C code for the Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec".

[26]	3GPP TS 26.273: "Speech codec speech processing functions; Extended Adaptive Multi-Rate - Wideband (AMR-WB+) speech codec; Fixed-point ANSI-C code".

[27]	Void.

[28]	3GPP TS 26.401: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description".

[29]	3GPP TS 26.410: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Floating-point ANSI-C code".

[30]	3GPP TS 26.411: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Fixed-point ANSI-C code".

[31]	W3C Recommendation 04 February 2004: "Extensible Markup Language (XML) 1.1", T. Bray, J. Paoli, C. Sperberg-McQueen, E. Maler, F. Yergeau and J. Cowan.

[32]	3GPP TS 26.244: "Transparent end-to-end streaming service; 3GPP file format (3GP)".

[33]	IETF RFC 4867 (April 2007): "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs", J. Sjoberg, M. Westerlund, A. Lakaniemi, Q. Xie.

[34]	IETF RFC 4352 (January 2006): "RTP Payload Format for the Extended Adaptive Multi-Rate Wideband (AMR-WB+) Audio Codec", Sjoberg J. et al.

[35]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[36]	Void.

[37]	IETF RFC 2557 (March 1999): "MIME Encapsulation of Aggregate Documents, such as HTML (MHTML)", J. Palme, A. Hopmann, N. Shelness.

[38]	IETF RFC 3890 (September 2004): "A Transport Independent Bandwidth Modifier for the Session Description Protocol (SDP)", M. Westerlund.

[39]	IETF RFC 3556 (July 2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth", S. Casner.

[40]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[41]	IETF RFC 3640 (November 2003): "RTP Payload Format for Transport of MPEG-4 Elementary Streams", J. van der Meer, D. Mackie, V. Swaminathan, D. Singer, P. Gentric.

[42]	IETF RFC 1952 (May 1996): "GZIP file format specification version 4.3", P. Deutsch.

[43]	ITU-T Recommendation H.264 (04/2013): "Advanced video coding for generic audiovisual services".

[44]	Void.

[45]	ITU-T Recommendation H.263 (1998): "Video coding for low bit rate communication".

[46]	ITU-T Recommendation H.263 - Annex X (04/01): "Annex X: Profiles and levels definition".

[47]	3GPP TS 26.234: "Transparent end-to-end streaming service; Protocols and codecs".

[48]	3GPP TS 26.071: "AMR speech codec; General description".

[49]	3GPP TS 26.090: "AMR speech codec; Transcoding functions".

[50]	3GPP TS 26.073: "AMR speech Codec; C-source code".

[51]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi-Rate (AMR) speech codec".

[52]	3GPP TS 26.171: "AMR speech codec, wideband; General description".

[53]	3GPP TS 26.190: "Mandatory Speech Codec speech processing functions AMR Wideband speech codec; Transcoding functions".

[54]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[55]	3GPP TS 26.204: "ANSI-C code for the floating-point Adaptive Multi-Rate Wideband (AMR-WB) speech codec".

[56]	Scalable Polyphony MIDI Specification Version 1.0, RP-34, MIDI Manufacturers Association, Los Angeles, CA, February 2002.

[57]	Scalable Polyphony MIDI Device 5-to-24 Note Profile for 3GPP Version 1.0, RP-35, MIDI Manufacturers Association, Los Angeles, CA, February 2002.

[58]	"Standard MIDI Files 1.0", RP-001, in "The Complete MIDI 1.0 Detailed Specification, Document Version 96.1", The MIDI Manufacturers Association, Los Angeles, CA, USA, February 1996.

[59]	Mobile DLS, MMA specification v1.0, RP-41 Los Angeles, CA, USA. 2004.

[60]	Mobile XMF Content Format Specification, MMA specification v1.0, RP-42, Los Angeles, CA, USA. 2004.

[61]	ITU-T Recommendation T.81 (1992) | ISO/IEC 10918-1:1993: "Information technology - Digital compression and coding of continuous-tone still images - Requirements and guidelines".

[62]	C-Cube Microsystems (September 1992): "JPEG File Interchange Format", Version 1.02.

[63]	CompuServe Incorporated (1987): "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA. See at http://www.dcs.ed.ac.uk/home/mxr/gfx/2d/GIF87a.txt.

[64]	CompuServe Incorporated (1990): "Graphics Interchange Format: Version 89a", Columbus, OH, USA.

[65]	IETF RFC 2083 (March 1997): "PNG (Portable Networks Graphics) Specification Version 1.0", T. Boutell.

[66]	W3C Working Draft 27 October 2004: "Scalable Vector Graphics (SVG) 1.2", http://www.w3.org/TR/2004/WD-SVG12-20041027/.

[67]	W3C Working Draft 13 August 2004: "Mobile SVG Profile: SVG Tiny, Version 1.2", http://www.w3.org/TR/2004/WD-SVGMobile12-20040813/.

[68]	Standard ECMA-327 (June 2001): "ECMAScript 3rd Edition Compact Profile".

[69]	Void.

[70]	ISO/IEC 10646-1 (2000): "Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane".

[71]	The Unicode Consortium: "The Unicode Standard", Version 3.0 Reading, MA, Addison-Wesley Developers Press, 2000, ISBN 0-201-61633-5.

[72]	3GPP TS 26.245: "Transparent end-to-end Packet switched Streaming Service (PSS); Timed text format".

[73]	IETF RFC 4646: "Tags for the Identification of Languages".

[74]	ISO 639: "Codes for the representation of names of languages".

[75]	ISO 3166: "Codes for the representation of names of countries and their subdivisions".

[76]	Void.

[77]	3GPP TS 23.003: "Numbering, addressing and identification".

[78]	IETF RFC 5905, "Network Time Protocol Version 4: Protocol and Algorithms Specification", D. Mills, J. Martin, J. Burbank and W. Kasch, June 2010.

[79]	OMA Push OTA Protocol (25-April-2001): WAP-235-PushOTA-20010425-a

http://www.openmobilealliance.org/tech/affiliates/LicenseAgreement.asp?DocName=/wap/wap-235-pushota-20010425-a.pdf

[80]	IETF RFC 3711 (March 2004): "The Secure Real-time Transport Protocol (SRTP)", M. Baugher, D. McGrew, M. Naslund, E. Carrara, K. Norrman.

[81]	Void.

[82]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings", Josefsson S., Ed., October 2006.

[83]	IETF RFC 3023: "XML Media Types", M. Murata, S. St.Laurent, D. Kohn, January 2001.

[84]	IETF RFC 5905: "Network Time Protocol Version 4: Protocol and Algorithms Specification", D. Mills, June 2010.

[85]	OMA OMNA Registered PUSH Application ID list

http://www.openmobilealliance.org/tech/omna/omna-push-app-id.htm

[86]	3GPP TR 26.936: "Performance characterization of 3GPP audio codecs".

[87]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[88]	IETF RFC 2326: "Real Time Streaming Protocol (RTSP)", Schulzrinne H., Rao A. and Lanphier R., April 1998.

[89]	Void.

[90]	"Service Guide for Mobile Broadcast Services", Open Mobile Alliance, OMA-TS-BCAST_ServiceGuide-V1_1, Candidate Version 1.1 –14 Sep 2010.

[91]	IETF RFC 5053 (October 2007): "Raptor Forward Error Correction Scheme for Object Delivery", M. Luby, A. Shokrollahi, M. Watson, T. Stockhammer.

[92]	IETF RFC 5285: "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni, July 2008.

[93]	IETF RFC 4396: "RTP Payload Format for 3rd Generation Partnership Project (3GPP) Timed Text", Rey J. and Matsui Y., February 2006.

[94]	OMA-ERELD-DM-V1_2-20070209-A: "Enabler Release Definition for OMA Device Management, Approved Version 1.2".

[95]	3GPP TS 26.430: "Timed Graphics".

[96]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[97]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[98]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[99]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[100]	IETF RFC 6064: "SDP and RTSP Extensions Defined for 3GPP Packet-Switched Streaming Service and Multimedia Broadcast/Multicast Service", M. Westerlund, P. Frojdh, January 2011.

[101]	Void.

[102]	Void.

[103]	Void.

[104]	3GPP TS 36.443: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M2 Application Protocol (M2AP)".

[105]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[106]	IETF RFC6681, "Raptor FEC Schemes for FECFRAME," M. Watson, T. Stockhammer and M. Luby, August 2012.

[107]	IETF RFC6363, "Forward Error Correction (FEC) Framework," M. Watson, A. Begen and V. Roca, October 2011.

[108]	3GPP TS 36.304: "User Equipment (UE) procedures in idle mode".

[109]	"Mobile Location Protocol (MLP) ", Open Mobile Alliance, OMA-LIF-MLP-V3_1, Approved Version 3.1 – 20 Sep 2011.

[110]	3GPP TR 26.946: "Multimedia Broadcast/Multicast Service (MBMS); User service guidelines".

[111]	Void.

[112]	ITU-T Recommendation H.265 (04/2013): "High efficiency video coding".

[113]	IETF RFC 7798 (2016): "RTP Payload Format for High Efficiency Video Coding (HEVC)", Y.-K. Wang, Y. Sanchez, T. Schierl, S. Wenger, M. M. Hannuksela.

[114]	IETF RFC 1035 (November 1987): "Domain Names – Implementation and Specification", P. Mockapetris.

[115]	3GPP TR 26.906 Evaluation of HEVC for 3GPP Services (Release 12).

[116]	ISO/IEC 23009-1: 2019/Amd.1:2020: Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats.

[117]	3GPP TS 29.214 "Policy and charging control over Rx reference point".

[118]	3GPP TS 23.271 "Functional stage 2 description of Location Services (LCS)".

[119]	IETF RFC 5651: "Layered Coding Transport (LCT) Building Block", M. Luby, M. Watson, L. Vicisano.

[120]	3GPP TS 23.468, "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[121]	3GPP TS 29.468, "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[122]	3GPP TS 23.179. "Functional architecture and information flows for mission critical communication, services; Stage 2".

[123]	3GPP TS 24.379, "Mission Critical Push to Talk (MCPTT) call control Protocol specification".

[124]	3GPP TS 26.307, "Presentation Layer for 3GPP Services".

[125]	3GPP TS 26.116, "Television (TV) over 3GPP Services; Video Profiles".

[126]	Standard ECMA-262, 5.1 Edition / June 2011, ECMAScript Language Specification.

[127]	IETF RFC 6347 (January 2012): "Datagram Transport Layer Security Version 1.2".

[128]	3GPP TS 33.310, Network Domain Security (NDS); Authentication Framework (AF)

[129]	3GPP TS 29.116, "Representational state transfer over xMB reference point between Content Provider and BM-SC".

[130]	IETF Internet-Draft, "JSON Schema: A Media Type for Describing JSON Documents", draft-wright-json-schema-01, April 15, 2017.

[131]	3GPP TS 24.116, "Stage 3 aspects of system architecture enhancements for TV services".

[132]	3GPP TS 24.117, "TV service configuration Management Object (MO)".

[133]	IANA Multicast IPv4 Address Space Registry, https://www.iana.org/assignments/multicast-addresses/multicast-addresses.xhtml.

[134]	IANA Multicast IPv6 Address Space Registry, https://www.iana.org/assignments/ipv6-multicast-addresses/ipv6-multicast-addresses.xhtml.

[135]	IANA Service Name and Transport Protocol Port Number Registry, https://www.iana.org/assignments/service-names-port-numbers/service-names-port-numbers.xhtml.

[136]	3GPP TS 26.347, "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

[137]	IETF RFC 5795, "The Robust Header Compression (ROHC) Framework".

[138]	IETF RFC 3095, "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

[139]	IETF RFC 5225, RObust Header Compression Version 2 (ROHCv2): Profiles for RTP, UDP, IP, ESP and UDP-Lite

[140]	3GPP TS 26.118, "3GPP Virtual reality profiles for streaming applications".

[141]	3GPP TS 23.180, "Common functional architecture to support mission critical services; Stage 2".

[142]	3GPP TS 23.379, "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[143]	3GPP TS 26.348, "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[X]	IETF RFC 8216: "HTTP Live Streaming"

NEXT CHANGE:

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply:

Application content component: An individual content component of an MBMS User Service, such as a video or audio media stream, or a non-real-time file, delivered by an MBMS transport session, and is available to and can be used by the MBMS application.

Application Service: An end-user service for which the entry point document is contained in the User Service Description and whereby all associated content components are delivered through one or more MBMS User Services via broadcast and/or unicast bearers.

Broadcast session: See 3GPP TS 22.146 [2].

Forward Error Correction (FEC): in the context of MBMS, a FEC mechanism is used at the application layer to allow MBMS receivers to recover lost SDUs.

FLUTE channel: equivalent to an ALC/LCT channel.
An ALC/LCT channel is defined by the combination of a sender and an address associated with the channel by the sender (RFC 3926 [9]).

Frame-packed stereoscopic 3D video: a video consisting of two views in which both views were packed into a single stream before compression.

Hybrid DASH/HLS service: a media streaming service which can be consumed both by DASH clients and HLS clients.

Multicast joining: See 3GPP TS 22.146 [2].

Multicast session: See 3GPP TS 22.146 [2].

Multimedia Broadcast/Multicast Service (MBMS): See 3GPP TS 22.146 [2].

MBMS application: The application which resides in or interfaces with the UE, and which provides to the end-user an application service by using one or more application content components of one or more MBMS User Services.

MBMS over a Single Frequency Network: See 3GPP TS 25.346 [5].

MBMS User Service:  The transport-level service, comprising one or more application content components, delivered by the MBMS bearer service across one or more Broadcast and/or Multicast sessions, and/or delivered by the unicast bearer service. The application content components of the MBMS User Service, in part or in whole, are provided to the MBMS application.

See 3GPP TS 22.246 [3].

MBMS user service discovery/announcement: user service discovery refers to methods for the UE to obtain the list of available MBMS user services along with information on the user service and the user service announcement refers to methods for the MBMS service provider to make the list of available MBMS user services along with information on the user service available to the UE.

MBMS user service initiation: UE mechanisms to setup the reception of MBMS user service data
The initiation procedure takes place after the discovery of the MBMS user service.

MBMS delivery method: mechanism used by a MBMS user service to deliver content.
An MBMS delivery method uses MBMS bearers in delivering content and may make use of associated procedures.

MBMS download delivery method: delivery of discrete objects (e.g. files) by means of a MBMS download session.

MBMS group communication delivery method: delivery of group communication service data by means of the BM-SC ingesting UDP/IP packets from a group communications server and forwarding them over the MBMS path provided by the MBMS Bearer Service.

MBMS streaming delivery method: delivery of continuous media (e.g. real-time video) by means of a MBMS streaming session.

MBMS Transparent delivery method: transparent delivery of application data by means of an MBMS Transparent session.

MBMS download session: time, protocols and protocol state (i.e. parameters) which define sender and receiver configuration for the download of content files.

MBMS streaming session: time, protocols and protocol state (i.e. parameters) which define sender and receiver configuration for the streaming of content.

MBMS Transparent session: time, area, protocols, and protocol state (i.e. parameters) which define sender and receiver configuration for the transparent delivery of application data flows.

Partial-File-Accept Request: HTTP GET request that includes a partial-accept header in the HTTP request header.

Receive Only Mode: See 3GPP TS 23.246 [4].

Receive-Only-Mode (ROM) service: A type of eMBMS broadcast service receivable by a UE configured in Receive Only Mode. A TMGI associated with a ROM service shall contains a standardized [MCC+MNC] value. See Annex E of 3GPP TS 23.246 [4] and TS 24.116 [131] for further details on Receive Only Mode.

RTP Session: The RTP and RTCP traffic sent to a specific IP multicast address and port pair (one port each for RTP and RTCP) during the time period the session is specified to exist. An RTP session is used to transport a single media type (e.g. audio, video, or text). An RTP session may contain several different streams of RTP packets using different SSRCs.

Stereoscopic 3D video: a video bitstream consisting of two views.

Unicast Bearer Service: Synonymously used as the term "UMTS Bearer Services with interactive and/or streaming traffic classes".

NEXT CHANGE:

## 3.2	Abbreviations

For the purposes of the present document, abbreviations given in 3GPP TR 21.905 [1] and the following apply:

ADPD	Associated Delivery Procedure Description

ADU	Application Data Unit

ALC	Asynchronous Layered Coding

API	Application Programming Interface

AS	Application Server

ASD	Application Service Description

AVC	Advanced Video Coding

BM-SC	Broadcast-Multicast - Service Centre

CC	Congestion Control

CPB	Coding Picture Buffer

CVS	Coded Video Sequence

DASH	Dynamic Adaptive Streaming over HTTP

ERT	Expected Residual Time

ESI	Encoding Symbol ID

FDT	File Delivery Table

FLUTE	File deLivery over Unidirectional Transport

FMT	Feedback Message Type

GCS	Group Communication Service

GCSE	Group Communication Service Enabler

HDR	High Dynamic Range

HDTV	High-DefinitionTeleVision

HEVC	High Efficiency Video Coding

HLS	HTTP Live Streaming

IANA	Internet Assigned Numbers Authority

IDR	Instantaneous Decoding Refresh

IRAP	Intra Random Access Point

ISD	Initialization Segment Description

LCT	Layered Coding Transport

LI	Leap Indicator

MBMS SAI	MBMS Service Area Identity

MCPTT	Mission Critical Push to Talk

MIME	Multipurpose Internet Mail Extensions

MMS	Multimedia Messaging Service

MooD	MBMS operation on Demand

MPD	Media Presentation Description

MSK	MBMS Service Key

MTK	MBMS Traffic Key

MUK	MBMS User Key

NAL	Network Abstraction Layer

NTP	Network Time Protocol

OMNA	Open Mobile Naming Authority

PAC	Proxy Auto-Config

PSS	Packet-switched Streaming Service

PTT	Push To Talk

RASL	Random Access Skipped Leading picture

ROM	Receive-Only-Mode

SA File	Service Announcement File

SACH	Service Announcement CHannel

SBN	Source Block Number

SCT	Sender Current Time

SEI	Supplemental Enhancement Information

SNTP	Simple Network Time Protocol

TMGI	Temporary Mobile Group Identity

TOI	Transport Object Identifier

TSI	Transport Session Identifier

UHD	Ultra High Definition

URL	Uniform Resource Locator

USBD	User Service Bundle Description

USD	User Service Description

UTC	Universal Time Coordinated

VCL	Video Coding Layer

VR	Virtual Reality

NEXT CHANGE:

#### 5.2.2.9	Application Service Description

An MBMS User Service Description may contain one or more Application Service Description fragment. If present, an Application Service Description fragment corresponds to an application service entry point document, e.g. an MPD to a DASH Media Presentation, an HTML page, or a manifest for some other type of streaming formatted content. The entry point document itself typically references additional objects through URIs.

If the MBMS User Service Description contains an Application Service Description fragment, then it indicates that all resources that are directly or indirectly referenced in the application service entry point document are delivered through one of the following means:

-	through at least one of the delivery methods defined under the userServiceDescription element,

-	in the MBMS User Service Announcement as a metadata fragment,

-	through unicast and accessible with HTTP protocol according to clause 7.6.

In Figure 5, the MIME type of the Application Service Description metadata fragment enables the MBMS client to determine, for example, whether the service content is formatted in DASH, or is an HTML5 presentation, and whether and how to process the associated Application Service Description document. In the case of a DASH-over-MBMS service with support for service continuity between broadcast and unicast, the Application Service Description is an MPD. The definition of any other application service and associated Application Service Description is outside the scope of this specification.

The definition of any application service which is not a DASH-over-MBMS service, any specialized handling for such application service delivery over MBMS, as well as the content format with the exception that it is an HTML5 document, and management and hosting of the associated Application Service Description are outside the scope of this specification.

NEXT CHANGE:

## 5.6	DASH and MBMS

The 3GPP Dynamic Adaptive Streaming over HTTP (3GP-DASH) as defined in 3GPP TS 26.247 [98] specifies formats and methods that enable the delivery of streaming service(s) from standard HTTP servers to 3GP-DASH client(s).  It specifies the description of a collection of Media Segments and auxiliary metadata (all referenced by HTTP-URLs) through a Media Presentation Description (MPD). In addition, ISO/IEC 23009-1 [116] defines DASH services.

MBMS is designed to serve large receive groups with same content. The MBMS Download Delivery Method is designed to deliver an arbitrary number of objects via MBMS to a large receiver population. MBMS download delivery defines several methods to increase reliability such as FEC and file repair. The download delivery method allows the delivery of 3GP-DASH Segments, Media Presentation Descriptions, as well as other objects referenced in the MPD as defined in [98].

In order to support DASH Streaming in MBMS, the USBD metadata fragment for a service shall contain either or both a r9:mediaPresentationDescription element referencing an MPD, and a r12:appService referencing an MPD, which is also a metadata fragment describing the service. The referenced MPD corresponds to a metadata fragment as defined in [98]. If the USBD contains a reference to an MPD containing broadcast Representation(s), then

1)	The user service shall be a download delivery service, i.e. shall include at least one deliveryMethod element referencing an SDP that describes FLUTE transport.

2)	If objects are not already provided during MBMS User Service Announcement, the MBMS download session shall deliver objects that are referenced by the MPD, all updates of the MPD and objects that are referenced by any update of the MPD, which are not already provided during MBMS User Service Announcement.

3)	If a Segment is delivered as a FLUTE object then all of the following shall hold:

a)	The MBMS download session shall deliver segments such that the last packet of the delivered object is available at the UE latest at its segment availability start time as announced in the MPD.

b)	The Content-Location element in the FDT for the delivered object shall match the Segment URL in the MPD.

4)	If an MPD update (with or without a metadata envelope) is delivered as a FLUTE object then all of the following shall hold:

c)	The URL of the delivered object shall match the URI of the appropriate referenced MPD.

d)	The MPD update shall be a valid update to a previously delivered MPD or an MPD delivered during MBMS User Service Announcement.

5)	If an Initialization Segment (with or without a metadata envelope) is delivered as a FLUTE object then the URI of the delivered object shall match the appropriate reference in the MPD.

6)	If any other resource in the MPD is delivered (e.g. xlinked resource, metrics, etc.) then

e)	The Content-Location element in the FDT Instance for the delivered object shall match the URL of the object in the MPD.

f)	The MBMS download session shall deliver objects such that the last packet of the delivered object is available at the UE latest at the earliest time a DASH client operating on the delivered MPD sequence may ask for the resource.

In the case a real-time streaming service is provided as DASH streaming over MBMS, then the MPD@type (attribute ‘type’ of the MPD) shall be set to "dynamic", i.e. this indicates that the segments get available over time, latest at its announced segment availability start time.  When MPD@minimumUpdatePeriod (attribute ‘minimumUpdatePeriod’ of the MPD) is present, then the UE should expect MPD updates to be sent in the FLUTE session with the media segments and treat these updates as defined in step 4 above.

The objects delivered with the MBMS download delivery method shall be formatted according to the announcement in the MPD. The MPD and the described Media Presentation should conform to a profile specified in [98].

Furthermore, the Media Presentation Description fragment may contain reference(s) to Initialization Segment Description fragment(s) whose content is an Initialization Segment as defined in [98].

If the DASH Media Presentation conforms to the DASH Profile for CMAF content as defined in ISO/IEC 23009-1 [116], then the DASH Media Presentation follows the constraints for the CMAF Segment formats and additional CMAF constraints. In this case, the userServiceDescription element should contain an r12:appService element with a MIME type containing  "application/dash+xml profiles=' urn:mpeg:dash:profile:cmaf:2019'" indicating that the DASH Media Presentation conforms to DASH Profile for CMAF content as defined in ISO/IEC 23009-1. The MBMS distributed CMAF content may be consumed by streaming clients that support playback of CMAF content. The announcement of CMAF content to such a CMAF capable streaming client is implementation-specific.

If a Hybrid DASH/HLS service is provided as DASH over MBMS, then the content should conform to DASH Profile for CMAF content as defined in ISO/IEC 23009-1 [116] and the announcement in the userServiceDescription element should be provided as documented above.

The r9:mediaPresentationDescription element refers to an MPD which describes only the Representation(s) available over the MBMS bearer(s), and shall be used by UEs complying to previous versions of this specification.

The r12:appService element may refer to a unified MPD which describes Representations available for both broadcast and unicast reception, and shall be used by UEs compliant to this specification. If r12:appService element is absent, and r9:mediaPresentationDescription element is present, then a UE complying with this release of the specification shall use the r9:mediaPresentationDescription. This case also applies to UE compliant to the current release of this specification, deployed in networks of a previous releases not having r12:appService element defined. In practical deployments, different subsets of the Representations described by the unified MPD and referenced by such r12:appService may be specified for

-	broadcast only availability

-	both unicast and broadcast availability,

-	unicast only availability and the Representation is redundant in broadcast coverage, i.e. the usage of these resources does not provide an improved user experience. As an example, this may be a lower bitrate Representation of a media component for which a higher bitrate is available over broadcast, and

-	unicast always availability and the Representation is supplementary in broadcast coverage, i.e. even in broadcast coverage these resources provide an improved user experience. As an example, this may be a secondary language that is only accessible over unicast.

For more details on consistent signalling for resources in hybrid service offerings, refer to clause 7.6.

Clause 4.4.3 of this specification enables integrity and/or confidentiality protection of MBMS user services data according to 3GPP TS 33.246 [20]. In this case each DASH formatted file is protected using the Protection of Download Data as described in [20].

As this protection mechanism is performed in the underlying layer of the DASH client it is transparent to 3GP-DASH client and not reflected in the MPD associated to the DASH representation.

For HTTP streaming, QoE reporting on MBMS level can be activated as described in section 8.3.2.1 or 8.3.2.2, and QoE reporting shall in such case be done as specified in section 8.4. The Network Resource, Loss of Objects, and Distribution of Symbol Count Underrun for Failed Blocks QoE metrics are relevant to DASH over MBMS.

QoE reporting can also be activated on DASH level as specified in clause 10 or Annex F of [98], and reporting shall in such case be done according to [98].

NOTE:	One way of supporting the delivery of a subset of the nominally requested content by the DASH client which indicates explicit willingness to accept such incomplete content, and based on a specific UE implementation architecture, is described in clause 7.2.3 of TR 26.946 [110].

Media decoders for DASH streaming as defined in this clause delivered over MBMS are specified in clause 10 of this specification.

DASH streaming may also be switched from MBMS to unicast. In the case of PSS-based delivery, the media decoders for DASH are specified in clause 7.3.6 of [98].

Service interactivity functionality may be supported in a DASH-over-MBMS service. If supported, such functionality shall be implemented in accordance to TS 26.247 [98] as follows:

-	by the BM-SC: delivery of interactivity event signaling as described in [98], clause 15, and

-	by the DASH client: processing of interactivity event signaling, and forwarding of event metadata to the subscribing service interactivity application, as described in [98], clause 15.

NEXT CHANGE:

# J.1	User Service Description schema

This clause specifies the main USD Schema according to the current release. In addition to clause 11.2.1 specifying the USD schema content, additional requirements are included in this clause to ensure backward and forward compatibility.

In this version of the specification the network shall set the schemaVersion element, defined as a child of bundleDescription element, to 5.

The schema version attribute (part of the schema instruction) shall be included in the UE schema and the network schema.

NOTE 1:	The value of the schemaVersion element and version attribute is intended to be increased by 1 in every future releases where new element(s) or attribute(s) are added.

When a UE receives an instantiation of a USD compliant to this schema, it shall determine the USD schema version required to parse the USD instantiation as follows:

-	If the UE supports one or more versions of the USD schema with the schema version attribute, then the UE shall use the USD schema that has the highest schema version attribute value that is equal to or less than the value in the received schemaVersion element;

-	Otherwise, if the UE supports a USD schema without a schema version attribute, or all of its USD schemas with the schema version attribute have a value greater than the value received in the schemaVersion element, then the UE shall use its USD schema without a version attribute.

NOTE 2:	To avoid a schema validation error (Unique Particle Attribution rule) if an optional element from another namespace is specified just prior to the xs:any namespace="##other"  instructions (3 instances in the USD schema), a schemaVersion element is defined as the last child of the root element (bundleDescription element), and a delimiter element is specified preceding the 2 other occurrences of xs:any instructions. These 2 elements are defined in clause J.2, and the schema file name is "schema-version.xsd".

Release 12 USD extension schema may be extended in future release of this specification. The version of the Release 12 USD extension (as identified by the version attribute in the Rel-12 extension schema, see 11.2.1.2) that shall be selected by the UE depends on the main USD schema version (as identified by the version attribute in the main USD schema) selected by the UE according to the requirements above. The following table shows which version of the Release 12 USD extension schema is to be used depending on which version of the main USD schema version is selected at the UE.

The Release 14 USD extension schema may also be extended in future release of this specification. The version of the Release 14 USD extension (as identified by the version attribute in the Rel-14 extension schema, see 11.2.1.2) that shall be selected by the UE depends on the main USD schema version (as identified by the version attribute in the main USD schema) selected by the UE according to the requirements above. The following table shows which version of the Release 14 USD extension schema is to be used depending on which version of the main USD schema version is selected at the UE.

Table J1-1: Release 12 and Release 14 USD extension schema version selection

| Main USD Schema version selected | Release 12 USD Extension schema version | Release 14 USD Extension schema version |
| --- | --- | --- |
| 1 | Not applicable | Not applicable |
| 2 | 1 | Not applicable |
| 3 | 1 | 1 |
| 4 | 1 | 1 |
| 5 | 1 | 1 |

Release 15 USD extension schema may be extended in future release of this specification. The version of the Release 15 USD extension (as identified by the version attribute in the Rel-15 extension schema, see 11.2.1.2) that shall be selected by the UE depends on the main USD schema version (as identified by the version attribute in the main USD schema) selected by the UE according to the requirements above. The following table shows which version of the Release 15 USD extension schema is to be used depending on which version of the main USD schema version is selected at the UE.

Table J1-1a: Release 15 USD extension schema version selection

| Main USD Schema version selected | Release 15 USD Extension schema version |
| --- | --- |
| 3 | Not applicable |
| 4 | 3 |
| 5 | 3 |

The schema file names associated to the USD schemas specified in sub-clause 11.2.1 are as follows:

-	Release 7 extension schema: USD-Rel-07-schema-snippet.xsd

-	Release 8 extension schema: USD-Rel-08-schema-snippet.xsd

-	Release 9 extension schema: USD-Rel-09-schema-snippet.xsd

-	Release 12 extension schema: USD-Rel-12-schema-snippet.xsd

-	Release 14 extension schema: USD-Rel-14-schema-snippet.xsd

-	Release 15 extension schema: USD-Rel-15-schema-snippet.xsd

The file name for the main USD schema (as defined below) is "USD-schema-main.xsd".

User Service Description

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3GPP:metadata:2005:MBMS:userServiceDescription"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:r7="urn:3GPP:metadata:2007:MBMS:userServiceDescription"

xmlns:r8="urn:3GPP:metadata:2008:MBMS:userServiceDescription"

xmlns:r9="urn:3GPP:metadata:2009:MBMS:userServiceDescription"

xmlns:r12="urn:3GPP:metadata:2013:MBMS:userServiceDescription"

xmlns:r14="urn:3GPP:metadata:2017:MBMS:userServiceDescription"

xmlns:r15="urn:3GPP:metadata:2018:r15:MBMS:userServiceDescription"

xmlns:sv="urn:3gpp:metadata:2009:MBMS:schemaVersion"

targetNamespace="urn:3GPP:metadata:2005:MBMS:userServiceDescription"

elementFormDefault="qualified"

version="5">

<xs:import schemaLocation="USD-Rel-07-schema-snippet.xsd" namespace="urn:3GPP:metadata:2007:MBMS:userServiceDescription"/>

<xs:import schemaLocation="USD-Rel-08-schema-snippet.xsd" namespace="urn:3GPP:metadata:2008:MBMS:userServiceDescription"/>

<xs:import schemaLocation="USD-Rel-09-schema-snippet.xsd" namespace="urn:3GPP:metadata:2009:MBMS:userServiceDescription"/>

<xs:import schemaLocation="USD-Rel-12-schema-snippet.xsd"  namespace="urn:3GPP:metadata:2013:MBMS:userServiceDescription"/>

<xs:import schemaLocation="USD-Rel-14-schema-snippet.xsd"  namespace="urn:3GPP:metadata:2017:MBMS:userServiceDescription"/>

<xs:import schemaLocation="USD-Rel-15-schema-snippet.xsd"  namespace="urn:3GPP:metadata:2018:r15:MBMS:userServiceDescription"/>

<xs:import schemaLocation="schema-version.xsd"

namespace="urn:3gpp:metadata:2009:MBMS:schemaVersion"/>

<xs:element name="bundleDescription" type="bundleDescriptionType"/>

<xs:complexType name="bundleDescriptionType">

<xs:sequence>

<xs:element name="userServiceDescription" type="userServiceDescriptionType" maxOccurs="unbounded"/>

<xs:element ref="r7:initiationRandomization" minOccurs="0"/>

<xs:element ref="r7:terminationRandomization" minOccurs="0"/>

<xs:element ref="sv:schemaVersion"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="fecDescriptionURI" type="xs:anyURI" use="optional"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:complexType name="userServiceDescriptionType">

<xs:sequence>

<xs:element name="name" type="nameType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="serviceLanguage" type="xs:language" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="requiredCapabilities" type="requirementsType" minOccurs="0"/>

<xs:element name="deliveryMethod" type="deliveryMethodType" maxOccurs="unbounded"/>

<xs:element name="accessGroup" type="accessGroupType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element ref="r7:serviceGroup" minOccurs="0"/>

<xs:element ref="r7:initiationRandomization" minOccurs="0"/>

<xs:element ref="r7:terminationRandomization" minOccurs="0"/>

<xs:element ref="r8:Registration" minOccurs="0"/>

<xs:element ref="r9:mediaPresentationDescription" minOccurs="0"/>

<xs:element ref="r9:schedule" minOccurs="0"/>

<xs:element ref="r9:availabilityInfo" minOccurs="0"/>

<xs:element ref="sv:delimiter"/>

<xs:element ref="r12:appService" minOccurs="0"/>

<xs:element ref="r12:KeepUpdatedService" minOccurs="0"/>

<xs:element ref="r12:mooDConfiguration" minOccurs="0" maxOccurs="1"/>

<xs:element ref="r12:consumptionReporting" minOccurs="0" maxOccurs="1"/>

<xs:element ref="sv:delimiter"/>

<xs:element ref="r15:ROMSvcRfParams" minOccurs="0"/>

<xs:element ref="sv:delimiter"/>

<xs:element ref="r12:appService" minOccurs="0" maxOccurs="unbounded"/>

<xs:element ref="sv:delimiter"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="serviceId" type="xs:anyURI" use="required"/>

<xs:attribute ref="r7:serviceClass"/>

<xs:attribute ref="r14:romService"  use="optional"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:complexType name="accessGroupType">

<xs:sequence>

<xs:element name="accessBearer" type="xs:string" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="id" type="accessGroupIdType" use="required"/>

</xs:complexType>

<xs:complexType name="deliveryMethodType">

<xs:sequence>

<xs:element ref="r7:unicastAccessURI" minOccurs="0" maxOccurs="unbounded"/>

<xs:element ref="r8:alternativeAccessDelivery" minOccurs="0"/>

<xs:element ref="sv:delimiter"/>

<xs:element ref="r12:broadcastAppService" minOccurs="0" maxOccurs="unbounded"/>

<xs:element ref="r12:unicastAppService" minOccurs="0"/>

<xs:element ref="r12:appComponent" minOccurs="0" maxOccurs="unbounded"/>

<xs:element ref="r12:serviceArea" minOccurs="0" maxOccurs="unbounded"/>

<xs:element ref="sv:delimiter"/>

<xs:element ref="r15:supplementaryUnicastAppService" maxOccurs="unbounded"/>

<xs:element ref="sv:delimiter"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="accessGroupId" type="accessGroupIdType" use="optional"/>

<xs:attribute name="associatedProcedureDescriptionURI" type="xs:anyURI" use="optional"/>

<xs:attribute name="protectionDescriptionURI" type="xs:anyURI" use="optional"/>

<xs:attribute name="sessionDescriptionURI" type="xs:anyURI" use="required"/>

<xs:attribute name="accessPointName" type="xs:anyURI" use="optional"/>

<xs:attribute name="group" type="xs:unsignedByte" use="optional"/>

<xs:attribute name="PLMN" type="xs:string" use="optional"/>

<xs:attribute name="p-serviceArea" type="p-serviceAreaType" use="optional"/>

<xs:attribute ref="r12:inbandMetadata"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:simpleType name="p-serviceAreaType">

<xs:list itemType="xs:unsignedShort"/>

</xs:simpleType>

<xs:complexType name="nameType">

<xs:simpleContent>

<xs:extension base="xs:string">

<xs:attribute name="lang" type="xs:language" use="optional"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="accessGroupIdType">

<xs:restriction base="xs:nonNegativeInteger"/>

</xs:simpleType>

<xs:complexType name="requirementsType">

<xs:sequence>

<xs:element name="feature" type="xs:unsignedInt" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

NEXT CHANGE:

## L.2.1	Introduction

The function of Service Discovery is to allow UEs to find the available MBMS User Services defined on the MBMS enabled network and the associated service access information (e.g., FLUTE session parameters, TMGI, file repair servers, etc.) for MBMS User Services of interest. Among others, the UE needs the service access information to initiate the reception of a particular MBMS User Service and to find the data of associated with the MBMS User Service on the radio interface.

In a typical deployment scenario that is covered by this profile, the MBMS metadata fragments are announced to the UEs in advance of a MBMS session. UEs monitor with a certain periodicity a Service Announcement Channel and stores received metadata for potential later usage, so that UEs have in most cases all relevant access information when the UE wants to activate reception of an MBMS User Service. There are also use cases where it is necessary to update the metadata fragments during, and sometimes after, the completion of MBMS file download delivery.

Clause 5.2 of this specification defines multiple optional features, such as Service Announcement over MBMS and unicast bearers. The present clause specifies one profile for Service Discovery / Announcement over MBMS bearer services. The intention of this profile is to limit the number of optional features and to define the set of provided metadata fragments.

This section profiles Service Announcement over MBMS bearers as described in clause 5.2.3 by defining a Service Announcement Channel (SACH). The profile describes how the MBMS metadata (as defined in clause 5.2.2) defines the User Service access information to enable UEs to receive MBMS User Services for Live DASH, Live HLS, Live hybrid DASH/HLS service or Non-Real-Time file delivery.

NEXT CHANGE:

## L.2.3	Service Announcement (SA) File structure

Service announcement metadata shall be transported via MBMS Download Delivery. A Service Announcement file (SA file) shall be formatted as an aggregated Multipart MIME file of multipart/related type as defined in clause 5.2.6. All metadata fragments of one MBMS User Server shall be contained within the same SA file. The SA file may contain metadata fragments of more than one MBMS User Service.

An SA file shall be uniquely identified by its URL, and provided as the value of the Content-Location field in FDT Instances. Its MIME type is provided as value of the Content-Type field in FDT Instances. When a USBD or any other metadata fragment of an MBMS User Service is updated, an updated SA file shall be transmitted with a new Content-MD5 value in the FLUTE FDT Instance describing the file with the same URL. The UE shall use the Content-MD5 to identify new SA File versions.

The SA file shall contain exactly one metadata envelope, and for each service, at least an USBD (with one userServiceDescription element), one Session Decription, and one Schedule Description fragment. Optionally, the SA file may contain Associated Delivery Procedure Description (ADPD). For DASH services, the SA file may further contain the Media Presentation Description (MPD), the Application Service Description (ASD) whose content is the unified MPD, and all Initialization Segment Description (ISD) metadata fragments of the MBMS User Service. For HLS services, the SA file may further contain the Application Service Description (ASD) whose content is the Master Playlist [X] and all Initialization Segment Description (ISD) metadata fragments [144].

Some metadata fragments may be added and / or updated in-band with the content as defined in clause L.2.8. When metadata fragments are referenced, but not delivered within the SA file, such as the case of the Associated Delivery Procedure Description, the metadata fragments shall be provided as in-band fragments as defined in clause L.2.8. The SA file might not include Associated Delivery Procedure Description (ADPD) fragments in multiple BMSC deployments where different serviceURLs are used to direct file repair and reception reporting to different services on each BM-SC.

The metadata envelope shall be included as the root body part on the SA file and shall include a list of item child elements, with one item instance for every included metadata fragment. The metadataURI attribute of a given item element shall represent a unique and absolute HTTP URL referencing the metadata fragment associated with that item. The metadata envelope in the SA file shall not include the metadataFragment element, i.e., the metadata envelope does not embed a metadata fragment, as described in section 11.1.4. This is shown in the example in clause L.2.10.

The SA file should contain all DASH Initialization Segment Description metadata fragments, which are referenced by all broadcast Representations on any Media Presentation Description fragment, referenced by r9:mediaPresentationDescription element, included in the SA file. Any Initialization Segment Description fragment in the SA file shall be base64 encoded. The URL in the metadataURI in the metadata envelope and the Content-Location field in the Multipart MIME boundary header shall contain the same Initialization Segment Description URL, which is included in the MPD to reference the ISD.

The SA file should contain all HLS Initialization Segment Description fragments, each of which contains a Media Initialization Section [X], which are referenced by the broadcasted HLS Media Playlists. Any HLS Initialization Segment Description fragment in the SA file shall be base64 encoded. The URL in the metadataURI in the metadata envelope and the Content-Location field in the Multipart MIME boundary header shall contain the same Media Initialization Section URL, as given in the HLS Media Playlists with the EXT-X-MAP tag [X].

The SA file shall be compressed using the RFC 1952 [42] content/transport encoding for transmission and shall have a .gzip file extension. The multipart MIME file shall be carried as a compressed (gzipped) file and uncompressed by the MBMS client, i.e., FLUTE level compression is not to be used.

As allowed in RFC1952, the gzip file format shall include the original file name and the FLG.FNAME flag shall be set to true. This allows for the uncompressed file name to be signaled in gzip files.

NEXT CHANGE:

## L.2.4	Metadata Envelope

The metadata envelope provides references to all metadata fragments within the same SA file document via the metadataURI attribute, each instance of which shall be unique and represents an absolute HTTP URL.

When delivering metadata fragments in-band (see clause L.2.8), the metadata fragments may be embedded in or referenced by metadata envelopes (see clause 11.1.4). In-band metadata fragments shall be sent on the FLUTE session of a MBMS User Service together with the actual content for that MBMS User Service, not on the SACH.

The validity time, defined by the validFrom and validUntil attributes for all metadata fragments of the same MBMS User Service shall be identical to ensure that all fragments of that the User Service are valid for the same period. Therefore, an MBMS User Service is valid only during the period between validFrom and validUntil identically defined for all fragments describing that User Service.

The validFrom and validUntil attributes in the following required metadata fragments define when a service is valid: {USBD, Session Description, Schedule Description} for file delivery services, {USBD, Session Description, Schedule Description, Media Presentation Description, Application Service Description, Initialization Segment Description(s)} for DASH services and {USBD, Session Description, Schedule Description, Application Service Description, Media Initialization Section(s)} for HLS services; a service is valid (fully defined) only if each of these fragments is available during the common validity period. Note that the validity of the ADPD is not considered as it is not critical for service reception, and it might be delivered in-band only. The aligned validity information shall include the ADPD back-off and random period such that a validUntil occurring before the ADPD’s validity period is over will not unintentionally expire the service and thus abort the ADPD procedures.

The validUntil value for a metadata fragment may be adjusted without an increment to the version value for that fragment. When this occurs the BM-SC shall deliver the file with an updated MD5 checksum.  Upon detecting the MD5 value change, the MBMS client shall parse the metadata envelope for the corresponding fragments and update the validUntil and version values that have changed.

If the version does not change, the UE only needs to update the validity information and does not need to overwrite the fragment if the version did not change. If the fragment has changed and needs to be overwritten, a new version number shall be signaled in the envelope.

The system shall assign relatively short validUntil in the future for all fragments and it shall also periodically update the validUntil for all fragments to a new short time in the future to ensure proper memory management on the UEs. Only when the validUntil for the fragments of a User Service have elapsed should the UE safely delete those metadata fragments and therefore delete the associated MBMS User Service. If signaling an MBMS User Service deletion is desired before the currently defined validUntil for that service, the system shall setting the validUntil for all fragments of that MBMS service to a time in the past.

Version management is handled in such a way that the system shall ensure version numbers are always increasing. When a version number change is required for a given metadata fragment, the system shall send a new version of that fragment, both in-band and via the SACH in the Multipart MIME file.

The system shall describe and signal only one version of each metadata fragment at any point in time.  The eMBMS client shall also only maintain the fragment of the highest version for each metadata fragment with the earlier version removed from the MBMS client storage.

NEXT CHANGE:

## L.2.5	User Service Bundle Description Fragment

The MBMS User Service Bundle Description (USBD) describes only a single user service in the Multipart MIME file. There shall be one userServiceDescription element per USBD. To announce multiple MBMS User Services, there shall be separate USBD instances for each MBMS User Service within the Multipart MIME file.

Each MBMS User Service instance is described through the following metadata fragments. For each MBMS User Service instance, exactly one bundleDescription element shall be present, which shall contain exactly one userServiceDescription element. The userServiceDescription element shall contain exactly one deliveryMethod element and exactly one r9:schedule element. The deliveryMethod element references a Session Description in the form of an SDP file, which shall describe one MBMS Download Delivery Session.

The USBD shall contain the requiredCapabilities element with its feature child element. The feature element value shall be set to "22", which identifies the "MBMS User Service Discovery / Announcement Profile 1a", as defined in clause 11.9.

In the case of Live DASH services, the userServiceDescription element shall also contain exactly one r9:mediaPresentationDescription element, which references a DASH Media Presentation Description (MPD).

In the case of Live DASH services, the userServiceDescription element may also contain one r12:appService element, with a MIME type containing  "application/dash+xml", which references an Application Service Description whose content is a unified MPD (see subclause 7.6).

In the case of Live DASH services based on a CMAF profile, the userServiceDescription element may also contain one r12:appService element, with a MIME type containing  "application/dash+xml profiles=' urn:mpeg:dash:profile:cmaf:2019'" or a compatible signaling as documented in clause 5.6.

In the case of Live HLS services, the userServiceDescription element shall also contain exactly one r12:appService element with the mimeType attribute set to "application/vnd.apple.mpegurl", which references an Application Service Description whose content is an HLS Master Playlist [x].

In the case of hybrid Live DASH/HLS services, the userServiceDescription element:

-	shall contain exactly one r9:mediaPresentationDescription element, which references a DASH Media Presentation Description (MPD);

-	should contain one r12:appService element, with a MIME type containing "application/dash+xml profiles=' urn:mpeg:dash:profile:cmaf:2019", which references an Application Service Description whose content is a unified MPD (see subclause 7.6); and

-	shall contain exactly one r12:appService element with the mimeType attribute set to "application/vnd.apple.mpegurl", which references an Application Service Description whose content is an HLS Master Paylist [x].

The USBD fragment may contain an r9:availabilityInfo element, with one or more infoBinding elements. The infoBinding element shall contain the child elements radioFrequency and serviceArea. If the r9:availabilityInfo element is absent, then the device shall assume that the corresponding MBMS User Service offering is not geographically constrained within the service area footprint of the MBMS service operator.

For this service announcement profile and for E-UTRAN, the UE shall ignore the radioFrequency child element. Instead, the UE shall read and use the radio frequency from the E-UTRAN System Information Block 15 (SIB), which provides the binding between the frequency agnostic Service Area ID (SAI) and the radio frequency.

This is the list of supported attributes and elements:

-	Mandatory

-	bundleDescription.sv:schemaVersion

-	set as specified for the schema version in use

-	bundleDescription.userServiceDescription

-	bundleDescription.userServiceDescription.serviceId

-	bundleDescription.userServiceDescription.r7:serviceClass

-	bundleDescription.userServiceDescription.deliveryMethod

-	bundleDescription.userServiceDescription.deliveryMethod@sessionDescriptionURI

-	bundleDescription.userServiceDescription.deliveryMethod.sv:delimiter

-	set to 0 and positioned as specified by the schema version in use

-	bundleDescription.userServiceDescription.requiredCapabilities

-	bundleDescription.userServiceDescription.requiredCapabilities.feature

-	bundleDescription.userServiceDescription.r9:schedule

-	bundleDescription.userServiceDescription.sv:delimiter

-	set to 0 and positioned as specified by the schema version in use

-	Mandatory for DASH services

-	bundleDescription.userServiceDescription.r9:mediaPresentationDescription

Optional for DASH services

-	bundleDescription.userServiceDescription.r12:appService

-	Mandatory for HLS services

-	bundleDescription.userServiceDescription. r12:appService

-	Optional

-	bundleDescription.userServiceDescription.name

-	bundleDescription.userServiceDescription.serviceLanguage

-	bundleDescription.userServiceDescription.r9:availabilityInfo

-	bundleDescription.userServiceDescription.r9:availabilityInfo.infoBinding.serviceArea

-	A list of serviceArea advertises SAIs where the service is available – requires SIB15 support

-	bundleDescription.userServiceDescription.r9:availabilityInfo.infoBinding.radioFrequency

-	Includes at least one radioFrequency

-	bundleDescription.userServiceDescription.deliveryMethod

-	bundleDescription.userServiceDescription.deliveryMethod@associatedProcedureDescriptionURI

-	bundleDescription.userServiceDescription.deliveryMethod@accessPointName

-	accessPointName – may be set to the same value for all services

-	bundleDescription.userServiceDescription.deliveryMethod.r12:broadcastAppService

This is the list of not supported attributes and elements:

-	bundleDescription@fecDescriptionURI.

-	bundleDescription.userServiceDescription@accessGroup

-	bundleDescription.userServiceDescription.serviceGroup

-	bundleDescription.initializationRandomization

-	bundleDescription.terminationRandomization

-	bundleDescription.userServiceDescription.initializationRandomization

-	bundleDescription.userServiceDescription.terminationRandomization

-	bundleDescription.userServiceDescription.r8:Registration

-	bundleDescription.userServiceDescription.deliveryMethod@accessGroupID

-	bundleDescription.userServiceDescription.deliveryMethod@protectionDescriptionURI

-	bundleDescription.userServiceDescription.deliveryMethod.r8:alternativeAccessDelivery

-	bundleDescription.userServiceDescription.deliveryMethod.r12: broadcastAppService.serviceArea

bundleDescription.userServiceDescription.deliveryMethod.r12:unicastAppService

-	bundleDescription.userServiceDescription.deliveryMethod.r12:appComponent

-	bundleDescription.userServiceDescription.deliveryMethod.r12:serviceArea

-	bundleDescription.userServiceDescription.deliveryMethod@r12:inbandMetadata

-	bundleDescription.userServiceDescription.r12:appService.identicalContent

-	bundleDescription.userServiceDescription.r12:appService.alternativeContent

-	bundleDescription.userServiceDescription.r12:KeepUpdatedService

NEXT CHANGE:

# L.3	MBMS User Service Discovery / Announcement Profile 1b

The function of Service Discovery is to allow UEs to find the available MBMS User Services defined on the MBMS enabled network and the associated service access information (e.g. FLUTE session parameters, TMGI, file repair servers, etc.) for MBMS User Services of interest. The UE needs the service access information to initiate the reception of a particular MBMS User Service, and to find the data associated with the MBMS User Service on the radio interface.

The Service Announcement Profile defined in this clause follows the principles of the Service Announcement Profile 1a as defined in clause L.2 with the following constraints:

-	Service Announcement metadata fragments shall be delivered as one SA file.

-	In the case of Live DASH Services, the SA file shall contain the Media Presentation Description (MPD) fragment for the MBMS User Service, referenced by the r9:mediaPresentationDescription element in the USBD. If the USBD also references an Application Service Description fragment with the r12:appService element, the SA file shall also contain the Application Service Description fragment whose content is the unified MPD fragment.

-	In the case of Live HLS Services, the SA file shall contain the Application Service Description fragment whose content is a Master Playlist as defined in [X] for the MBMS User Service, referenced by the r12:appService element in the USBD.

-	In the case of Live hybrid DASH/HLS Services, the SA file shall contain:

-	the Media Presentation Description (MPD) fragment for the MBMS User Service, referenced by the r9:mediaPresentationDescription element;

-	the Application Service Description (ASD) fragment whose content is the unified MPD, if the USBD also references an ASD fragment with the r12:appService element whose MIME type contains "application/dash+xml"; and

-	the Application Service Description (ASD) fragment whose content is the HLS master playlist fragment for the MBMS User Service, referenced by the r12:appService element in the USBD, with the mimeType attribute set to "application/dash+xml profiles=' urn:mpeg:dash:profile:cmaf:2019".

-	In the case of Live DASH Services, the SA file shall contain (all) needed Initialization Segment Description (ISD) fragments for the MBMS User Service.

-	In the case of Live HLS Services, the SA file shall contain (all) needed Initialization Segment Description (ISD) fragments whose content is a Media Initialization Section ([x]) for the MBMS User Service.

-

In the case of Live hybrid DASH/HLS Services, the SA file shall contain (all) needed Initialization Segment Description (ISD) fragments for the MBMS user service, used as Initialization Segment by the DASH clients, and used as Media Initialization Section by the HLS clients.

-	Each In-Band Fragment shall be embedded in one Metadata Envelope as defined in clause 11.1.4. Each metadata envelope shall contain exactly one metadata fragment.

-	The feature child element of the requiredCapabilities element shall have the value "23", corresponding to the "MBMS User Service Discovery / Announcement Profile 1b", as defined in clause 11.9.

NEXT CHANGE:

Annex M (informative):
Guidelines for audiovisual streaming using HLS over MBMS broadcasts

# M.1	Introduction

In order to interoperate with MBMS, the HLS client should be MBMS-aware. This allows it to take best advantage of broadcast content and unicast content when and where broadcast is not available.

The HLS Master Playlist of an asset pertains equally to both the broadcast and independent unicast availability of a media asset. Generally only a small subset of HLS Media Playlists published in a Master Playlist will be available via an MBMS broadcast. This subset is typically chosen to satisfy the union of all supported UE media decoder compatibilities. A shared Master Playlist is required to support the unicast fallback described in this section.

# M.2	Guidelines

## M.2.1	Providing unicast fallback for both HLS and DASH

### M.2.1.1	Recommended client support for unicast fallback

An MBMS-aware client should provide support for unicast fallback, although a content provider is not required to offer one. A simultaneous unicast & broadcast delivery provides a fallback for broadcast delivery. Moreover a simultaneous broadcast, with unicast fallback, affords clients the ability to transition between broadcast and non-broadcast domains such as transitioning from a WiFi to cellular service during presentation.

### M.2.1.2	Recommended unicast availability for HLS and HLS/DASH Hybrid Services

It is recommended that both an MBMS HLS and Hybrid HLS/DASH service include a mechanism for unicast fallback. In this context, both the HLS Master Playlist and the unified DASH MPD serve the dual purpose of describing streams available via the MBMS bearer service and those available, separately, via unicast.

### M.2.1.3	Implied unicast stream availability

In subclause 7.6.2.2, Redundant Unicast Resource Specific Metadata, an explicit mechanism for nominating unicast availability is detailed. An MBMS-aware client should assume that any media playlist not included in an instance of r12:broadcastAppService is available via unicast.

## M.2.2	Media Playlist delivery

The HLS Media Playlist specifies available segments of a given stream. Media Playlists for live content are updated each time a new segment is generated. In order to control the order of delivery, the Media Playlist corresponding to the broadcasted HLS Renditions should be delivered within the same MBMS session as the media segments.

It is recommended to transmit the updated Media Playlist with a new Content-MD5 value in the FLUTE FDT, so that the UE can identify new versions of Media Playlists. It is also recommended to use Cache-control directives in the FDT (7.2.10.5) for Media Playlists, to provide an expiration date with the Expires subelement, so that the UE won’t keep in cache an old version.

Following requirements from subclause 5.7, the MBMS delivers media segment(s) and Media Playlist in a way that the last packet of the delivered object (that represents the Media Playlist specifying a media segment) is transmitted after the last packet of the delivered object (that represents the said media segment), as illustrated in figure x.

seg#12
seg#13
seg#14

Segment#10

Segment#12

Segment#14

Media playlist

seg#8
seg#9
seg#10

seg#9
seg#10
seg#11

seg#10
seg#11
seg#12

seg#11
seg#12
seg#13

Segment#13

Segment#11

Figure x: Transmission of the media playlist within the MBMS session