3GPP TSG-S4 Meeting #110e	S4-201275

# Home Office, Everywhere,  19. – 28. August 2020						revision of S4-201192

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | 0025 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of FLUS F-C Stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Tencent, Samsung, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_FLUS |  |  |  |  |  | Date: |  |  | 16.8.20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FLUS F-C stage 3 contains some error and is inconsistent with FLUS stage 2. This CR corrects FLUS stage 3, based on the agreements in S4-AHM529. The FLUS sink resources are corrected and clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some general description of the capability query and the FLUS sink discovery is added to Clause 4.2.1 (General Architecture description). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FLUS Stage 3 is not aligned to Stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.2, 5.2.3, 5.3.6, 7.1, 7.2, 7.3, 7.4, 7.5, 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.2.1	General

Figure 4.2-1 depicts the architecture showing the relevant entities for providing an uplink streaming service.

Figure 4.2-1: FLUS architecture

The uplink streaming service architecture is based on a FLUS source located in a UE and a FLUS sink located in either another UE or the network.

The FLUS source receives media content from one or more capture devices. In the context of this specification, the capture devices are considered as parts of a UE or are connected to it.

When the FLUS sink is located in a UE, the FLUS sink shall forward media content to a decoding and rendering function.

When the FLUS sink is located in the network,

-	the FLUS sink may forward media content to a processing or distribution sub-function. The processing and distribution sub-functions are not in scope of the present specification. The FLUS sink may act as a Media Gateway Function (MGW) and/or an Application Function (AF).

-	an optional Sink Discovery Server offers functionality that allows to discover available FLUS sinks.

-	each FLUS sink provides functionality to query its capabilities and FLUS sessions.

The F reference point connects a FLUS source to a FLUS sink and optionally to the Sink Discovery Server. The F reference point enables the discovery of available FLUS sinks. In addition, the F reference point enables the establishment and control of a single FLUS session between the FLUS source and FLUS sink.

The F reference point also enables the FLUS sink and the FLUS source to mutually authenticate and authorize each other.

FLUS source and FLUS sink are each split into:

-	Media Source and Sink (F-U end-points),

-	Control Source and Sink (F-C end-points),

-	Remote Controller and Remote Control Target (F-RC end-points) and

The UE, the FLUS source and the FLUS sink are considered to be logical functions. Functions are not required to be located in the same physical device; they can be spread over multiple physical devices and interconnected via other interfaces.

Multiple F-RC end-points can exist in a single FLUS source. F-RC end-points are independent of a FLUS sink and depend on the offered service.

The F reference point shall support security functionality for confidentiality protection for all subfunctions.

Details of the functions are shown in Figure 4.2-2.

Figure 4.2-2: Sub-functions of FLUS

Note that in the above diagram, hatched-filled boxes represent FLUS control plane functionality, solid line boxes represent mandatory functionality, and dashed-line boxes corresponds to optional functionality. Also, note that F-C and F-U denote FLUS control and FLUS user plane interactions, respectively, and do not represent reference points.

Control Source and Sink (F-C): FLUS control plane functionality including the associated processing by the FLUS sink of the uploaded media for subsequent downstream distribution, plus FLUS media instantiation selection. F-C may also support configuration of static metadata for the Media session.

Remote Controller and Remote Control Target (F-RC): The Remote Control Target on the FLUS source receives control messages. The messages affect the behaviour of the Media Source in the FLUS source. Examples of commands issued to the Remote Control Target are start or stop of the media upstreaming process in the FLUS source.

Media Source and Sink (F-U): FLUS user plane functionality which includes setup of one or more Media sessions and subsequent media data transmission via Media streams. In some cases, a Media session establishment protocol (e.g. IMS session set-up for MTSI-based FLUS instantiation) is necessary.

NOTE: F-C is not needed when the FLUS sink is an MTSI client that is only capable of rendering. In such event, logical control plane functions such as media and session descriptions and support for a FLUS session establishment are encapsulated in FLUS user plane functionality.

F-C represents the interactions associated with the creation and modification of the configuration of the FLUS sink. F-C allows the Control Source to:

- select a FLUS media instantiation,

- provision static metadata associated with each Media session present in the FLUS session,

- discover, select and configure the processing and distribution sub-functions.

In addition to the delivery of control messages, F-RC represents the interactions associated with the discovery, creation, modification and selection of the FLUS sink configuration by the Remote Controller. F-RC allows the Remote Controller to:

- provide the Media Sink information to the FLUS source,

- select a FLUS media instantiation,

- determine capture device settings and other FLUS source parameters.

The FLUS media instantiation is defined as part of the FLUS session. The user plane (F-U) may also contain the Media stream establishment procedures when needed. Multiple Media streams may be established for one FLUS session.

A Media stream may contain media components of a single content type, e.g., audio, or media components of different content types, e.g., audio and video. A FLUS session may be composed of more than one Media stream containing the same content type, e.g., multiple Media streams of video.

### 4.2.2	Uplink streaming for MTSI

The architecture of uplink streaming for MTSI is depicted in Figure 4.2-3.

Figure 4.2-3: Uplink streaming for MTSI

The reception function of an MTSI client, i.e., the "MTSI rx client" as shown in Figure 4.2-3, is used to realize the Media Sink in the FLUS sink. The FLUS sink may be instantiated as another UE or as an MTSI rx client function in the network.

F-U contains all MTSI-related signalling.

The transmission function of an MTSI client, MTSI tx client, is used to realize the  Media Source in the FLUS source.

**** Next Change ****

### 5.2.3	Data transport

The FLUS source and sink that implement the IMS-based FLUS system shall support data transport as specified in section 7 of TS 26.114 [4].

## 5.3	Generic FLUS system

### 5.3.1	System configuration

In this version of the specification, this function is left for implementation, but expected to be specified in a future version.

### 5.3.2	Session management

#### 5.3.2.1a	General

A FLUS session is the association between a source and a sink. The Session Management procedures defined in this section target the FLUS sink configuration and management of the FLUS session.

#### 5.3.2.1b	FLUS sink discovery

This procedure allows a Control Source to discover a list of available FLUS sinks.

Figure 5.3.2.1b-1: Get current a list of available FLUS sinks

1. The Control Source sends the FLUS sinks request to the FLUS Sink Discovery Server.

2. The Sink Discovery Server provides a list of FLUS sinks

**** Next Change ****

### 5.3.6	List of FLUS Sink Configuration properties

All FLUS Sink Configuration properties, except for the resource id, are always carried in an HTTP message body. The access-token is always carried as part of HTTP headers. Except for the FLUS session creation request (where the id is not present), the resource id shall be present in the URL of all requests that relate to a specific FLUS Sink Configuration.

In the table below, the following assertions are made:

-	Table header: C stands for Create FLUS session procedure, G is for Get FLUS session properties procedure, U is for Update FLUS session properties procedure and T is for Terminate FLUS session procedure. "I", and "O" respectively denote "request" (going Into the FLUS sink), and response (going Out of the FLUS sink).

-	Optional ("O") means that the property may or may not be sent/received during a REST transaction. It does not necessarily mean that the property is optional. It is possible, for example, that a session is not yet active because the FLUS source has not set the property in any previous update transaction using the PUT or PATCH HTTP method, as opposed to representing a hint on the importance of the property for the FLUS sink.

-	A property marked as optional (O) in a request message may be present in the request. When not present in the request body, the property, if present in the FLUS sink, will not be updated.

-	A property marked as optional (O) in a response message is only present in the response when a value is assigned or changed by the FLUS sink.

-	A property marked as mandatory (M) in a response message is always present in the response. The FLUS sink provides defaults, which may be modified subsequently by the content provider.

-	A blank cell in the table means "forbidden" (the property cannot be added to the request or returned by the FLUS sink, depending on the transaction direction).

Table 5.3.6-1: List of FLUS Sink Configuration properties

| Property Name | Property Description | C I | C O | G I | G O | U I | U O | T I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| id | Identifier of the FLUS Sink Configuration resource. Note that "id" is only provided within an HTTP body during the Create FLUS session response. Otherwise, "id" should be present in the message URL to identify the resource in the FLUS sink. Type; Unit; Default. Integer; None; N/A. |  | M |  |  |  |  |  |
| fu_instantiation | Identifier of the FLUS media instantiation that is used by this FLUS session. Vendor specific enumeration values shall start with "vnd-" followed by a unique vendor name and optionally followed by additional characters. The F-U instantiation shall be provided as a globally unique URN. Type; Unit; Default. URI; None; All. |  |  |  | M | O |  |  |
| entrypoint_URL | Entry point URL information (e.g., SIP URL) for establishing the F-U connection to start the Media streaming. Details on the Entrypoint URL is F-U instantiation specific. |  |  |  |  |  |  |  |
| processing_description | This object provides a media processing description document that defines the post processing pipeline that the FLUS sink shall apply to received media components. The pipeline description may also set the distribution target (incl FLUS sink storage) for the media. The Object has the following properties: - type: the MIME type of the media processing description document - document: the media processing document may be embedded in this element. The document may be base64 encoded depending on the MIME type. - url: the URL to the media processing document. The type and either the document property or the url property shall be provided. The following formats are supported: - The MPEG NBMP Workflow Resource, UTF-8 encoded,, as defined in [17], which describes the requested media processing and the desired distribution mechanism after the processing has been performed. The type field shall be set to "application/mpeg-nbmp-wdd+json" See Annex X on use of NBMP in FLUS. | O | O |  | O | O |  |  |

**** Next Change ****

# 7	Uplink Streaming Control Interface

## 7.1	General

### 7.1.0	Introduction

### A FLUS source can connect to a FLUS sink and start live uplink streaming of content that is captured by a set of connected capture devices connected to the FLUS source. This section specifies REST API procedures for FLUS sink discovery, capability discovery, session setup, and session termination between the FLUS source and the FLUS sink.

### 7.1.1	Resources

#### 7.1.1.0		General

F-C defines a set of resources managed at the FLUS sink that are controlled by the FLUS source.

The request URI used in each HTTP request from the Control Source towards the FLUS sink shall have the structure defined in subclause 4.4.1 of 3GPP TS 29.501 [21], i.e.:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [21].

-	The {apiName} shall be "flus".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificResourceUriPart} shall be set as described in this clause.

The apiPath is defined as {apiName}/{apiVersion}/{apiSpecificResourceUriPart}.

The set of resources are defined in the following table:

Table 7.1.1-1: Resources for managing FLUS sessions at FLUS sink

| Resource Name | Resource Type | Description |
| --- | --- | --- |
| sinks | Collection resource | Allows discovery of available Sinks as described in clause 7.2. The resource server of a Sinks Resource is discovery server. |
| capabilities | Instance resource | Represents a single FLUS sink as described in clause 7.1.1.1. The FLUS source can query the properties of that sink. However, a FLUS source cannot create, update, or terminate a FLUS sink. The creation, update, and termination of FLUS sinks are controlled exclusively by the network |
| session | Instance resource | Represents a single FLUS session resource as described in clause 7.1.1.2. The FLUS source can provision or modify a single session at the FLUS sink. |
| sessions | Collection Resource | Represents a collection of FLUS session resources. |

The logical relationship between the above resources in show in Figure 7.1.1-1.

Sinks

- Sink 1

- Sink 2

…

- Sink i

…

- Sink N

- Capabilities

- Sessions

- Session 1

- Session 2

…

- Session M

Figure 7.1.1-1: Logical relationship between FLUS resources

Note that Sinks is an array of Sink objects. Each Sink object contain a URL indicating the location of an object describing its capabilities and sessions. Therefore, the relationship between Sink object and resource showing its capabilities and sessions is logical (shown by dotted arrow in Figure 7.1.1-1).

#### 7.1.1.1	Capabilities Resource

A capabilities resource provides a representation of the capabilities of a FLUS sink. Different properties of capabilities resources represent the capabilities of the corresponding FLUS sink.

Each capabilities resource has the set of properties described in Table 7.1.1.1-1.

Table 7.1.1.1-1: Properties of Sink Resource

| Property Name | Description | Example Values |
| --- | --- | --- |
| capabilities | List of supported features and instantiations by the FLUS sink. Each capability is to be expressed using an object element of an array. The object has the following attributes: - A scheme URN to identify the capability - An optional location URL, from which a description for the capability can be retrieved. The format of description is defined by the scheme URN.. | { “scheme” : urn:vnd:xzy:capability-name, “location” : “http://vnd.com/xzy/capability-name” } |

As described in Table 7.1.1.1-1 above, each capabilities resource describes the capabilities of the corresponding FLUS sink. A FLUS source can retrieve the capabilities resource description of a FLUS sink and make a decision if it wants to use the corresponding FLUS sink as described using the capability exchange procedure in section 7.3.

#### 7.1.1.2	Session Resource

A Session resource provides the representation of a session between a FLUS source and a FLUS sink. A FLUS source can create a session at a FLUS sink using REST API methods operating on the session resource described in this section.

Each session resource has the set of properties described in Table 5.3.6.

FLUS sources use the above session resource representation to create and manage sessions at the selected FLUS sink. Different procedures that operate on the sink and session resources are discussed in the following sections.

### 7.1.2	Supported Methods

### The F-C API follows the RESTful design principles. All operations shall be performed using HTTP 1.1 (IETF RFC 7231 [6]) over TLS (3GPP TS 33.246 [7]).

Table 7.1.2-1: Supported Methods

| HTTP Method | CRUD | Resource | {apiPath} |
| --- | --- | --- | --- |
| POST | Create | Session | /flus/v1.0/sessions |
| GET | Read | Session | /flus/v1.0/sessions/{session-res-id} |
| GET | Read | Sinks | /flus/v1.0/sinks |
| GET | Read | Capabilities | /flus/v1.0/capabilities |
| PUT | Replace | Session | /flus/v1.0/sessions/{session-res-id} |
| PATCH | Modify | Session | /flus/v1.0/sessions/{session-res-id} |
| DELETE | Delete | Session | /flus/v1.0/sessions/{session-res-id} |

### 7.1.3	Error Handling

The Uplink Streaming Interface API shall use the HTTP status codes to indicate any errors that might occur in the processing of operations on FLUS resources. Unless defined otherwise, the HTTP status codes shall be interpreted as specified in IETF RFC 7231 [6]. API operations that are not successfully handled shall not leave the resource at an undefined state. The response should provide sufficient information for a human operator to understand and locate the error.

## 7.2	Discovery

### Example: GET /flus/v1.0/sinks

### The FLUS sink discovery procedure is used by the FLUS source to discover available FLUS sinks that are provided by the operator.

### To query the list of available FLUS sinks, the FLUS source shall use one of the following procedures:

-	The operator provides one or more FLUS sink Discovery Server URLs to the FLUS source. The Control Sources uses the pre-defined URL "apiPath" /flus/v1.0/sinks in an HTTP request to the FLUS Discovery Server URL above. The response shall be a JSON document that represents an array of object containing each the apiRoot to the entry point to the FLUS Sink as value of property "apiRoot" and optionally the capabilities of the FLUS Sinks.

-	DNS discovery of the FLUS sink, using the FQDN flus.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org in the URL of form http://flus.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org/flus/v1.0/sinks/

An example json document, describing two FLUS sinks is

[

{"apiRoot" : "https://sink1.example.com/api", "capabilities" : ["urn:vnd:xzy:capability-name"] },

{"apiRoot" : "https://sink2.example.com"}

]

## 7.3	Capability retrieval

Example: GET /flus/v1.0/capabilities

### Using this capability exchange procedure, the FLUS source enquires about the supported features and instantiations of the FLUS sink.

### To retrieve the capabilities of a FLUS sink, the Control Source shall use the HTTP GET method on the " capabilities" instance resource as follows:

-	the request URI with the "apiPath" is set to "/flus/v1.0/capabilities"

-	the Host field is set to the FQDN of the FLUS sink

### The URL in the request is set according to the information retrieved using the Discovery procedure described in section 7.2.

The capabilities resource information provides in detail the capabilities of the FLUS sink. The response from the Control Sink to the Control Source shall contain the following:

-	The Content-Type field specifying the MIME type (JSON) using which the capabilitiesresource information is encoded.

-	The Content-Length is the length of the content body.

The above header fields are followed by the content body in the format indicated by the Content-Type field. The content body includes the detailed representation of the capabilities resource as described in section 7.1.1.1 from which the capabilities of that FLUS sink can be inferred.

The possible response messages from the Control Sink to the Control Source are shown in Table 7.3-1.

Table 7.3-1: Response status code, message, and contents for sink resource retrieval using HTTP GET

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink provides the sink capabilities to the FLUS source. |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| Note: In addition to the above response codes, the Control Source can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.4	Uplink streaming configuration

### 7.4.1	FLUS session properties fetch procedure

Example: GET /flus/v1.0/sessions/{session-res-id}

Sessions can be read by the FLUS source when it wishes to know the latest representation of the session resource at the FLUS sink. To fetch the properties of a FLUS session, the Control Source sends a HTTP GET request to the Control Sink as follows:

-	the request URI with the "apiPath" set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

### The {session-res-id} in the request URI is the session resource identifier of the session previously created between the FLUS source and the FLUS sink.

### Upon receiving HTTP GET request from the Control Source, the Control Sink checks to see if such a session exists that matches the given session resource identifier. If such a session exists, the Control Sink shall respond to the Control Source with a 200 OK message along with the complete representation of the session resource. The response shall contain the following:

-	The Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	The Content-Length is the length of the content body.

### The content body of this response message shall be the representation of the session resource as described in sub clause 7.1.1.2

Alternatively, if such a session cannot be found by the Control Sink, it shall send a 404 Not Found message to the Control Source. If the request cannot be fulfilled, the Control Sink shall send a 403 Forbiddent message to the Control Source.

The possible response messages from the FLUS sink, depending on whether the GET request is successful or unsuccessful, are shown in Table 7.4.1-1.

Table 7.4.1-1: Response status code, message, and contents for service modification using HTTPS GET

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink provides session properties of the session resource to the FLUS source |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the Control Sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

### 7.4.2	FLUS session update procedure

The sessions at the FLUS sink can be either partially or completely modified by using the procedures as specified in clauses 7.4.2.1 and 7.4.2.2, respectively.

#### 7.4.2.1	Partial modification of FLUS session

Example: PATCH /flus/v1.0/session/{session-res-id}

To update some of the FLUS session parameters at the sink, the Control Source sends an HTTP PATCH request as follows:

-	the request URI with the "apiPath" is set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

-	The Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	The Content-Length is the length of the content body.

### The {session-res-id} in the request URI is the session resource identifier of the session whose modification is sought.

The content body of the HTTP PATCH message shall contain the updated partial representation of the session resource.

Upon receiving the HTTP PATCH request from the Control Source, the Control Sink checks to see if such a session exists at the sink. If such a sink exists, Control Sink may update the session properties based on the values from the incoming request. Upon successful update of the requested session, the Control Sink shall respond to the Control Source with a 200 OK success message indicating that the session was successfully updated. The Control Sink shall also include the session resource identifier of the session that is updated. As alternative to the 200 OK message, Control Sink may send a 204 No Content success message without any message content to the Control Source. If the session cannot be updated, the Control Sink shall send a 403 message. If the session is not found, the Control Sink shall send a 404 message.

The possible response messages from the Control Sink, depending on whether the PATCH request is successful or unsuccessful, are shown in Table 7.4.2.1-1.

Table 7.4.2.1-1: Response status code, message, and contents for session modification using HTTP PATCH

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink provides the session resource identifier of the session that is modified |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the Control Sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

#### 7.4.2.2	Full modification of FLUS session

Example: PUT /flus/v1.0/sessions/{session-res-id}

For complete update of the session parameters at the FLUS sink, the Control Source sends an HTTP PUT request as follows:

-	the request URI with the "apiPath" is set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

-	The Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	The Content-Length is the length of the content body.

### The {session-res-id} in the request URI is the session resource identifier of the session whose modification is sought.

The content body of the HTTP PUT message shall contain the updated complete representation of the session resource.

Upon receiving the HTTP PUT request from the Control Source, the Control Sink checks to see if such a session exists at the sink. If such a sink exists, Control Sink may update the session properties based on the values from the incoming request. Upon successful update of the requested session, the Control Sink shall respond to the Control Source with a 200 OK success message indicating that the session was successfully updated. The Control Sink shall also include the session resource identifier of the session that is updated. As alternative to the 200 OK message, Control Sink may send a 204 No Content success message without any message content to the Control Source. If the session cannot be updated, the Control Sink shall send a 403 message. If the session is not found, the Control Sink shall send a 404 message.

The possible response messages from the FLUS sink, depending on whether the PATCH request is successful or unsuccessful, are shown in Table 7.4.2.2-1.

Table 7.4.2.2-1: Response status code, message, and contents for session modification using HTTP PATCH

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink shall send the session resource identifier of the session that is modified |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the Control Sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.5	Session establishment

Example: POST /flus/v1.0/sessions

To create a session at the FLUS sink, the Control Source shall use the HTTP POST method on the "sessions" collection resource as follows:

-	the request URI with the "apiPath" is set to "/flus/v1.0/sessions"

-	the Host field is set to the FQDN of the FLUS sink

-	the Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	the Content-Length is the length of the content body.

The content body of the POST request shall contain the representation of the session resource. The session resource representation includes the details such as the Media streams, their formats and codecs, the relationship between them, and the selected user plane instantiation and user plane control protocol.

Upon receipt of HTTP POST to create a session, the Control Sink shall create a session. Upon successful creation of session resource, the Control Sink shall respond back to the Control Source with a 201 success message indicating that the session is successfully created along with the session resource of the created session. The session information returned to the Control Source along with the 201 message includes the parameters applied for the session, thus confirming the parameters, and any default values assigned to session parameters which were not supplied by the Control Source in the HTTP POST message.

The session resource identifier is the identifier that uniquely identifies the session within the list of all sessions at that sink. When the Control Source receives the session resource identifier, it shall use this identifier in subsequent requests to the Control Sink to refer to this session.

Alternatively, if the creation of session is failed, the Control Sink shall send a 403 message.

The possible response messages from the Control Sink, depending on whether the POST request is successful or unsuccessful, are shown in Table 7.5-1.

Table 7.5: Response status code, message, and contents for session creation

| Status Code | Message | Contents |
| --- | --- | --- |
| 201 Created | Session created successfully | The FLUS sink provides the session resource identifier of the created session and detailed session information |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| Note: In addition to the above response codes, the Control Sink can also send appropriate response codes described in IETF RFC 7231 [11] as applicable |  |  |

## 7.6	Session termination

Example: DELETE /flus/v1.0/sessions/{session-res-id}

To terminate a FLUS session, the Control Source shall use the HTTP DELETE method on the "session" instance resource as follows:

-	the request URL with the "apiPath" is set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

The {session-res-id} is the session resource identifier for which the session termination is sought.

Upon reception of a HTTP DELETE request, the Control Sink will check to see if the session with the given session resource identifier exists at the given sink. If such a session exists, the Control Sink shall delete the session instance associated with the given session resource identifier. Further, the Control Sink responds back to the Control Source with a 200 success message along with the session resource identifier indicating that the session was successfully deleted. Alternately, if the session cannot be deleted, the Control Sink shall send a 403 Forbidden message. If the session was not found, or if the sink was not found, the Control Sink returns back with a 404 message.

The possible response messages from the Control Sink, depending on whether the DELETE request is successful or unsuccessful, are shown in Table 5.2.2.2.4-1.

Table 7.6-1: Response status code, message, and contents for session deletion

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink shall send the session resource identifier of the session that is deleted |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the Control Sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

**** Last Change ****