3GPP TSG-SA4 Meeting #110-e 	S4-201060

# Electronic meeting, Telco, Aug 19-28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | -- | rev | -- | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Metrics Reporting API for 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Metrics reporting API on M5d interface is currently unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal of metric reporting API for the M5d interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability to support the reporting of QoE metrics from the 5GMS Client to the network in the context of 5G media streaming services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE: Text for clause 11.4

## 11.4		Metrics Reporting API

### 11.4.1	General

The Metrics Reporting API is a RESTful API that allows the Media Session Handler to report QoE metrics information to the 5GMSd AF. The API defines data models, resources and the related procedures for the creation and management of the metrics reporting procedures.

### 11.4.2	Data model

#### 11.4.2.1	Introduction

This clause defines data structures to be used in the metrics reporting procedure.

Table 11.4.2.1-1 specifies data types re-used by the Metrics Reporting API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the 5GMS Metrics Reporting API.

Table 11.4.2.1-1: Re-used Data Types by 5GMS Metrics Reporting API

| Data type | Reference | Comments |
| --- | --- | --- |
| domainNames | 3GPP TS 29.122 [12] | Indicates an FQDN or a regular expression as a domain name matching criteria. |

#### 11.4.2.2	MetricsReportingConfiguration resource

This type represents the configurations parameters pertaining to the measurement, collection and reporting of metrics.

Table 11.4.2.2-1: Definition of MetricsReportingConfiguration resource

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| serverAddress | Array(domainName) | 1..N | A list of 5GMSd AF addresses to which metrics reports shall be sent by the 5GMS Media Session Handler. |
| dNN | String | 0..N | Zero or more Data Network Names (DNN), each of which corresponds to an instance of serverAddress to which metrics reports are to be sent. If not specified, the default DNN shall be used |
| scheme | String | 1..N | A list of metrics schemes, each of which defines its rules regarding metrics measurement, collection and reporting to be performed by the 5GMS Client. |
| reportingInterval | DurationSec | 1..1 | Indication of the sending interval between consecutive metrics reports. |
| samplePercentage | Percentage | 0..1 | The proportion of streaming sessions for which metrics reporting shall be performed. If not specified, reports shall be sent for all sessions. |
| streamingSourceFilter | Array(urlPattern) | 1..N | A list of URL patterns for which metrics reporting shall be performed. |
| metrics | Array(metricsNames) or Object | 1..N | One or more lists of scheme-dependent metrics for measurement, collection and subsequent reporting by the 5GMS Client. For progressive download and DASH streaming services, the listed metrics are associated with the 3GPP metrics scheme and shall correspond to one or more of the metrics names as specified in TS 26.247 [7] clauses 10.3 and 10.4, respectively. Naming and definitions of metrics for non-3GPP defined schemes are outside the scope of this specification. |

#### 11.4.2.3	MetricsReporting resource

This type represents data components in a metrics report instance. This structure is used by the Media Session Handler to report collected metrics from the Media Player.

Table 11.4.2.3-1: Definition of MetricsReporting resource

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| mediaPlayerEntry | String | 1..1 | Identifier of the Media player entry point. In the case of 3GP-DASH, the media player entry point could be a URL of the MPD. |  |
| reportingSessionId | Integer | 1..1 | Identifier of the streaming session to which a metrics report pertains. |  |
| metricsReportingUnit | MetricsReportingUnit | 1..N | Identifier of a list of metrics reporting units. |  |

#### 11.4.2.4	Type: MetricsReportingUnit

This type represents each metrics report unit.

Table 11.4.2.4-1: Definition of type MetricsReportingUnit

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| mediaConsumed | String | 1..N | Identifier(s) of the of or more media types to which the metrics report pertains. In case of DASH, the Representation@id may be used. |  |
| startTime | DateTime | 1..1 | Specifies the start time of measurement of the metrics in this report unit. |  |
| duration | DurationSec | 1..1 | Specifies the duration of the measurement of the metrics in this report unit. |  |

### 11.4.3	Resource structure

#### 11.4.3.1	Introduction

Editor’s Note: URI definition shall be unified in this document. A global section where the URI is defined should be created. This section may define the URI as a global URI. It may also simplify the URI construction in a way that for instance {apiRoot}/3gpp-5gms-metrics-reporting/v1/ would be replaced by {5GMSAmetricsReportApiRoot} which is defined as a global URI by the AF without any constraint on the way this URI is built.
If this section exists, it will impact every Resource structure defined in this section.

All resource URIs of this API shall have the following root:

{apiRoot}/3gpp-5gms-metrics-reporting/v1/

"apiRoot" is set as described in subclause X. "apiName" shall be set to "3gpp-5gms-metrics-reporting " and "apiVersion" shall be set to "v1" for the version defined in the present document. All resource URIs in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods initiated by the 5GMS Client are supported for this API:

Table 11.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Meaning |
| --- | --- | --- | --- |
| Metrics Reporting Session | 3gpp-5gms-metrics-reporting/v1/{aspId} /session/ | GET | Forbidden. |
|  |  | PUT | Create a Metrics Reporting Session resource. |
| Metrics Reporting Configuration Acquisition | 3gpp-5gms-metrics-reporting/v1/{aspId} /session/{sessionId}/acquire | GET | Acquire metrics reporting configuration. |
|  |  | POST | Forbidden. |
| Metrics Reporting Data | 3gpp-5gms-metrics-reporting/v1/{aspId} /session/{sessionId} | GET | Forbidden. |
|  |  | POST | Report of measured QoE metrics (for the time interval as defined by the ‘startTime’ and ‘duration’ attributes of a Metrics Reporting Unit. |

#### 11.4.3.2	Metrics Reporting Session resource

#### 11.4.3.2.1	Introduction

This resource allows the Media Session Handler to create Metrics Reporting Data resources when the Metrics Reporting feature is activated for a corresponding downlink or uplink streaming session.

#### 11.4.3.2.2	Resource definition

Resource URI: {apiRoot}/3gpp-5gms-metrics-reporting/v1/{aspId}/session

This resource shall support the resource URI variables as specified  in Table 11.4.3.2.2-1.

Table 11.4.3.2.2-1: Resource URI variables for resource "Metrics Reporting Session"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [12]. |
| aspId | Identifier of the 5GMS Application Provider. |

#### 11.4.3.2.3	Resource methods

##### 11.4.3.2.3.1	GET

This HTTP method is not supported for this resource.

##### 11.4.3.2.3.2	PUT

The PUT method allows the Media Session Handler to create a Metrics Reporting resource. It is initiated by the Media Session Handler and answered by the 5GMS AF.

This method shall support the request and response data structures, and response codes, as specified in Table 11.4.3.2.3.2-1.

Table 11.4.3.2.3.2-1: Data structures supported by the PUT request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | MetricsReporting | 0..1 | 201 OK | The reported metrics parameters may be returned by the 5GMS AF. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.6-1 of TS 29.122 [12] also apply. |  |  |  |  |

##### 11.4.3.2.3.3	PATCH

This HTTP method is not supported for this resource.

##### 11.4.3.2.3.4	POST

This HTTP method is not supported for this resource.

##### 11.4.3.2.3.5	DELETE

This HTTP method is not supported for this resource.

#### 11.4.3.3	Metrics Reporting Configuration Acquisition resource

##### 11.4.3.3.1	Introduction

This method allows the Media Session Handler to acquire metrics reporting configuration parameters when the Metrics Reporting feature is activated for a corresponding downlink or uplink streaming session.

##### 11.4.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-5gms-metrics-reporting/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables as specified in Table 11.4.3.3.2-1.

Table 11.4.3.3.2-1: Resource URI variables for resource "Metrics Reporting Configuration Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [12]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

##### 11.4.3.3.3	Resource methods

###### 11.4.3.3.3.1	GET

The GET method allows the Media Session Handler to acquire metrics reporting configuration parameters. It is initiated by the Media Session Handler and answered by the 5GMS AF.

This method shall support the request and response data structures, and response codes, as specified in Table 11.4.3.3.3-1.

Table 11.4.3.3.3-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | MetricsConfiguration | 1..1 | 200 OK | The metrics reporting configuration parameters shall be returned by the 5GMS AF. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in Table 5.2.6-1 of TS 29.122 [12] also apply. |  |  |  |  |

###### 11.4.3.3.3.2	PUT

This HTTP method is not supported for this resource.

###### 11.4.3.3.3.3	PATCH

This HTTP method is not supported for this resource.

###### 11.4.3.3.3.4	POST

This HTTP method is not supported for this resource.

###### 11.4.3.3.3.5	DELETE

This HTTP method is not supported for this resource.

#### 11.4.3.4	Metrics Reporting Data resource

##### 11.4.3.4.1	Introduction

Editor’s Note: Whether reports should be sent as RESTful resources is still under consideration

This resource allows the Media Session Handler to send the actual metrics data when the Metrics Reporting feature is activated for a corresponding downlink or uplink streaming session.

##### 11.4.3.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-5gms-metrics-reporting/v1/{aspId}/session/{sessionId}

This resource shall support the resource URI variables as specified in Table 11.4.3.4.2-1.

Table 11.4.3.4.2-1: Resource URI variables for resource "Metrics Reporting Data"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [12]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS downlink or uplink streaming session to which the metrics report applies. |

##### 11.4.3.4.3	Resource methods

###### 11.4.3.4.3.1	GET

This HTTP method is not supported for this resource.

###### 11.4.3.4.3.2	PUT

This HTTP method is not supported for this resource.

###### 11.4.3.4.3.3	PATCH

This HTTP method is not supported for this resource.

###### 11.4.3.4.3.4	POST

The POST method allows the Media Session Handler to send metrics reporting data. It is initiated by the 5GMS Media Session Handler and acknowledged by the 5GMS AF.

This method shall support the request and response data structures, and response codes, as specified in Table 11.4.3.4.3.4-1.

Table 11.4.3.4.3.4-1: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | MetricsReporting | 1..1 | Metrics Reporting data to be sent to the 5GMS AF. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | none |  | 200 OK | The metrics reporting data is successfully received by the 5GMS AF. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.6-1 pf TS 29.122 [12] also apply. |  |  |  |  |

###### 11.4.3.4.3.5	DELETE

This HTTP method is not supported for this resource.

END OF  CHANGE