3GPP TSG SA WG4 Meeting #110e 19th – 28th August 2020 	S4-201140

# 19th – 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on consumption reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Enensys Technologies, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Updates to harmonise the Consumption Reporting API with the current specification status. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a periodic polling procedure for refresh Service AcessInformation - Add a datatype for location - Supress duplicate information in 11.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification broken. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2.3;4.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 4.7.2.3	Read Service Access Information properties

This procedure shall be used by the Media Session Handler to acquire Service Access Information from the 5GMSd AF. The Media Session Handler uses the GET method for this purpose.

The downlink streaming session for which the Media Session Handler is requesting data is identified by a unique reference contained in the path of the URL, as specified in clause 5.x.3.

Once it has obtained an initial set of Service Access Information, the Media Session Handler shall periodically check for updated Service Access Information by issuing a conditional HTTP GET request containing either

- an If-None-Match request header with the value of the entity tag (ETag) that was returned with the most recently acquired ServiceAccessInformation resource, or else

- an If-Modified-Since request header with the Last-Modified value of that most recently acquired resource.

The periodicity of polling for updated Service Access Information shall be guided by the value of the Expires and/or Cache-control: max-age headers that shall be included along with every response message for this procedure.

Next Change

### 4.7.4	Procedures for consumption reporting

These procedures are used by the Media Session Handler and the Consumption Reporting functions of the 5GMSd Client to submit a consumption report via the M5d interface if Consumption Reporting is applied for a downlink streaming session.

The Service Access Information indicating whether Consumption Reporting is provisioned for downlink streaming sessions is described in clause 11.2.3. When the ClientConsumptionReportingConfiguration.samplePercentage value is 100, the Media Session Handler shall activate the consumption reporting procedure. If the samplePercentage is less than 100, the Media Session Handler shall generate a random number which is uniformly distributed in the range of 0 to100, and the Media Session Handler shall activate the consumption report procedure when the generated random number is of a lower value than the samplePercentage value.

Editors’note: -Missing text that will describe M6d/M7d APIs--

If the consumption reporting procedure is activated, the Media Session Handler shall submit a consumption report to the 5GMSd AF when any of the following conditions occur:

- Start of consumption of a downlink streaming session;

- Stop of consumption of a downlink streaming session;

- Upon determining the need to report ongoing 5GMS consumption at periodic intervals determined by the ClientConsumptionReportingConfiguration.reportingInterval property.

- Upon determining a location change, if the ClientConsumptionReportingConfiguration.locationReporting property is set to True.

Whenever a consumption report is sent, the Media Session Handler shall reset its reporting interval timer to the value of the reportingInterval property and it shall begin countdown of the timer again. Whenever the Media Session Handler stops the consumption of a downlink streaming session, it shall disable its reporting interval timer.

In order to submit a consumption report, the Media Session Handler shall send an HTTP POST message to the 5GMSd AF. If several 5GMSd AF addresses are listed in the ClientConsumptionReportingConfiguration.‌serverAddresses array (see table 11.2.3.1-1), the Media Session Handler shall choose one and send the message to the selected. The request body shall be a ConsumptionReport resource, as specified in clause 11.3.3.1.The server shall respond with a 200 (OK) message to acknowledge successful processing of the consumption report.

The Consumption Reporting API, defining the resources, data models and related procedures for consumption reporting, is described in clause 11.3.

Next Change

## 6.2	Usage of HTTP

All responses from the 5GMSd/5GMSu Application Function that carry a message body shall include a strong entity tag in the form of an ETag response header and a modification timestamp in the form of a Last-Modified response header.

All endpoints shall support the conditional HTTP requests If-none-Match and If-Modified-Since.

Next Change

### 6.4.2	Simple data types

Table 6.4.2-1 below specifies common simple data types used within the 5GMS APIs, including a short description of each. In cases where types from other specifications are reused, a reference is provided.

Table 6.4.2-1: Simple data types

| Type name | Type definition | Description | Reference |  |
| --- | --- | --- | --- | --- |
| Percentage | number | A percentage expressed as a floating point value between 0.0 and 100.0 (inclusive). |  |  |
| DurationSec | integer | An unsigned integer identifying a period of time expressed in units of seconds. | TS 29.122 [12] Table 5.2.1.3.2-2 |  |
| DateTime | string | An absolute date and time expressed using the OpenAPI date-time string format. | TS 29.122 [12] Table 5.2.1.3.2-2 |
| LocationType | integer | Identify the type of location used. CGI, ECGI and NCGI shall be represented by the values 0, 1 and 2, respectively. | TS 23.003 [7] |

Next Change

## 11.3	Consumption Reporting API

### 11.3.1	General

The Consumption Reporting API allows the Media Session Handler to report media consumption to the 5GMSd AF. This procedure is configured by the ServiceAccessInformation resource, as defined in clause 11.2.3.

### 11.3.2	Resource structure

Consumption reports shall be submitted to one of the URLs selected from the ClientConsumptionReportingConfiguration[serverAddresses] array of the ServiceAccessInformation resource (see clause 11.2.3). The path of the URL shall conform to the following general format:

{apiRoot}/3gpp-m5d/v1/consumption-reporting/{aspId}

where {aspId} shall be substituted by the 5GMS Client with the relevant Application Service Provider identifier.

The only HTTP method supported by this endpoint is POST.

### 11.3.3	Data model

| zxasdasasdasdasdasdasdasfsfkjk |  |  |
| --- | --- | --- |

#### 11.3.3.1	ConsumptionReport resource

This data structure is used by the Media Session Handler to report media consumption.

Table 11.3.3.1.-1: Definition of ConsumptionReport resource

| Property | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| mediaPlayerEntry | string | 1..1 | Identifies the Media player entry. In the case of DASH, the media player entry pointer shall be the URL of the MPD. |
| reportingClientId | string | 1..1 | Identifies the identifier of the UE that consumes data. The client ID can be a MSISDN. |
| locationType | LocationType | 0..1 | Identify the UE location type if location reporting is enabled (only for trusted AF). |
| location | string | 0..1 | Identify the UE location where the consumption media if location reporting is enabled (only for trusted AF). |
| consumptionReportingUnits | Array(ConsumptionReportingUnit) | 1..1 | An array of consumption reporting units. |

#### 11.3.3.3	ConsumptionReportingUnit type

This type represents a single consumption reporting unit.

Table 11.3.3.3-1: Definition of type ConsumptionReportingUnit

| Property | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| mediaConsumed | string | 1..1 | Identifies the media consumed. In the case of DASH, the value of the Representation[@id] attribute shall be quoted. |
| startTime | DateTime | 1..1 | The time when this consumption reporting unit started. |
| duration | DurationSec | 1..1 | The duration of this consumption reporting unit. |

End of changes