3GPP TSG-SA4 Meeting #111e	SA4-201414

e-Meeting, 11-20 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.928 | CR | xx | rev | 0 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use Case Update for AR/MR Device Type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics. Co., Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GXR |  |  |  |  |  | Date: |  |  | 2020-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Recently, several use cases in TR26.928 were adopted to FS_5GSTAR as the relevant use cases for AR/MR glass-type devices. However, the descriptions of such use cases in TR26.928 do not mention the appropriate device type of AR/MR glasses. In addition, more industrial exercises to the relevant use case are added in the feasibility sections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Device type of XR5G-AX and the feasibility are to be added for relevant use cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion that some use cases are not applicable to FS_5GSTAR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10, A.2, A.3, A.8, A.9, A.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

4.10	XR Use Cases

In Annex A of this document, a significant amount of use cases are collected that serve for identifying potential interfaces, formats, protocols and requirements for the 5G system in order to support XR applications.

Table 4.10 provides an overview of the use cases and their characterization.

In addition, this table more explicitly adds the device types that have been developed in clause 4.8.

Table 4.10: Overview of Use cases as documented in Annex A

| No | Use Case | Type | Experience | Delivery | Device Types |
| --- | --- | --- | --- | --- | --- |
| 1 | 3D Image Messaging | AR | 3DoF+, 6DoF | Upload and Download | XR5G-P1 XR5G-AX |
| 2 | AR Sharing | AR, MR | 6DoF | Local, Messaging Download and Upload | XR5G-P1 XR5G-AX |
| 3 | Streaming of Immersive 6DoF | VR | 3DoF+, 6DoF | Streaming Interactive Split | XR5G-V3 or XR5G-V4 with controller |
| 4 | Emotional Streaming | 2D, AR and VR | 2D, 3DoF+, 6DoF | Streaming Interactive, Split | XR5G-P1 XR5G-V3 XR5G-V4 |
| 5 | Untethered Immersive Online Gaming | VR | 6DoF | Streaming, Interactive, Split | XR5G-V3 XR5G-V4 with gaming controller |
| 6 | Immersive Game Spectator Mode | VR | 6DoF | Streaming, Split | XR5G-P1 XR5G-V3 XR5G-V4 |
| 7 | Real-time 3D Communication | 3D, AR | 3DoF+ | Conversational | XR5G-P1 XR5G-AX |
| 8 | AR guided assistant at remote location (industrial services) | 2D video with dynamic AR rendering of graphics | 6DoF (2D + AR) | Local, Streaming, Interactive, Conversational | XR5G-P1 XR5G-AX |
| 9 | Police Critical Mission with AR | AR, VR | 3DoF to 6DoF | Local, Streaming, Interactive, Conversational, Group Communication | XR5G-A3 XR5G-A4 |
| 10 | Online shopping from a catalogue – downloading | AR | 6DoF | Download | XR5G-P1 XR5G-AX |
| 11 | Real-time communication with the shop assistant | AR | 6DoF | Interactive, Conversational | XR5G-P1 XR5G-AX |
| 12 | 360-degree conference meeting | AR, MR, VR | 3DoF | Conversational | XR5G-P1 XR5G-V3 XR5G-V4 |
| 13 | 3D shared experience | AR, MR, VR | 3DoF+ 6DoF | Conversational | XR5G-P1 XR5G-V3 XR5G-V4 |
| 14 | 6DOF VR conferencing | VR | 6DoF | Interactive, Conversational | XR5G-V3 XR5G-V4 |
| 15 | XR Meeting | AR, VR, XR | 6DoF | Interactive Conversational | XR5G-P1 XR5G-A1 XR5G-A2 XR5G-A5 |
| 16 | Convention / Poster Session | AR, VR, MR | 6DoF | Interactive Conversational | XR5G-P1 XR5G-A1 XR5G-A2 XR5G-A5 |
| 17 | AR animated avatar calls | AR | 2D, 3DoF | Conversational | XR5G-P1 XR5G-A1 XR5G-A2 XR5G-A5 |
| 18 | Online shopping from a catalogue – downloading | AR | 6DoF | Download | XR5G-P1 XR5G-A1 XR5G-A2 XR5G-A5 |
| 19 | Front-facing camera video multi-party calls | AR | 3DoF | Conversational | XR5G-P1 XR5G-AX |
| 20 | AR Streaming with Localization Registry | AR, Social AR | 6DoF | Streaming, Interactive, Conversational | XR5G-A1 XR5G-A2 XR5G-A5 |
| 21 | Immersive 6DoF Streaming with Social Interaction | VR and Social VR | 3DoF+, 6DoF | Streaming Interactive Conversational Split | XR5G-V3 XR5G-V4 |
| 22 | 5G Online Gaming Party | VR | 6DoF | Streaming, Interactive, Split, D2D | XR5G-V3 XR5G-V4 |
| 23 | Spatial Shared Data | AR | 6DoF | Streaming Interactive Conversational Split | XR5G-AX |

The use cases are summarized in clause 5 into several core use cases and scenarios.

Second Change

A.2	Use Case 1: 3D Image Messaging

Use Case Description: 3D Image Messaging

Alice uses her phone that is equipped with a depth camera to capture an image of a statue in 3D. The phone captures a set of images and builds a 3D model of the object. After a few seconds the 3D image is ready to share and Alice sends the image to Bob as an MMS message.

Categorization

Type: AR Degrees of Freedom: 3DoF+ or 6DoF Delivery: Upload and Download Device: Phone, AR glasses

Preconditions

- Phone is equipped with 3D capture capabilities, such as depth camera or a stereo camera on the back of the phone, possibly supported by an app for processing multiple images. - Phone is equipped with a 3D image viewer

Requirements and QoS/QoE Considerations

- QoS: Reliable delivery of a File of a few MByte distributed over MMS - QoE: Quality of the 3D object representation, level of details

Feasibility

Some new smartphone releases are equipped with a Time of Flight (ToF) depth camera (see for example https://en.wikipedia.org/wiki/Time-of-flight_camera) that can be used to build accurate 3D models of objects of interest. Compared to structured light cameras, ToF do not require a large baseline to achieve good depth accuracy. Applications such as 3D Photo are using the stereo camera on the back of some phone models to generate a 3D model using a set of pictures taken consecutively. To compensate for the small baseline, complex processing (e.g. deep model to reconstruct the depth map) may be required. The 3D image can be stored as a point cloud, a mesh, or a layered image. The content maybe compressed to reduce the message size. The content is identified through its mime type and can be embedded with other content such as text.

Potential Standardization Status and Needs

The following aspects may require standardization work: - Standardized formats for 3D images, e.g. meshes, point clouds, and/or depth-layered images - Extensions to MMS to support 3D images

Third Change

A.3	Use Case 2: AR Sharing

Use Case Description: AR Sharing

Alice is shopping for a new couch at the furniture store close to her. Alice finds a couch that she likes and wants to check Bob's opinion who sits back home. Alice scans a QR code with her phone to download a 3D model of the couch and sends it to Bob via MMS. Bob places the virtual model of the couch on a plane surface in the living room. Bob likes how the couch fits in their living room and captures a 3D picture of the room with the couch and shares it with Alice.

Categorization

Type: AR, MR Degrees of Freedom: 6DoF Delivery: Local, Messaging Download and Upload Device: Phone, AR glasses

Requirements and QoS/QoE Considerations

- QoS: Reliable Delivery (Upload and Download) of a File of a few or several MByte - QoE: Quality of the 3D object representation, level of details

Preconditions

- Bob's smartphone has support for AR technology

Feasibility

Modeling of sale items in 3D will be increasing. This will facilitate purchase decisions for millions of customers. Texture of the 3D models may vary to reflect available choices for the item. A user can use ARCore [4] or ARKit [5] to detect flat surfaces and place the 3D model on it. The AR scene can be captured with the real scene in the background and the 3D object in the foreground. To achieve physically-based rendering (PBR), additional characteristics of the 3D object's texture are stored. These may include properties such as specular, diffuse, transparency, reflectivity, etc. An example of AR sharing experience was implemented by using MS HoloLens 2 TM and Quest 2TM: https://www.youtube.com/watch?v=Rf_6etOJXfE&feature=youtu.be

Potential Standardization Status and Needs

The following aspects may require standardization work: - Standardized format for 3D objects is needed - Standardized format for mixed reality 3D scenes is needed - Extensions to MMS to support sharing of 3D objects and scenes

Fourth Change

A.8	Use Case 7: Real-time 3D Communication

| Use Case Description: Real-time 3D Communication |  |
| --- | --- |
| Alice uses her mobile phone to start a video call with Bob. After the call starts, Alice sees a button on her screen that reads "3D". Alice clicks on the button to turn on the 3D mode on the video call app. Bob is able to see Alice's head in 3D and he uses his finger to rotate the view and look around Alice's head. Bob may not be able to see the full head or may see a reconstructed model of it (e.g. based on a pre-captured model). Alice is able to apply a selected set of 3D AR effects to her 3D head (e.g. putting a hat or glasses). |  |
| Categorization |  |
| Type: 3D Real-time communication, AR Degrees of Freedom: 3DoF+ Delivery: Conversational Device: Phone, AR glasses |  |
| Preconditions |
| - Alice's phone is equipped with 3D capture capabilities, such as front depth camera - Bob's phone can receive a proper 3D object in real-time and apply the facial expressions during the rendering |
| Requirements and QoS/QoE Considerations |  |
| - QoS: - conversational QoS requirements - sufficient bandwidth to delivery compressed 3D objects, e.g. point cloud compression - QoE: - Quality of the 3D object representation, level of details - Quality of facial expressions |  |
| The following requirements are considered: - High quality, very low delay 3D reconstruction of Head/Face, e.g. resolution of the 3D head representation measured in number of points or polygons |  |
| Feasibility |  |
| Advances in image and video processing together with the proliferation of front-facing depth sensors are going to enable real-time reconstruction of the call participants. To run in real-time, extensive hardware capabilities are required, such as multi-GPU or Tensor Processing Unit (TPU) processing. These operations may be performed in the network, e.g. by a media gateway or a dedicated processing engine. The representation of the call participant's head can be done in Point Cloud format to avoid the expensive Mesh reconstruction operation. |  |
| Potential Standardization Status and Needs |  |
| The following aspects may require standardization work: - Extension of the MTSI service to support dynamic 3D objects and their formats |  |

Fifth Change

A.9	Use Case 8: AR guided assistant at remote location (industrial services)

Use Case Description: AR guided assistant at remote location (industrial services)

- Pedro is sent to fix a machine in a remote location. - Fixing the machine requires support from a remote expert. - Pedro puts his AR 5G glasses on and turns them on. He connects to the remote expert, who uses a tablet or a touch-screen computer, or uses AR glasses, headphones, as well as a gesture acquisition device that is connected and coordinated with his glasses. - The connection supports conversational audio and Pedro and the expert start a conversation. - Pedro's AR 5G glasses support accurate positioning and Pedro's position is shared live with the expert such that he can direct Pedro in the location. - The AR 5G glasses are equipped with a camera that also has depth capturing capability. - Pedro activates the camera such that the expert can see what Pedro is viewing. - The expert can provide guidance to Pedro via audio but also via overlaying graphics to the received video content, by activation of appropriate automatic object detection from his application, and via drawing of instructions as text and/or graphics and via overlaying additional video instructions. In the case that the expert uses AR glasses, the expert can also identify the depth of the video sent by Pedro and more accurately place the overlay text or graphics. - The overlaid text and/or graphics are sent to Pedro's glasses and they are rendered to Pedro such that he receives the visual guidance from the expert on where to find the machine and how to fix it. - Note: the video uplink from Pedro's glasses might be "jumpy" as Pedro moves his head. A second camera and corresponding video uplink to show an overview video of Pedro and the machinery or alternatively a detailed video of the machinery functioning, is a help to the expert when performing this type of service.

Categorization

Type: AR Degrees of Freedom: 2D video with dynamic AR rendering of graphics (6DoF) Delivery: Local, Streaming, Interactive, Conversational Device: 5G AR Glasses, 5G touchscreen computer or tablet

Preconditions

Pedro has AR Glasses with the following features - 5G connectivity - Support for conversational audio - Positioning (possibly even indoor) - Camera with depth capturing - Rendering of overlay graphics - Rendering of overlay video The remote expert has a tablet or touch-screen device (with peripheries) with the following features - Securily connected to Pedro - Headphones - Gesture acquisition - Composition tools to support Pedro - Access to a second stationary camera that is provides synchronized video to Pedro's uplink traffic

Requirements and QoS/QoE Considerations

QoS: - conversational QoS requirements - sufficient bandwidth to delivery compressed 3D objects, e.g. point cloud compression - Accurate user location (indoor/outdoor) (to find machine or user location) QoE: - For Pedro: - Fast and accurate rendering of overlay graphics and video - Synchronized rendering of audio and video/graphics - For remote expert: - High-quality depth video captured from Pedro's device - Synchronized and good video signal from second camera - Synchronized voice communication from Pedro - Accurate positioning information

Feasibility

- Vuzix Blade ™ AR glasses with WiFi connectivity to a smartphone with 4G connectivity - Specific applications. For example: - https://www.vuzix.com/appstore/app/gemvision (Remote assistance for hands-on workforce) - https://play.google.com/store/apps/details?id=com.utilityar.workflow (Remote Adviser from Utility AR) - https://www.youtube.com/watch?v=d3YT8j0yYl0 (Dynamics 365 Remote Assist + MS HoloLens 2™) - https://www.youtube.com/watch?v=lzYg32ngWmU&t=9s (Assistance from virtual AR trainer)

Potential Standardization Status and Needs

- 5G connectivity: Release-15 and Release-16 3GPP standardization - 5G positioning: ongoing 3GPP standardization – API required for sharing with low latency - MTSI regular audio between Pedro and expert - MTSI 2D video call from Pedro to expert, potentially a second video source as help for the expert. - Pedro received video + graphics (manuals, catalogs, manual indications from the expert, object detection) + overlaid video rendering either in the network or locally - Synchronization of different capturing devices - Coded Representations of 3D depth signals and delivery in MTSI context

Sixth Change

A.20	Use Case 19: Front-facing camera video multi-party calls

Use Case Name

Front-facing camera video multi-party call

Description

This use case is based on front-facing camera calls, i.e. a user is having a video call, seeing the other participants on the display of e.g. a smartphone he holds at arm's length. The use case has some overlap with UC 6 (AR face-to-face calls) and UC 10 (Real-time 3D Communication), extended by spatial audio rendering for headphones/headsets. The spatial audio rendering is based on the head-tracker data extracted from the smartphones front-facing camera, giving a user the impression, even with movements, that the voice of the other participants originates from a virtual stage in the direction of the phone with the video of the other's faces. A potential user experience is described as a user story: Bob, Jeff, and Frank are back in New York City and each of them is walking to work. They just have their smart phones with a front-facing camera and a small headset, allowing the real world to be augmented with audio. They start a multi-party video call to discuss the plans for the evening, where each of them gets the other two friends displayed on the phone and can hear the audio, coming from the direction on the horizontal plane where the phone is placed in their hand and some small spread to allow easy distinction. While they walk around in the streets of New York, they have a continuous voice call with their phones at arm's length, with the, potentially cut-out, faces of their pals displayed on their phones. For Bob the acoustic front is always in the direction of his phone, thus the remote participants are always in the front. When Bob rotates his head though, the front-facing camera tracks this rotation and the spatial audio is binauralized using the head-tracking information, leaving the position of the other participants steady relative to the phone's position. As Bob turns around a corner with the phone still at arm's length for the video call using the front-facing camera, his friends remain steady relative to the phone's position.

Type: AR Degrees of Freedom: 3DoF Delivery: Conversational Device: Smartphone with front-facing camera, headset, AR glasses

Preconditions

Phone with front-facing camera, motion sensors, and headset (more or less acoustically transparent). Motion sensors to compensate movement of the phone, front-facing camera to capture the video for the call and potentially track the head's rotation.

Requirements and QoS/QoE Considerations

QoS: QoS requirements like MTSI requirements (conversational, RTP), e.g. 5GQI 1 and 2. QoE: Immersive voice/audio and visual experience, Quality of the capturing, coding and rendering of the participant video (potentially cut out faces), Quality of the capturing, coding and rendering of the participant audio, including binaural rendering taking head tracking data into account.

Feasibility

Several multi-party video call applications using the front-facing camera exist, e.g. https://www.cnet.com/how-to/how-to-use-group-facetime-iphone-ipad-ios-12/ , https://faq.whatsapp.com/en/android/26000026/?category=5245237 Head tracking using cameras exists, e.g. https://xlabsgaze.com Binaural rendering with head-tracking also exists (see also TS26.118)

Potential Standardization Status and Needs

Visual coding and transmission of video recorded by front-facing camera; potentially cut-out heads, alpha channel coding Audio coding and transmission for streams from all participants NOTE: scene composition is usually a differentiating factor