3GPP TSG-SA4 Meeting #111-e	S4-201500

# Electronic meeting, 11th-20th Nov. 2020 												rev of S4-201450

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR |  | rev |  | Current version: | 0.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Screen content encoding configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 15th Sept. 2020 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Contribution addressing the encoding configurations fo Screen Content scenario for both AVC and HEVC |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of  first Change:

### 6.4.8	Test Sequences

The test sequences illustrating the screen content scenario are described in Annex C. They contain either synthetic content from a presentation such as a slide deck with text and graphics.

### 6.4.9	Detailed Test Conditions

#### 6.4.9.1	Overview

The screen content configuration relies on low delay encoding modes with the optional use of Screen Content encoding tools when possible (foe HEVC).

#### 6.4.9.2 Reference Software H.264 (AVC)

The bitstreams are generated with the H.264 (AVC) reference software called the JM (Joint model) in version 19.0.

-	http://iphome.hhi.de/suehring/tml/download/

The encoder configuration file is:

-	https://vc-git.hhi.fraunhofer.de/jct-vc/JM/-/blob/master/cfg/encoder_main.cfg

With the following adaptations:

-	ProfileIDC = 77 (Main Profile)

-	LevelIDC = 42 (for 1080p60) : 52 (for 4Kp60)

-	IntraPeriod = 0 (only first) or 64 (approaching 1s for 60fps)

-	IDRPeriod = [Same as IntraPeriod]

-	QPISlice = QPPSlice = [17, 22, 27, 32, 37, 42]

-	SearchRange = 64 for fullHD ; 128 for 4K

-	NumberOfReferenceFrames = 2

-	NumberBFrames = 0 [TBC]

-	SearchMode = 0 (fast full search)

#### 6.4.9.3 Reference Software H.265 (HEVC) Main 10 Profile

To generate the anchor bitstream, the following HEVC reference software was used:

-	https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tags/HM-16.22

The settings are defined as follow:

-	https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/blob/HM-16.22/cfg/encoder_lowdelay_P_main10.cfg

Each source sequence is encoded with the following changes:

-	QP: [22, 27, 32, 37]

Editor’s Note: QP values may need to be refined

-	For the intra period, two configurations are tested

-	IntraPeriod with no fix interval

-	GOPSize is equal to 8. Each P picture refers to up to 4 preceding pictures in display order

-	IntraQPOffset is -1. P picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

-	IntraPeriod such that 1 second is achieved

-	DecodingRefreshType: (2) IDR

-	IntraQPOffset and QPoffsets are set equal to 0

-	Each P picture refers to immediately preceding pictures in display order.

#### 6.4.9.4 Reference Software H.265 (HEVC) SCC Profile

To generate the anchor bitstream, the following reference software for HEVC Screen Content profile was used:

SCM-8.8: https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tags/HM-16.21+SCM-8.8

The settings are defined as follow:

-	https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/blob/HM-16.21+SCM-8.8/cfg/encoder_lowdelay_main_scc.cfg

Each source sequence is encoded with

-	QP: [22, 27, 32, 37]

Editor’s Note: QP values may need to be refined

-	For the intra period, two configurations are tested:

-	IntraPeriod with no fix interval

-  The settings are defined in the attached file “encoder_lowdelay_P_main_scc_gop8”:

-  	GOPSize is equal to 8. Each P picture refers to up to 4 preceding pictures in display order

-  	IntraQPOffset is -1. P picture QP offsets are alternatively equal to 4, 5, and are set to 1 every 8 pictures.

-	IntraPeriod such that 1 second is achieved

-  	The settings are defined in the [soon to be provided] attached file “encoder_lowdelay_P_main_constant_qp_scc_gop8”.

- 	DecodingRefreshType: (2) IDR

-  IntraQPOffset and P pictures QPoffsets are set equal to 0

-  Each P picture refers to immediately preceding pictures in display order.

End of First Change