3GPP TSG-SA4 meeting #111-e	S4-201624

# 11-20th November, 2020					    						    					Revision of S4-201493

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.802 | CR | – | rev | – | Current version: | 0.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR TR 26.802 - key issues for multicast distribution in private networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Europe B.V., TELUS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_Multicast |  |  |  |  |  | Date: |  |  | 2020-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Add an important aspect of multicast delivery for consideration, namely the use case of multicast distribution in private networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal for key issues around the use case of localised multicast distribution in private networks. Here the term “private networks” is used in the generic sense. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the important overall use case for localised multicast distribution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.X	Key Issue #X: Multicast distribution in private networks

### 5.X.1	Description

This key issue will study the use case whereby a localised private network includes media streaming applications that rely on local multicast delivery to 5GMSd Clients. Here the term “private networks” is used in the generic sense. In 3GPP terminology this can be mapped to various areas, for example Non-Public Networks (NPN) or Local MBS service. In the present key issue it maps to the “Local MBS service” key issue #6 documented in TR 23.757 clause 5.6.

An exemplary real-world use case is that of a signage system in a private managed network, for example deployed in a large shopping mall. A common feature of this use case is that the system plays back the same media streaming content either to a group of displays, or to the whole set of displays at the mall, for example to advertise special promotions or attractions, to provide broadcast announcements to visitors, or to provide general information or background entertainment.

Such a signage system installation would typically make use of pre-installed network fibres/cabling to many display locations, but to also reach indoor or outdoor locations via 5G, where cabling was not pre-installed. Since multicast distribution is commonly used in such cabled solutions, it would make sense to include the wireless locations in that multicast distribution capability. Thus 5GMS could be the overall architecture for the whole system if it was multicast-capable. The Local MBS service provides the basis for this capability.

On some occasions it is useful to be able to relay a real-time media stream, e.g. televised broadcast or emergency notification, via the ingest interface, to the whole set of display devices at the location, using local multicast delivery.

TR 23.757 clause 5.6 also mentions other examples of applicable service scenarios, namely V2X and Public Safety.

### 5.X.2		Identified gaps

Initial set of identified gaps:

-	Multicast delivery of media streams from AS to 5GMSd Clients in the Local MBS service;

-	Offline ingest of media steaming content to a media streaming server in the Local MBS service;

-	Media streaming ingestion interface extension for Local MBS service, potentially including the multicast distribution of content to certain cells (e.g. identified by Cell ID), tracking areas (e.g. identified by TAC) or based on a geographic metric;

-	Real-time ingestion of a unicast media stream to a Local MBS service multicast stream;

-	Clarify with SA2 the scope for application of the Local MBS service in a Public Network Integrated NPN;

-	Capability to deliver non-segmented media streams in the Local MBS service, e.g. to enable a more straightforward conversion from a broadcast feed.